Linda White                                                                   Oct. 15, 1997

7036 Bickstan Place

Brentwood Bay, BC

V8M 1G6

Rod Forbes

Plankton Productivity

Institute of Ocean Sciences

9860 West Saanich Rd

Sidney, BC

V8L 4B2

Report:  Chlorophyll a and Salinity Analyses of Cruise 9736 seawater samples.

Summary

Chlorophyll a

Chlorophyll a was extracted from filtered frozen seawater samples by the steeping method.  The extracts were read on a Turner Model 10 fluorometer.

A check standard and reference standard were analyzed at the same time as the samples.

Reference material - In-house: SI 2/19/94: 0.86mg/L, a 3.4% variance from established analyses of 0.89mg/L, n = 18.

Salinity

Salinity samples were analyzed on a Portasol Salinometer #  58879, standardized against Standard Seawater #P130, Sal. 34.999, batch Mar. 4, 1996.

Analyst was Elaine Baird as I was called away for family matters.

Summary report includes raw data and calculated data for chlorophyll a and salinity.  Elaine delivered the results on disc.

Filename: lin9736.xls