## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 3-Feb-2014 | The CTD salinity channel was removed from the Bottle files since the salinity data from the rosette files was not processed or calibrated. For details see section “Rosette Files, April 30, 2003” in file “\\OSD_Data_Archive\Cruise_Data\Documents\CS Wong’s Archive\ Station Papa Data Arching 2002.doc”. J.L. |
| 13-Jan-2014 | Added underway pCO2 data from Sophia Johannessen’s Excel files prepared for The Carbon Dioxide Information Analysis Center (CDIAC). The file is located in the cruise .DOC directory. |
| 18-Dec-2013 | Added units to Iron:Filtered:Buffered:0.45 channel. J.L. |
| 18-Jun-2013 | Merged DMS data to bottle casts from Wendy Richardson’s spreadsheet file located in the \DOC directory. For information on methods see file "Method 1996 to 1999 for Dimethylsulfide Analysis.doc” in directory \\OSD_Data_Archive\Cruise_Data\Documents\Analysis Reference Papers\. |
| 5-Jun-2013 | Added Iron profile files with cast numbers 8xxx from Keith Johnson’s spreadsheet file which can be found in the cruise .DOC directory. |
| 14-Feb-2012 | Added one productivity cast containing Primary Productivity, Chlorophyll, Nutrients, POC and PON, from Frank Whitney’s Productivity spreadsheet. |

# Moving files to new archive. April 30, 2003 – Joe Linguanti

- Program HEADER EDIT was used to change non-standard channel names and units and add additional metadata to the CTD and Rosette files.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files .CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.* “.

- The loop file was obtained from Frank Whitney. The latitude and longitude values were computed to 3 decimal places. The spreadsheet file was then converted to IOS Header format with meta-data added to it. The original .XLS file is in the DOC directory.

- The thermosalinograph files were copied from the TRACKOBS directory in the OSAP Data Archive.

- Program HEADER EDIT was  used to change non-standard channel names and units and add additional metadata to the files.

- IOS Header Date and Time channels were added.

- The FLAG channel was removed

- Channels were re-ordered

- Program Clean was used to set the start and end times and the geographical area.

May 7, 2003 J. Linguanti

Added chlorophyll and ammonium data from Frank Whitney’s spreadsheet file, 9815-HYD.xls, to 13 bottle files

CTD Processing Notes

The CTD casts were initially processed immediately after each cast using the most recent SBE calibrations and the batch procedure recommended in the manual. The bottle files, .btl, were summarized in spreadsheet form using “get_btl.bas” , see 9815allbot.xls. This allowed the CTD to be evaluated for internal consistency between the sensors and fouling events to be diagnosed and remedied with cleaning procedures. When the bottle salinities were available, they were added to the spreadsheet and were used to arrive at calibration slopes and offsets for final processing.

# Final Processing

Pressure: OK
	Temperature: Sensors disagreed by .001C at 2C and by -.002 at 12C so there is a
                                  small error in temperature and a slope error in salinity amounting to about
                                 .003 which cannot be corrected because there is no information on which
                                  sensor is wrong and the errors are small in any case. T0 was used without
                                 modification throughout.

Conductivity: At the outset, these sensors disagreed somewhat with bottles and an initial
                                  slope correction of .99980 for C0 and 1.00018 for C1 were used. Both
                                  sensors subsequently drifted and experienced shifts probably due to
                                  biological fouling.

A careful look at the above plot shows a systematic difference between Cell0 and Cell1 changing with pressure in the deep casts. These effects were taken into account in determining the cell slope corrections but could not be addressed in the Seabird software and were left to post processing in IOSShell.

Each cast was given its own slope correction based on bottle salinities. Cell0 seemed to be changing in a consistent way through the cruise so it was decided to use Cell0 and its slope correction K0 with T0 to compute salinity, S00, although Cell1 and T1 were still carried through the various processing stages.

| Station | Cast | Slope Correction, K00 | Slope Correction, K01 |
| --- | --- | --- | --- |
| S3 | 2, 1-5 | .999841 | 1.000199 |
| P4 | 8, 6-8 | .999874 | 1.000246 |
| P4 | 10, 9-12 | .999802 | 1.000159 |
| P8 | 14 | .999753 | 1.000135 |
| P12 | 19, 17-19 | .999763 | 1.000176 |
| P12 | 20, 20-22 | .999795 | 1.000236 |
| P16 | 24, 23-25 | .999748 | 1.000208 |
| P16 | 26,26-28 | .999726 | 1.000597 |
| P20 | 31,29-31 | .999789 | 1.000306 |
| P20 | 32,32-36 | .999732 | 1.000254 |
| P26 | 40,37-40 | .999731 | 1.000270 |
| P26 | 41 | .999769 | 1.000309 |
| P26 | 42 | .999811 | 1.000325 |
| P26 | 43,43-46 | .999738 | 1.000312 |
| Z04 | 50,47-50 | .999766 | 1.000309 |
| Z04 | 51,51-54 | .999689 | 1.000223 |
| Z09 | 58,55-58 | .999702 | 1.000253 |
| Z09 | 59,59-61 | .999720 | 1.000264 |
| Z14 | 64,62-64 | .999715 | 1.000273 |
| Z14 | 65,65-68 | .999674 | 1.000238 |
| Z19 | 72,69-72 | .999731 | 1.000325 |
| Z19 | 73,73- | .999725 | 1.000281 |

Just before the end of the cruise, the raw data were run again through the SBE batch processing. The calibrated data was brought into a new spreadsheet, 9815allbot_after.xls to check the accuracy of the calibrations. The uncompensated pressure effect now shows up clearly.

Some bottles in high gradients were taken out of the calibration determinations but are shown below as crosses.

The pressure dependent correction was: 1.0e-7*p for cell0 and 0.5e-7*p for cell1. This correction was applied in IOSSHELL since there is no facility for doing so in the Seabird software.

Data were processed using the procedures recommended in the SBE manual and the following IOSSHELL steps.

- Datcnv to convert the binary data(.dat files) to ascii using the .con files modified with the slope corrections in the table above so that all subsequent files have the correct slope and offset corrections for the raw sensors. The resulting ascii .cnv file was copied to a zip file.

- Rossum was run on the .ros files to produce .btl files with averaged data at the bottle points.

- Wildedit with 2 standard deviations on pass1 and 20 on pass 2 using 10 points; all raw channels.

- Celltm to remove sef-heating with the following parameters
# celltm_alpha = 0.0300, 0.0300
# celltm_tau = 9.0000, 9.0000

- Filter to smooth pressure before loopedit: tau=.15 s.

- Loopedit to remove data affected by wave motion; min. speed=.25 m/s.

- Binave; 1 metre avg both down and up. This file is saved as an .avg file.

- Split to remove upcast was not used since bottle comparisons would be done later.

- Import .avg files into IOSSHELL to get .ios files, apply pressure correction to get .cor files and compute derived quantities to get .ctd files and remove upcast to get .del files.

- Data was thinned to get .thn files then was “Page Plotted” to make up the report.

- Print the header summary; Cast 3 had to be corrected.

- Convert .del files from pressure to gamma-theta cordinate to compress the data into .dyn files.

- Export .dyn files to spreadsheet. Do plots, etc.

- Import into Access data base for analysis.

- Removech was executed on the .del files to remove derived and superfluous channels. The resulting .rem files were renamed .ctd for the final archive.

## July 2, 1999 - Joe Linguanti

Salinities in the upper 4 db were found to be too low. The values for 2, 3

and 4 db were replaced with the value at 5 db.

Input parameter file = H:\temp\iosshell\IS1

List file = Summ9815

************************************************

*  Program: HSUMMARY                           *

*  Version: 1.1         Date: October 1994     *

*                                              *

*  Copyright 1994 Government of Canada         *

*  Department of Fisheries and Oceans          *

*                                              *

*  Time of run:  25-JUN-1998 10:40:12          *

************************************************

Input file is in ASCII format.

Input path = h:\linep_98\sb_avg\

Cruise year,cruise number = 98,15

First and last cast number =    1,  82

Filename extension = IOS

Summary listing will be produced.

1

1

Cruise  9815

Cast no:   1     Station  S3                   Project  LineP

Latitude  48 35.60 N          Longitude   123 30.15 W

Start date   3/ 6/98          Start time     2:56 UTC

Maximum depth    215          No. of records      214

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150001.DAT

Cast no:   2     Station  S3                   Project  LineP

Latitude  48 35.60 N          Longitude   123 30.15 W

Start date   3/ 6/98          Start time     3: 6 UTC

Maximum depth    215          No. of records      215

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150002.DAT

Cast no:   3     Station  PLC2                 Project  LineP

Latitude  48 27.61 N          Longitude   125  7.81 W

Start date   3/ 6/98          Start time    15: 3 UTC

Maximum depth    159          No. of records      316

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150003.DAT

Cast no:   4     Station  PLC3                 Project  LineP

Latitude  48 23.46 N          Longitude   125 20.84 W

Start date   3/ 6/98          Start time    17:28 UTC

Maximum depth    117          No. of records      229

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150004.DAT

Cast no:   5     Station  P1                   Project  LineP

Latitude  48 34.49 N          Longitude   125 29.98 W

Start date   3/ 6/98          Start time    20:59 UTC

Maximum depth    110          No. of records      217

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150005.DAT

1

Cruise  9815

Cast no:   6     Station  P2                   Project  LineP

Latitude  48 35.95 N          Longitude   125 59.98 W

Start date   3/ 6/98          Start time    23:17 UTC

Maximum depth    102          No. of records      202

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150006.DAT

Cast no:   7     Station  P3                   Project  LineP

Latitude  48 37.56 N          Longitude   126 19.80 W

Start date   4/ 6/98          Start time     1: 3 UTC

Maximum depth    789          No. of records     1575

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150007.DAT

Cast no:   8     Station  P4                   Project  LineP

Latitude  48 39.07 N          Longitude   126 40.17 W

Start date   4/ 6/98          Start time     3:30 UTC

Maximum depth   1320          No. of records     2636

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150008.DAT

Cast no:   9     Station  P4                   Project  LineP

Latitude  48 39.03 N          Longitude   126 39.82 W

Start date   4/ 6/98          Start time    13:47 UTC

Maximum depth    152          No. of records      303

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150009.DAT

Cast no:  10     Station  P4                   Project  LineP

Latitude  48 38.98 N          Longitude   126 40.13 W

Start date   4/ 6/98          Start time    18:12 UTC

Maximum depth    402          No. of records      800

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150010.DAT

1

Cruise  9815

Cast no:  11     Station  P5                   Project  LineP

Latitude  48 41.52 N          Longitude   127 10.13 W

Start date   4/ 6/98          Start time    23:55 UTC

Maximum depth   2111          No. of records     4220

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150011.DAT

Cast no:  12     Station  P6                   Project  LineP

Latitude  48 44.72 N          Longitude   127 40.00 W

Start date   5/ 6/98          Start time     3: 2 UTC

Maximum depth   2572          No. of records     5142

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150012.DAT

Cast no:  13     Station  P7                   Project  LineP

Latitude  48 46.60 N          Longitude   128 10.13 W

Start date   5/ 6/98          Start time     7: 5 UTC

Maximum depth   2526          No. of records     5049

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150013.DAT

Cast no:  14     Station  P8                   Project  LineP

Latitude  48 48.97 N          Longitude   128 39.85 W

Start date   5/ 6/98          Start time    10:32 UTC

Maximum depth   2539          No. of records     5076

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150014.DAT

Cast no:  15     Station  P9                   Project  LineP

Latitude  48 51.31 N          Longitude   129  9.96 W

Start date   5/ 6/98          Start time    14:48 UTC

Maximum depth   2390          No. of records     4775

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150015.DAT

1

Cruise  9815

Cast no:  16     Station  P10                  Project  LineP

Latitude  48 53.66 N          Longitude   129 39.91 W

Start date   5/ 6/98          Start time    18:19 UTC

Maximum depth   2676          No. of records     5350

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150016.DAT

Cast no:  17     Station  P11                  Project  LineP

Latitude  48 56.10 N          Longitude   130 10.00 W

Start date   5/ 6/98          Start time    22:45 UTC

Maximum depth   2791          No. of records     5580

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150017.DAT

Cast no:  18     Station  P12                  Project  LineP

Latitude  48 58.25 N          Longitude   130 39.96 W

Start date   6/ 6/98          Start time     2:21 UTC

Maximum depth    153          No. of records      305

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150018.DAT

Cast no:  19     Station  P12                  Project  LineP

Latitude  48 58.32 N          Longitude   130 39.74 W

Start date   6/ 6/98          Start time     2:38 UTC

Maximum depth   3279          No. of records     6555

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150019.DAT

Cast no:  20     Station  P12                  Project  LineP

Latitude  48 58.31 N          Longitude   130 39.87 W

Start date   6/ 6/98          Start time    17:18 UTC

Maximum depth    402          No. of records      799

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150020.DAT

1

Cruise  9815

Cast no:  21     Station  P13                  Project  LineP

Latitude  49  2.60 N          Longitude   131 39.89 W

Start date   7/ 6/98          Start time     1:20 UTC

Maximum depth   3056          No. of records     6107

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150021.DAT

Cast no:  22     Station  P14                  Project  LineP

Latitude  49  7.24 N          Longitude   132 40.13 W

Start date   7/ 6/98          Start time     7: 2 UTC

Maximum depth   3007          No. of records     6012

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150022.DAT

Cast no:  23     Station  P15                  Project  LineP

Latitude  49 12.00 N          Longitude   133 40.02 W

Start date   7/ 6/98          Start time    12:42 UTC

Maximum depth   3288          No. of records     6573

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150023.DAT

Cast no:  24     Station  P16                  Project  LineP

Latitude  49 17.00 N          Longitude   134 40.10 W

Start date   7/ 6/98          Start time    18:53 UTC

Maximum depth    403          No. of records      803

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150024.DAT

Cast no:  25     Station  P16                  Project  LineP

Latitude  49 17.02 N          Longitude   134 40.01 W

Start date   8/ 6/98          Start time     6: 6 UTC

Maximum depth   3693          No. of records     3693

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150025.DAT

1

Cruise  9815

Cast no:  26     Station  P16                  Project  LineP

Latitude  49 17.14 N          Longitude   134 40.64 W

Start date   8/ 6/98          Start time     7: 3 UTC

Maximum depth   3696          No. of records     3698

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150026.DAT

Cast no:  27     Station  P17                  Project  LineP

Latitude  49 20.91 N          Longitude   135 40.18 W

Start date   8/ 6/98          Start time    20: 4 UTC

Maximum depth   2511          No. of records     5019

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150027.DAT

Cast no:  28     Station  P18                  Project  LineP

Latitude  49 26.01 N          Longitude   136 40.03 W

Start date   9/ 6/98          Start time     1:29 UTC

Maximum depth   3001          No. of records     6000

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150028.DAT

Cast no:  29     Station  P19                  Project  LineP

Latitude  49 30.08 N          Longitude   137 40.02 W

Start date   9/ 6/98          Start time     7:19 UTC

Maximum depth   3006          No. of records     6007

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150029.DAT

Cast no:  30     Station  P20                  Project  LineP

Latitude  49 33.98 N          Longitude   138 40.02 W

Start date   9/ 6/98          Start time    14:34 UTC

Maximum depth    154          No. of records      307

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150030.DAT

1

Cruise  9815

Cast no:  31     Station  P20                  Project  LineP

Latitude  49 33.97 N          Longitude   138 39.98 W

Start date   9/ 6/98          Start time    19:17 UTC

Maximum depth    416          No. of records      830

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150031.DAT

Cast no:  32     Station  P20                  Project  LineP

Latitude  49 33.99 N          Longitude   138 40.02 W

Start date  10/ 6/98          Start time     4:11 UTC

Maximum depth   4023          No. of records     8044

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150032.DAT

Cast no:  33     Station  P21                  Project  LineP

Latitude  49 38.04 N          Longitude   139 39.93 W

Start date  10/ 6/98          Start time    10:53 UTC

Maximum depth   3004          No. of records     6007

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150033.DAT

Cast no:  34     Station  P22                  Project  LineP

Latitude  49 42.03 N          Longitude   140 39.94 W

Start date  10/ 6/98          Start time    17: 8 UTC

Maximum depth   3003          No. of records     6004

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150034.DAT

Cast no:  35     Station  P23                  Project  LineP

Latitude  49 46.04 N          Longitude   141 40.00 W

Start date  11/ 6/98          Start time     2:39 UTC

Maximum depth   3003          No. of records     6005

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150035.DAT

1

Cruise  9815

Cast no:  36     Station  P24                  Project  LineP

Latitude  49 50.03 N          Longitude   142 39.88 W

Start date  11/ 6/98          Start time     8:16 UTC

Maximum depth   3010          No. of records     6015

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150036.DAT

Cast no:  37     Station  P25                  Project  LineP

Latitude  49 59.95 N          Longitude   143 36.31 W

Start date  11/ 6/98          Start time    13:36 UTC

Maximum depth   3002          No. of records     6001

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150037.DAT

Cast no:  38     Station  P35                  Project  LineP

Latitude  50   .02 N          Longitude   144 18.22 W

Start date  11/ 6/98          Start time    18:15 UTC

Maximum depth   3002          No. of records     6001

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150038.DAT

Cast no:  39     Station  P26                  Project  LineP

Latitude  50   .16 N          Longitude   145   .01 W

Start date  12/ 6/98          Start time     2:27 UTC

Maximum depth    105          No. of records      208

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150039.DAT

Cast no:  40     Station  P26                  Project  LineP

Latitude  50   .20 N          Longitude   144 59.94 W

Start date  12/ 6/98          Start time     2:43 UTC

Maximum depth   4319          No. of records     8632

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150040.DAT

1

Cruise  9815

Cast no:  41     Station  P26                  Project  LineP

Latitude  50   .05 N          Longitude   145   .02 W

Start date  12/ 6/98          Start time    13: 8 UTC

Maximum depth    501          No. of records      998

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150041.DAT

Cast no:  42     Station  P26                  Project  LineP

Latitude  49 59.79 N          Longitude   145   .22 W

Start date  12/ 6/98          Start time    22:39 UTC

Maximum depth    403          No. of records      803

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150042.DAT

Cast no:  43     Station  P26                  Project  LineP

Latitude  50   .00 N          Longitude   144 59.94 W

Start date  13/ 6/98          Start time     4:42 UTC

Maximum depth   1503          No. of records     3003

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150043.DAT

Cast no:  44     Station  P26                  Project  LineP

Latitude  49 59.79 N          Longitude   144 59.69 W

Start date  13/ 6/98          Start time    16: 4 UTC

Maximum depth     33          No. of records       64

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150044.DAT

Cast no:  45     Station  P41                  Project  LineP

Latitude  50 27.06 N          Longitude   144 59.85 W

Start date  14/ 6/98          Start time    19:46 UTC

Maximum depth   1505          No. of records     3006

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150045.DAT

1

Cruise  9815

Cast no:  46     Station  P32                  Project  LineP

Latitude  50 53.92 N          Longitude   144 59.79 W

Start date  15/ 6/98          Start time     1:42 UTC

Maximum depth   1504          No. of records     2992

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150046.DAT

Cast no:  47     Station  Z01                  Project  LineP

Latitude  51 21.02 N          Longitude   144 59.96 W

Start date  15/ 6/98          Start time     6:28 UTC

Maximum depth   1501          No. of records     2997

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150047.DAT

Cast no:  48     Station  Z02                  Project  LineP

Latitude  51 48.09 N          Longitude   144 59.93 W

Start date  15/ 6/98          Start time    10:14 UTC

Maximum depth   1507          No. of records     3011

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150048.DAT

Cast no:  49     Station  Z03                  Project  LineP

Latitude  52 11.95 N          Longitude   145   .06 W

Start date  15/ 6/98          Start time    13:45 UTC

Maximum depth   1504          No. of records     3007

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150049.DAT

Cast no:  50     Station  Z04                  Project  LineP

Latitude  52 38.95 N          Longitude   144 59.76 W

Start date  15/ 6/98          Start time    18:34 UTC

Maximum depth    401          No. of records      799

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150050.DAT

1

Cruise  9815

Cast no:  51     Station  Z04                  Project  LineP

Latitude  52 39.00 N          Longitude   144 59.96 W

Start date  16/ 6/98          Start time     1: 4 UTC

Maximum depth   4135          No. of records     8265

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150051.DAT

Cast no:  52     Station  Z05                  Project  LineP

Latitude  53  6.05 N          Longitude   145   .07 W

Start date   1/ 1/80          Start time     0: 0 UTC

Maximum depth   1502          No. of records     3003

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150052.DAT

Cast no:  53     Station  Z06                  Project  LineP

Latitude  53 32.72 N          Longitude   145   .08 W

Start date  16/ 6/98          Start time    19:40 UTC

Maximum depth   1504          No. of records     3003

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150053.DAT

Cast no:  54     Station  Z07                  Project  LineP

Latitude  54   .08 N          Longitude   145   .08 W

Start date  16/ 6/98          Start time    23:18 UTC

Maximum depth   1505          No. of records     3008

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150054.DAT

Cast no:  55     Station  Z08                  Project  LineP

Latitude  54 26.67 N          Longitude   144 59.96 W

Start date  17/ 6/98          Start time     2:51 UTC

Maximum depth    108          No. of records      215

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150055.DAT

1

Cruise  9815

Cast no:  56     Station  Z08                  Project  LineP

Latitude  54 26.56 N          Longitude   144 59.82 W

Start date  17/ 6/98          Start time     3: 4 UTC

Maximum depth   1503          No. of records     3005

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150056.DAT

Cast no:  57     Station  Z09                  Project  LineP

Latitude  55   .04 N          Longitude   145   .12 W

Start date  17/ 6/98          Start time     7:31 UTC

Maximum depth   4106          No. of records     4153

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150057.DAT

Cast no:  58     Station  Z09                  Project  LineP

Latitude  54 59.93 N          Longitude   145   .09 W

Start date  17/ 6/98          Start time    14:43 UTC

Maximum depth   4090          No. of records     8177

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150058.DAT

Cast no:  59     Station  Z09                  Project  LineP

Latitude  54 59.96 N          Longitude   145  1.02 W

Start date  17/ 6/98          Start time    20: 0 UTC

Maximum depth    402          No. of records      799

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150059.DAT

Cast no:  60     Station  Z10                  Project  LineP

Latitude  55 29.97 N          Longitude   145   .08 W

Start date  18/ 6/98          Start time     2:39 UTC

Maximum depth   1502          No. of records     3001

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150060.DAT

1

Cruise  9815

Cast no:  61     Station  Z11                  Project  LineP

Latitude  56   .03 N          Longitude   145   .05 W

Start date   1/ 1/80          Start time     0: 0 UTC

Maximum depth   1504          No. of records     3007

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150061.DAT

Cast no:  62     Station  Z12                  Project  LineP

Latitude  56 30.09 N          Longitude   144 59.93 W

Start date  18/ 6/98          Start time    10:21 UTC

Maximum depth   1501          No. of records     2998

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150062.DAT

Cast no:  63     Station  Z13                  Project  LineP

Latitude  56 59.98 N          Longitude   145   .04 W

Start date  18/ 6/98          Start time    14: 1 UTC

Maximum depth   1503          No. of records     3004

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150063.DAT

Cast no:  64     Station  Z14                  Project  LineP

Latitude  57 30.03 N          Longitude   144 59.92 W

Start date  18/ 6/98          Start time    17:37 UTC

Maximum depth    400          No. of records      796

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150064.DAT

Cast no:  65     Station  Z14                  Project  LineP

Latitude  57 30.07 N          Longitude   145   .02 W

Start date  19/ 6/98          Start time     2:19 UTC

Maximum depth   4058          No. of records     8115

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150065.DAT

1

Cruise  9815

Cast no:  66     Station  Z14N                 Project  LineP

Latitude  57 31.13 N          Longitude   144 59.80 W

Start date  19/ 6/98          Start time    14:20 UTC

Maximum depth   3937          No. of records     7872

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150066.DAT

Cast no:  67     Station  Z15                  Project  LineP

Latitude  58   .06 N          Longitude   145   .17 W

Start date  19/ 6/98          Start time    19:31 UTC

Maximum depth   1503          No. of records     3002

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150067.DAT

Cast no:  68     Station  Z15                  Project  LineP

Latitude  58 29.99 N          Longitude   145   .00 W

Start date  19/ 6/98          Start time    23:30 UTC

Maximum depth   1504          No. of records     1847

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150068.DAT

Cast no:  69     Station  Z17                  Project  LineP

Latitude  58 50.02 N          Longitude   144 59.93 W

Start date  20/ 6/98          Start time     2:26 UTC

Maximum depth    104          No. of records      207

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150069.DAT

Cast no:  70     Station  Z17                  Project  LineP

Latitude  58 50.01 N          Longitude   144 59.98 W

Start date  20/ 6/98          Start time     2:37 UTC

Maximum depth   1502          No. of records     3000

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150070.DAT

1

Cruise  9815

Cast no:  71     Station  Z18                  Project  LineP

Latitude  59  9.98 N          Longitude   145   .06 W

Start date  20/ 6/98          Start time     5:33 UTC

Maximum depth   1503          No. of records     3001

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150071.DAT

Cast no:  72     Station  Z19                  Project  LineP

Latitude  59 26.28 N          Longitude   145   .03 W

Start date  20/ 6/98          Start time     8:12 UTC

Maximum depth   1014          No. of records     2025

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150072.DAT

Cast no:  73     Station  Z19                  Project  LineP

Latitude  59 26.31 N          Longitude   145   .11 W

Start date  20/ 6/98          Start time    17:49 UTC

Maximum depth    403          No. of records      801

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150073.DAT

Cast no:  74     Station  Z20                  Project  LineP

Latitude  59 41.32 N          Longitude   145   .09 W

Start date  20/ 6/98          Start time    19:52 UTC

Maximum depth    130          No. of records      258

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150074.DAT

Cast no:  75     Station  Z20                  Project  LineP

Latitude  59 56.98 N          Longitude   144 59.96 W

Start date  20/ 6/98          Start time    21:38 UTC

Maximum depth    150          No. of records      298

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150075.DAT

1

Cruise  9815

Cast no:  76     Station  Q 19                 Project  LineP

Latitude  53 57.88 N          Longitude   133 29.70 W

Start date  23/ 6/98          Start time     5:10 UTC

Maximum depth    209          No. of records      414

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150076.DAT

Cast no:  77     Station  BOWI                 Project  LineP

Latitude  53 16.96 N          Longitude   135 39.86 W

Start date   1/ 1/80          Start time     0: 0 UTC

Maximum depth    470          No. of records      939

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150077.DAT

Cast no:  78     Station  GI02                 Project  LineP

Latitude  51 30.25 N          Longitude   129 37.49 W

Start date  24/ 6/98          Start time    20:55 UTC

Maximum depth    134          No. of records      265

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150078.DAT

Cast no:  79     Station  ODAS                 Project  LineP

Latitude  50 51.43 N          Longitude   129 54.99 W

Start date  25/ 6/98          Start time     1:46 UTC

Maximum depth   2129          No. of records     4257

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150079.DAT

Cast no:  80     Station  J22                  Project  LineP

Latitude  50 39.86 N          Longitude   129 17.67 W

Start date   1/ 1/80          Start time     0: 0 UTC

Maximum depth    501          No. of records     1001

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150080.DAT

1

Cruise  9815

Cast no:  81     Station  LQ03                 Project  LineP

Latitude  50 39.80 N          Longitude   129  1.82 W

Start date  25/ 6/98          Start time     8:25 UTC

Maximum depth    502          No. of records     1001

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150081.DAT

Cast no:  82     Station  CPE2                 Project  LineP

Latitude  50 42.83 N          Longitude   128 40.14 W

Start date  25/ 6/98          Start time    10:44 UTC

Maximum depth    112          No. of records      221

No. of channels    8          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = D:\DATA\9815\98150082.DAT

1

Cruise

Start of run: 1998/06/25 10:40:12.96

End of run:   1998/06/25 10:40:38.83