REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 10-Jan-2006 | Surface loop data was added to the archive. The data was acquired from John Morris at PBS. The data archived in September 2005 was replaced with this new set since salinity and chlorophyll was included. The original spreadsheet file from John and more detailed processing notes can be found in the “Cruise_Data\Documents” directory. Any questions regarding this data should be directed to John Morris. J.L. |
| Sep 26, 2005 | Added loop data to the archive. J.L. |

# PROCESSING NOTES

Cruise: 9818

Agency: PBS/IOS

Project: High Seas Salmon

Geographic Area: N.E. Pacific, Gulf of Alaska

Chief Scientist: David Welch

Platform: W. E. Ricker

Date of Cruise: 24 August 1998 - 1 September 1998

Processed by: Germaine Gatien

Date of Processing :11 September 1998 - 17 September 1998

Number of original casts: 106

Number of casts processed:   52

# INSTRUMENT SUMMARY

One CTD was used during this cruise.

Instrument Type:  Guildline 8715 CTD

Instrument Serial Number: 53501

Transmissometer #572

# SUMMARY OF QUALITY AND CONCERNS

There was noise in temperature in some of the deeper casts; the upcasts were substituted.

There were severe spikes in the salinity in 5 casts two of which were replaced with upcasts.

Much of the transmissivity data is spiky; these spikes were not removed since some may not be noise.

The CTD salinities are probably high below the surface (by 0.003-0.02psu)  but the calibration results are too scattered to justify a correction. Near the surface the comparison looks good with most values within 0.003psu.

# PROCESSING SUMMARY

- PRELIMINARY STEPS

The Log Book was obtained and  note was made of problems that occurred during the cruise.

The salinity bottle data sheets were found; the samples have not been processed yet.

The cruise summary sheet was completed.

The header summary and header check were examined. No errors were found.

The surface check gave an average surface pressure of  1.688dbars. No corrections were made.

This cruise contains separate up and down casts.

- CONVERSION OF RAW DATA

- The files were converted using IOSSHELL routine to convert GUILDLINE files.

- CALIBRATION

All data was calibrated using file 9818cal.ccf which is attached.

- DESPIKE

- The following ADVANCED DESPIKE parameters were used:

- CHANNEL        FIT   OVER-  MIN       MAX       MIN         MAX         SPIKE

- WIDTH LAP    VALUE   VALUE  STDDEV  STDDEV     TOL.

- -------------------- ----- ----- ------- ------- ------- ------- -----

- Pres 		55     5        -2.00       4000.00	.10000        999.000    3.00

- Temp   	55     5        -2.00          15.00	.00500        999.000    3.00

- Cond    	55     5           .00            1.50	.00010        999.000    3.00

- Tran     	55     5           .00        100.00 	.10000        999.000    3.00

- After this step only downcasts were processed unless a decision was made to substitute an upcast; in that case the data was reversed at this stage.

- TIME COMPENSATION

- Temp. probe Dist (m):   .0700  Sample period (sec):   .0400

- DELETE

The following DELETE parameters were used:

Surface Record Removal: Low Sal & Last Press Min

Maximum Surface Pressure (relative):      20.00

Minimum Surface Salinity:     10.000

Surface Swell Pressure Tolerance:       1.00

Pressure filtered over width:   11

Swells deleted. Warning message if pressure difference of      2.00

Drop rates<    .30m/s (calculated over  11 points) will be deleted

Sample interval =       .04 seconds.

Due to warnings DELETE was rerun with the Slow Drop Rate feature turned off for casts: #21, 87, 91, 93, 96, 97, 100 and 103.

- FILTER

- A median box car filter with filter width 21 was applied to the salinity channel.

7.   BIN AVERAGE

The following Bin Average values were used:

Bin channel = pressure

Averaging interval =      1.000

Minimum bin value =       .000

Average value will be used

Interpolated values are NOT used for empty bins

- TEST PLOTS

- Page plots were produced. Large salinity spikes were found in casts 45, 49,55, 59 and 97. For the first three the upcasts had spikes too, but for 59 and 97 the upcasts (60 and 98) were better so the downcasts  were deleted and the upcasts were processed.

- The page plots were used to guide the editing described in the next section.

- HAND EDITING

The following casts were edited for S only in the top 10m: 15,39,69,91,96,105.

The following casts were edited for T and S only in the top 10m: 1,6,27,47,61,73.

The following casts were edited more extensively: 21,23,45,49,55,60,81,83,87,89,93,98,100.

Details are noted in the comment section of the individual files.

- COMPARISON WITH BOTTLE DATA

The four salinity samples below the surface indicate that the CTD was reading high by an average of 0.007psu. There were surface samples for all casts. A comparison was made at a selection of casts with fairly well-mixed surface layers which show that the CTD was low by 0.001psu. Given the large scatter in the few observations available there was no attempt to recalibrate. The observations in the top 600 meters appear to be within 0.005psu.

- OTHER INTERCOMPARISONS

Profiles plotted for casts #5,7,9,11,13,15,17,19,21,23,75,77 and 79 indicate that the salinity and temperature data all fell within the historic ranges.

12. FINAL PLOTS

Page plots were prepared using the edited data.

13.	REMOVE CHANNELS

The following channels were removed from all casts: Conductivity_Ratio.

14.	PRODUCE FINAL FILES

a.) The final files were renamed *.ctd.

b.) A cross-reference listing was produced.

15. PARTICULARS

Casts #3&4 - Bad salinity. Casts 4 & 5 are repeats at this site. Delete.

Cast #5 - Big spike in salinity. Upcast is better. Delete

Cast #6 - Upcast - use instead of #5.

Cast #45 - Large salinity spike removed with CTDEDIT.

Cast #49 - Large salinity spike removed with CTDEDIT.

Cast #55 - Large salinity spike removed with CTDEDIT.

Cast #59 - Large spike in salinity and noisy transmissivity. Upcast is better. Delete.

Cast #60 - Upcast - use instead of #59.

Cast #95 - Some noise on the temperature trace at 800m on the downcast. Use upcast. Delete.

Cast #96 - Upcast - use instead of 95.

Cast #97 - Noise in S and T on downcast; use upcast. Delete.

Cast #98 - Upcast - use instead of 97.

Cast #99 - Noise in the downcast temperature; use upcast. Delete.

Cast #100 - Upcast - use instead of 99.

Cast #101 - Downcast started before recording started; use the upcast. Delete.

Cast #102 - Upcast - use instead of 101.

Institute of Ocean Sciences

CRUISE SUMMARY

Cruise ID#:	9818

Dates: 	Start:	24 August 1998			    End:	1 September 1998

Location:	N.E. Pacific,Gulf of Alaska

Vessel:		W. E. Ricker

Scientific Party Chief:    David Welch

| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |
| --- | --- | --- | --- | --- | --- |
| 1 | Guildline | 8715 | 53501 | no | yes |

Institute of Ocean Sciences

CTD Calibration Information

Make/Model/Serial#:	Guildline /8715/53501

Cruise ID#:	9818

| Calibration Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sensor |  | Pre-Cruise |  | Post Cruise |  |
| Name | S/N | Date | Location | Date | Location |
| Pressure |  |  |  |  |  |
| Temperature |  | 7 Aug 98 | I.O.S. |  |  |
| Conductivity |  | 7 Aug 98 | I.O.S. |  |  |
| Transmissometer | 572 | 31 July 98 | I.O.S. /Doug Anderson |  |  |

| Calibration Coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel | Formula | Coefficients |  |  |  |  |  |
| Name | # | C1 | C2 | C3 | C4 | C5 | C6 |
| Temperature | 10 | .0027318 | .999875 |  |  |  |  |
| Conductivity | 10 | .0000503156 | .999729 |  |  |  |  |
| Transmissivity # | 10 | 125.304 | -2.50623 |  |  |  |  |