File List for SeaBird Data

Cruise Number: 9928

[work2.ctd.9928]

50MZ. – original files provided

50zip.exe          “

*.hdr - 	original files from SeaSave

*.dat - 		“

*.con -      	“

*.bl -		“

*.dir - subdirectories

9928proc.doc - processing notes

9928file.doc - this document

0437ctd.con - con file used for processing

9928rcal.ccf – recalibration file used for casts #143-153

9928comp.csv – input file for COMPARE

9928comp.xls – comparison of bottle salinities and CTD salinities for a few casts.

[work2.ctd.9928.convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[work2.ctd.9928.derive]

*.cnv - data after running ALIGNCTD, WILDEDIT, CELLTM and DERIVE (for S0, S1)

[work2.ctd.9928.ios]

Files containing all channels except SeaTech fluorescence:

*.avg - data after metre-averaging using BINAVE

*.cor – recalibrated files (after editing)

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios - data after conversion to IOS HEADERS

*.log and *.lo* - log files created by CTDEDIT.

*.re1 – edited, recalibrated data with extraneous channels removed

Delete.log - log from routine DELETE

Files containing only SeaTech fluorescence, pressure and scan number:

*.av1 - metre-averaged files with Sea Tech fluorescence, pressure and scan # only

Files containing all channels:

*.ctd - final files (merge of *.re1 files and *.av1)

[work2.ctd.9928.hydro]

*.ros  - rosette files created by DATCNV, no other processing done.

*.bot – rosette files converted to IOS HEADERS

*.sam – *.bot files with sample numbers added

*.hyd – bottle files provided by Bernard Minkley (most do not have salinity data)