## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 18-Nov-2004 | Added loop data to the archive. J.L. |

PROCESSING NOTES

Cruise: 9933

Agency: PBS/Stock Assessment

Location:  North-East Pacific

Project: Offshore Hake Survey

Chief Scientist: Mark Saunders

Platform: CCGS W. E. Ricker

Date: 16 August 1999 – 29 August 1999

Processed by: Germaine Gatien

Date of Processing: 22 October 1999 – 26 October 1999

Number of original CTD casts: 15

Number of casts processed: 14

INSTRUMENT SUMMARY

A SeaBird Model SBE 911+ CTD  (#0506) was used.

SUMMARY OF QUALITY AND CONCERNS

The data was in good order requiring only light editing.

PROCESSING SUMMARY

1. Seasave - This step was completed at sea; the raw data files are *.dat.

2. Preliminary Steps

The CTD log sheets were obtained.

The surface salinity data was obtained.

The cruise summary sheet was completed.

The configuration file was obtained and the calibration constants were checked. Some of the configuration files are incorrect containing calibrations from a previous set-up for CTD #0506. Conversion file 99AUG33.con contains the correct information.

- Conversion of Raw Data

The files 00330014.* and 00330015.* were renamed 99330014.* and 99330015.* respectively. The raw data was converted (without salinity) using conversion file 99aug33.con.

4. ALIGNCTD

A study was made of the effects of advancing the conductivity channels by 0.035s, 0.045s, 0.055s and 0.060s for cast #2. The choice of 0.055s gave the best results.  So for all casts the primary conductivity channel was advanced by -0.018s and the secondary conductivity advanced by 0.055s. The primary channel was aligned by +0.073 by the acquisition system, so the net effect is that both channels are advanced by +0.055s.

5. WILDEDIT

Program WILDEDIT was used to remove spikes in pressure, temperature, conductivity and transmissivity.  Parameters used were:

Pass 1 - Std Dev = 2;	Pass 2 - Std Dev = 5;	Points per block = 50

6. CELLTM

The conductivity cell thermal mass correction was done for both channels (alpha = 0.03; 1/beta = 9.0.)

7. DERIVE

Program DERIVE was run twice:

- on all casts to calculate primary and secondary salinity.

- on a selection of casts to calculate the differences between primary and secondary channels for temperature, conductivity and salinity and to examine the descent rate.

8. Test Plots and Channel Check

A sample of casts (#9 and 13) was plotted to check for agreement between the pairs of  T and C sensors. The temperature variations are very noisy but ignoring spikes are <0.001C° around 700db while the conductivity differences are <0.0001units. Salinity values were remarkably close with differences within 0.0006psu at 700db. The primary values were higher for temperature and conductivity, and salinity difference varied in sign.

- Conversion to IOS Headers

The IOSSHELL routine was used to convert SEA-Bird 911+ data to IOS Headers. Many header errors related to pressure spikes required editing to enable this routine.

10. Checking Headers

A header summary was produced and errors in one station name and one time were fixed.

The surface check was run. The average surface pressure is 5.6db.

The header check was produced.

The cruise track was plotted and looks reasonable.

- DELETE

The following DELETE parameters were used:

Surface Record Removal: Last Press Min

Maximum Surface Pressure (relative): 20.00

Surface Swell Pressure Tolerance: 1.0

Pressure filtered over width: 15

Swells deleted. Warning message if pressure difference of 2.00

Drop rates<    .3 m/s (calculated over 15 points) was deleted.

Sample interval =       .04 seconds.

The only warnings concerned the bottom of the casts.

12. BIN AVERAGE

The following Bin Average values were used:

Bin channel = pressure

Averaging interval = 1.000

Minimum bin value =   .000

Average value will be used

Interpolated values are NOT used for empty bins

13. Intercomparisons

Previous experience with these sensors – The secondary conductivity sensor was used during cruise 9909 in April 1999 (with a different temperature sensor) and was found to give salinities low by about 0.016psu. The primary temperature and conductivity sensors were used during cruises 9913 and 9914 in May and June of 1999. There was evidence of some drift in the calibrations at that time.

COMPARE - There was no rosette mounted on the CTD but 2 deep bottles were collected; comparing these with the CTD indicates that the CTD salinities for both channels were low by ~0.013psu and ~0.018psu. The surface loop samples were compared with CTD samples from 4 to 6m depth; the results suggest that the CTD gave values high by about 0.07psu. The deep values are in reasonable agreement with observations during 9909.

Historic ranges - Plots were examined with historic ranges superimposed for all casts. The only data that did not fall in the historical bounds was a section of high salinity in cast 12 between 60 and 90db.

- Recalibration

Based on these comparisons the secondary salinity was selected for further processing. It was recalibrated using file 9909rcal.ccf (from the archive for cruise 9909) after editing it to apply to secondary salinity. The secondary salinity was raised by 0.0156psu.

15. Test Plots

Profiles of all casts were examined for any evidence of problems with the processing. For cast #6 the primary and secondary salinity do not compare well because the pump was not turned on. Cast #7 is at the same site, so #6 will be deleted.

Page plots were produced using T1,S1 for all casts. These plots were examined for spikes and instabilities and used to guide the use of CTDEDIT.

16. CTDEDIT

CTDEDIT was used to clean noise in secondary salinity near the surface for casts #2 and #9 and in the top 25m for cast #10. Note was made of the editing details in the relevant files.

17. Final Plots

THIN and DERIVE were run to obtain values for tables and page plots were prepared using the edited data.

18. REMOVE

The following channels were removed from all casts: Primary Temperature, Primary Salinity, Conductivity:Primary, Conductivity:Secondary, Scan_Number and Flag.

19. Producing final files

a.) The final files were renamed *.ctd.

b.) A cross-reference listing was produced.

Particulars

Cast #1 –Station named changed from 1 to DS1.

Cast #6 – Pumps not turned on until 234m. Cast #7 is at the same site. Deleted.

Cast #12 – Error in station name in header. Changed to DS11.

Cast #14 – Originally saved as 00330014; renamed 99330014.

Cast #15 – Originally saved as 00330015; renamed 99330015.

# Institute of Ocean Sciences

CRUISE SUMMARY

Cruise ID#:	9933

Dates: 	Start:     21August 1999           	  End:	27 August 1999

Location: North-East Pacific

Vessel:	 CCGS W.E. Ricker

Chief Scientist:	  Mark Saunders

| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |
| --- | --- | --- | --- | --- | --- |
| 1 | SEABIRD | 911+ | 0506 | no | yes |

Institute of Ocean Sciences

CTD Calibration Information

Make/Model/Serial#:	SEABIRD/911+/	0506

Cruise ID#:		9933

| Calibration Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sensor |  | Pre-Cruise |  | Post Cruise |  |
| Name | S/N | Date | Location | Date | Location |
| Temperature | 2710 | 24Mar99 | Factory |  |  |
| Conductivity | 2280 | 19Mar99 | Factory |  |  |
| Secondary Temp. | 2371 | 16Jun99 | Factory |  |  |
| Secondary Cond. | 2278 | 19Mar99 | Factory |  |  |
| Transmissometer | 197 | 14Jan99 | lab |  |  |
| Pressure Sensor | 69698 | 11Dec99 | Factory |  |  |

Sensor Calibration Notes:

The configuration file used is attached; this includes the sensor calibrations.