## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 28-Oct-2013 | - Merged DMS data to bottle casts from Mike Arychuk’s spreadsheet file located in the \DOC directory. For information on methods see file "Method post 1999 for Dimethylsulfide Analysis.doc” in directory \\OSD_Data_Archive\Cruise_Data\Documents\Analysis Reference Papers\ |
| 19-Feb-2013 | - Added Iron profile files with cast numbers 8xxx and one surface file from Keith Johnson’s spreadsheet file which can be found in the cruise .DOC directory. |
| 17-Feb-2012 | - Added one productivity cast containing Primary Productivity, Chlorophyll, POC and PON, from Frank Whitney’s Productivity spreadsheet. - Replaced Alkalinity and DIC data with values from Jim Christian’s DIC files since they contained quality flags, for bottle cast number 57. Jim Christian’s DIC file is located in the \DOC directory. For more information, including quality control analysis see John Page’s spreadsheet files in directory \\OSD_Data_Archive\Cruise_Data\Documents\Station Papa Data Archiving\Alkalinity and DIC. |
| May 28, 2010 | An error was found in the calibration parameters used in processing this cruise. It is estimated that pressure is low by <0.5db, so no correction was applied. For details see file “Report on Calibration Errors for Pressure Sensor #77511, CTD0585 “ in Osd_Date_Archive\Cruise_Data\DOCUMENTS |
| June 3, 2004 | Added DIC and Alkalinity data to the archive. |
| April 21, 2003 | Moved files to new archive. |
| March 12, 2002 | Fl data was recalibrated for casts #2 through 75. See details at end. |
| August 31, 2001 | Reprocessed data to filter and calibrate fluorometer channels. Details in notes at end of this document. |

PROCESSING NOTES

Cruise: 2000-25

Agency: IOS

Project: Line P

Chief Scientist: Howard Freeland

Platform: J. P. TULLY

Date: 4 September 2000 –21 September 2000

Processed by: Ron Perkin

Date of Processing:

Number of original CTD casts: 39

Number of casts processed: 39

INSTRUMENT SUMMARY

CTD Sensors

| CTD | T0 | T1 | C0 | C1 |
| --- | --- | --- | --- | --- |
| 550 | 2023 | 2371 | 2173 | 1766 |
| 585 | 2663 | 2668 | 2399 | 2424 |

A SeaBird Model SBE 911+ CTD (#0550) was used for most rosette casts. Two Wetstar fluorometers (#2229 and #2228(pumped)), Seatech Transmissometer (#333DR) and Par Sensor (GSP-200L4S) were mounted on the CTD. A surface Par sensor (SPAR) was mounted on the container lab and wired into the CTD deck unit but did not work and provided no data.

A second CTD(#585) equipped with transmissometer(192D) was used for CTD casts 21,22,26,27,28 during foul weather.

After the deep rosette cast at station P(cast 57), the secondary temperature sensors were swapped and CTD 585 was mounted on the rosette for casts 65,67(shallow rosette casts) in order to link all the temperature sensors together for confirmation of the temperature calibration. The unpumped fluorometer(#2229) was also mounted on CTD 585 and the pumped flurometer(#2228) was mounted on the seawater loop to calibrate it against the chlorophyll samples.

On the return leg, for casts 68 and 70, two Ph(#180339 and #180923) and  sensors were added to CTD as “userpoly 1” and “userpoly 2” respectively. Keith Johnson is working out the calibrations for these two pH sensors.

Table 1. Instrument Summary

| Cast No. | CTD | .CON file | Config. File | Sensors | Comment |
| --- | --- | --- | --- | --- | --- |
| 1 to 20 | 550 | 550_1-57.con | Dat550.cfg | Tx, Fl(2), PAR | PAR on some casts |
| 21 to 28 | 585 | *.con | Dat585.cfg | Tx | No bottles |
| 29-57 | 550 | 550_1-57.con | Dat550.cfg | Tx, Fl(2), PAR | PAR on some casts |
| 65 & 67 | 585 | *.con | Dat585b.cfg | Tx, Fl | bottles |
| 68 | 585 | *.con | Dat585c.cfg | Tx, Fl, pH(2) | pH voltage, bottles |
| 70 | 585 | *.con | Dat585d.cfg | Tx, Fl, pH(2) | pH calibrated |
| 75 | 585 | *.con | Dat585d.cfg | Tx, Fl, pH(2) | PH voltage |

# SUMMARY OF QUALITY AND CONCERNS

The data was in good order and required only light editing. Header .hdr files on casts 22 and 30 were edited to remove data crossover between Longitude and “Start Time” information entered automatically by the deck unit. Processing was done with ..hdr files overriding the information in the .dat files.

Starting and ending pressure readings were reviewed and it was found that the ending values were consistently negative at the end of the cast although they were about correct at the beginning. It was decided to add 1 dbar to all pressures for CTD 550 – no corrections for CTD 585. This is not significant for most data but is important for comparing surface values with loop samples. This was accomplished using the confile “550_1-57.con”.

Static temperature comparisons at bottle points showed only small differences with 3 sensors agreeing perfectly and sensor 2023(primary of CTD 550) needing a .0005°C correction.

The pressure and temperature corrections were applied at the very beginning, “datcnv”, stage.

Comparison of temperatures(see plot below) measured simultaneously on the CTD’s showed differences of +.002°C and -.002 on down and up casts respectively. This is unlikely to be a time constant problem since the difference in time constants would have to be on the order of .84 seconds. Fouling? Difference in pump speeds? It raises questions on how accurately we are actually measuring temperature in a dynamic regime.

Conductivity noise on the upcast of cast 67 shifted the salinity down by .006. Looks like a fish problem for CTD 585.

PROCESSING SUMMARY

1. Seasave - This step was completed at sea; the raw data files are *.dat,, *.con, * hdr and *.bl.

2. Preliminary Steps

The salinities were analyzed on board and used to calibrate the ctd data. Hydro files were not available at the time of processing.

Prior to leaving IOS, calibrations in the .con files of both CTD’s used for data logging were checked and edited. Some problems (wrong set of cal constants) were corrected.

- Conversion of Raw Data

The raw data were converted (with salinity) using conversion 2025####.con where #### is the cast number. Rossum was run to get the .btl files and the data was summarized using get_btl.exe and entered into spreadsheet 2025_btl.xls. The bottle salinity and oxygen data were obtained from Bernard Minkley and comparisons between bottle and CTD data were made. The main and secondary sensors of CTD #550 were in good agreement: T0-T1 = -.0005, S00-S11 = .010 with the average salinity agreeing with the bottles. The temperature correction to T0 of +.0005 was made in the reprocessing of the data done on Sept. 18. A possible pressure effect in the conductivity sensors of CTD#585 was noted.

Converted data was run through the SeaBird processing schedule using a batch routine; finbatch.bat and process.bat. This was comprised of ALIGNCTD(CTD585 only), WILDEDIT, CELLTM, FILTER, and LOOPEDIT.

4. ALIGNCTD

The deck unit was configured to advance temperature by .073 seconds for both sensor pairs which compensates for the delay in the TC duct and the time constant of the temperature sensor so no alignment should be necessary. A comparison of different alignments between C and T were made and it was decided that no alignment was necessary on CTD #550 but that temperature should be advanced by one scan, .042 s, for CTD #585. The primary sensors on CTD #550 had less noise probably because they were not plumbed into the fluorometer flow.

# alignctd_date = Sep 12 2000 01:30:53, 4.246

# alignctd_in = 20250029.CNV

# alignctd_cond_advSecs = 0.000, 0.000

# alignctd_temp_advSecs = 0.000, 0.042

# alignctd_oxygen_current_advSecs = 0.000, 0.000

# alignctd_oxygen_temp_advSecs = 0.000, 0.000

# alignctd_wetstar_advSecs = 0.000

# filter_date = Sep 12 2000 01:33:18, 4.246

# filter_in = 20250029.CNV

5. WILDEDIT

Program WILDEDIT was used to remove spikes in Pressure.  Parameters used were:

Pass 1    Std Dev = 2		Pass 2    Std Dev = 5		Points per block = 50

# wildedit_date = Sep 12 2000 01:31:43, 4.246

# wildedit_in = 20250029.CNV

# wildedit_pass1_nstd = 2.0

# wildedit_pass2_nstd = 5.0

# wildedit_pass2_mindelta = 5.000e-002

# wildedit_npoint = 50

# wildedit_vars = pr

# wildedit_excl_bad_scans = no

6. CELLTM

The conductivity cell thermal mass correction was done for both channels (alpha = 0.03; 1/beta = 9.0.)

# celltm_date = Sep 12 2000 01:32:29, 4.246

# celltm_in = 20250029.CNV

# celltm_alpha = 0.0300, 0.0300

# celltm_tau = 9.0000, 9.0000

# celltm_temp_sensor_use_for_cond = primary, secondary

7. FILTER

Filter pressure as a preliminary for LOOPEDIT.

# filter_date = Sep 12 2000 01:33:18, 4.246

# filter_in = 20250029.CNV

# filter_low_pass_tc_A = 0.150

# filter_low_pass_tc_B = 0.100

# filter_low_pass_A_vars = pr

# filter_low_pass_B_vars =

8. LOOPEDIT

Mean speed was estimated using a 100 second time window and speeds less than 10% of mean were eliminated with a floor of 5 cm/s.

# loopedit_date = Sep 12 2000 01:34:49, 4.246

# loopedit_in = 20250029.CNV

# loopedit_minVelocity = 0.050

# loopedit_excl_bad_scans = yes.

9. BINAVG

To produce 1 dbar averages for analysis.

10. Derive

S(T0,C0) and S(T1,C1) were derived

10. Test Plots and Channel Check

A sample of casts (#8, 45 and 68) was plotted to check for agreement between the pairs of sensors. The temperature variations are very noisy but ignoring spikes are <0.001C° below 150db while the conductivity differences are about 0.001 S/m. Salinity values varied by ~0.01. Primary values were higher in each case. A quick check of CTD salinity versus bottle salinity suggests that the secondary straddle the bottle values, each being off by .005,.

Cast 46 has a huge temperature step at 41 dbar with  resulting salinity spike of over 1 ppt..Investigation shows that CTD550 should have been aligned as well as CTD 585 – the SeaBird deck unit compensation is not working. Temperature should be advanced by a further .046 seconds for S00 and .054 seconds for S11 based on this one feature.

Discussions with Germaine led to the conclusion that his may be a second order evvect or cell thermal mass effect and should not change the processing scheme based on smaller gradients.

9.  Conversion to IOS Headers

The IOSSHELL routine was used to convert SEA-Bird 911+ data to IOS Headers to produce .ios files. ITS90 temperatures were converted to IPTS68 using the calibrate program to produce .cal files. Records marked bad by the SeaBird software was deleted in this conversion.

10. Checking Headers

A header summary was produced and two header translation errors were found and corrected.

The surface check of CTD 550 was run. The average surface pressure is 1.125db which is a little low, expect about 2 dbar. Also, the upcast produces negative pressures at the last bottle point(also at about 2dbar). Therefore in the re-processing done on Sept.18, a 1 dbar offset was added to the pressures for CTD 550.

The header check was produced.

The cruise track was plotted and looks reasonable.

- DELETE

- The following DELETE parameters were used:

- Surface Record Removal: None

- Pressure filtered over width: 48

- Swells deleted. Warning message if pressure difference of  2.00

- Drop rates<  .3 m/s (calculated over 48 points) was deleted.

- Sample interval =       .042 seconds.

- The log was examined and all warnings found to pertain to the surface or upcast.

- Examples of salinity spiking under wavy conditions were plotted in high temperature gradient sections.

- The large spike in cast 57 was stable in density and may be real.

- The secondary sensors were used for cast 51 because of apparent fouling of the primary conductivity.

- 12. BIN AVERAGE

- The following Bin Average values were used:

- Bin channel = pressure

- Averaging interval = 1.000

Minimum bin value =  .000

- Figure 1. Some casts showing before and after SeaBird LOOPEDIT and the DELETE program in IOSSHELL. Some bad records have been deleted but many spikes large and small remain through the thermocline. It will be necessary to quote reduced accuracy in the depth range 35m to 55 m where large temperature gradients prevail and slow lowering in high wave states cause the pumps to ingest the rosette wake.. The worst data is connected to cast 52(check lowering rate). Binaveraging will smooth much of the high wavenumber spikes.

- Figure 2

- Average value will be used

- Interpolated values are NOT used for empty bins

- Test Plots

Figure 3

Profiles of all casts were examined for large differences between sets of sensors. The primary sensors in cast 51 were blocked, possibly by a jellyfish, at about 50 m so the secondary sensors were used.

Figure 4

Page plots were produced using T1,S1 for all casts. These plots were examined for spikes and instabilities and used to guide the use of CTDEDIT.

Figure 5 The spikes are sometimes on one sensor pair and sometimes on the other. LOWERING RATE IS NOT A FACTOR. From these plots, one gets the impression that the pumps may be intermittently slowing down leading to sensor mismatches.

14. CTDEDIT

CTDEDIT was used to clean noise in S and/or T near the surface or at the bottom for the following casts: #2,5,6,15,17,28,30,41,43,45,47,54,55,63,66,68,69,71,72 and 73.

CTDEDIT was used for mid-depth editing of the following casts: #42 and 52.

Note was made of the editing details in the relevant files.

15. Intercomparisons

Secondary temperature sensors were switched between CTD550 and CTD585 at cast 65 on Station P26 to link all the temperature sensors together. T2663 – T2371 = -.00013°C(Cast 68, deeper than 500 dbar), essentially zero. From the deep cast at P26, T2023 -   T2371 = -.0005. Therefore, if we take 2663 to be correct because it has the latest calibration and agrees well with 2668 which also has a recent calibration, then we should correct 2371 by +.0000 and 2023 by +.0005. The two T sensors originally used on CTD585, and with recent calibrations, need no correction. This correction was applied to the Sept. 18 re-processing.

16. Compare – A spreadsheet was used to compare bottle salinities with CTD data – 2025_btl.xls.

Cell constants for CTD 550 are shown below: there was no noticeable pressure dependence. Deep bottles are shown next at a higher resolution.

A detailed comparison of the cell constants shows some evolution with time with both cell constants following the same behavior. On inspection of the graph below, it is apparent that most of the change occurred between casts at P20 and P26. There was a fouling event on cast 51(P22), and bottle samples collected at the bottom of casts between P22 and P35 suggest that changes took place between casts 53(P24) and 57(P26), possibly a progressive fouling due to the presence of jellyfish, oil slicks, etc.. It is worthwhile noting that the winch wire was coated with an oily substance which sticks to everything. The first point on the graph represents P4 and because of the small number of deep calibrations, cannot be said to differ significantly from adjacent estimates, diff. < .001 ppt. Therefore, take :

(K0,K1)=(.99990,1.000164) up to cast 52:

(K0,K1)=(.99993,1.000185) cast 53 up to cast 55:

(K0,K1)=(.999978,1.000230) cast 57.

These corrections were applied to the IOSSHELL .cal files to produce .cor  files.

*In order to convert to conductivity ratio from conductivity, the SeaBird conductivities were divided by 4.2914.

For CTD 585, the agreement between the sensors was exceptionally good and the temperature sensors agreed well with those on CTD550. One deep bottle cast was achieved at station OE1(cast 68) and yielded the result that the cell constants were both in need of correction by an amount of about 20 ppm in salinity!

Therefore, take (K0,K1) = (1.00045,1.00045).

- Recalibration

The conversion from ITS90 to IPTS68 was done with the CALIBRATE program using 2025orig.ccf cal. control file to produce .cal files. The conductivity correction, and conversion to cond. ratio was done using 2025.ccf  to produce .cor files.

Table 2 Cell Constants

| CTD | Casts | K0 | K1 | K0, ratio | K1, ratio |
| --- | --- | --- | --- | --- | --- |
| 550 | 1-52 | 0.9999 | 1.000164 | 0.233001 | 0.233062 |
| 550 | 53, 56 | 0.99993 | 1.000185 | 0.233008 | 0.233067 |
| 550 | 57 | 0.999978 | 1.00023 | 0.233019 | 0.233078 |
| 585 | 21-28, 65-70 | 1.00045 | 1.00045 | 0.233129 | 0.233129 |

The corrected files were used as input to the following batch process:

Table 3

| Batch Step | Output File Extension | Purpose |
| --- | --- | --- |
| DESPIKE | .SPK | edit spikes out of T and C channels |
| DELETE | .DEL | delete swells and droprates less than .3 m/s |
| REMOVE | .REM | remove flag, SPAR, salinity |
| DERIVE | .SAL | compute salinity |
| BINAVG | AVG | 1 dbar averages |
| DERIVE | DER | add theta, gamma:theta, geo. Anom, T:ITS90 |

Salinity was recomputed using primary channels; secondary for cast 51 which had fouling in the primary conductivity to produce .sal files.

18. Final Plots

Profile and TS spaghetti plots were examined for problems.

Station P was compared Tabata’s averaqged data.

All OK.

Figure 6. Difference between primary and secondary salinities for downcast, 1 m average files. The bias at the surface is thought to be the effect of a different pumping rate for the secondary salinity duct which included the fluorometer. Only primary salinities were used for the final data.

19. REMOVE

The following channels were removed from all .der files: Number-of-scans, Primary &Secondary Temperature, Primary&Secondary Conductivity, Gamma-Theta, Geopotential anomaly, Theta.

20. Producing final files

a.) The final files were renamed *.ctd.

b.) A cross-reference listing was produced and is appended.

Particulars

Cast 20250051 – The primary duct ingested a fouling substance so the secondary sensors were used.

Cast 2025oe1 was renamed 20250068.

Institute of Ocean Sciences

CRUISE SUMMARY

Cruise ID#:	2025

Dates: 	Start: 4  Sept. 2000 	  End:	20 Sept., 2000

Location: Line P

Vessel:		CCGS  TULLY

Chief Scientist:	Howard Freeland

| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |
| --- | --- | --- | --- | --- | --- |
| 1 | SEABIRD | 911+ | 0550 | Yes | No |
| 2 | SEABIRD | 911+ | 0585 | No | No |
| 3 | SEABIRD | 911+ | 0585 | Yes (after cast 57) | No |

Institute of Ocean Sciences

CTD Calibration Information

Make/Model/Serial#:	SEABIRD/911+/	0550

Cruise ID#:		2025

| Calibration Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sensor |  | Pre-Cruise |  | Post Cruise |  |
| Name | S/N | Date | Location | Date | Location |
| Temperature | 2023 | 5 May1999 | Factory |  |  |
| Conductivity | 2173 | 5 May1999 | Factory |  |  |
| Secondary Temp. | 2371 | 13 Mar1999 | Factory |  |  |
| Secondary Cond. | 1766 | 16 Mar1999 | Factory |  |  |
| Transmissometer | 333DR | 14 July 2000 | IOS |  |  |
| Seapoint Fluorometer | 2229 |  |  |  |  |
| Seapoint Fluorometer | 2228 |  |  |  |  |
| PAR Sensor GSP-200L4S | 4565 | July 2000 |  |  |  |
| Pressure Sensor | 75636 | 6 Apr, 1999 | Factory |  |  |

Sensor Calibration Notes:

The configuration file 550_1_57.con has the corrections used in the DATCNV program. 2025orig.ccf and 2025.ccf have the corrections used in IOSSHELL.

To summarize, the pressure on CTD 550 was increased by 1 dbar and the temperature on sensor 2023 was increase by .0005 based only on comparisons with the other sensors.

Cell constants are summarized above.

Casts 75 was taken off Nanaimo after the batch processing was done with a net tow for UBC. It was processed individualyy.

Log file: D:\LineP_00\process\IOSshell\summary.log

==============================================================================

CRUISE HEADER SUMMARY

REPORT GENERATION DATE: 2000/09/19 14:01:20

==============================================================================

------------------------------------------------------------------------------

INPUT PARAMETERS

------------------------------------------------------------------------------

Source File Type   : ASCII

Header Type        : IOS

Source Directory   : D:\LineP_00\process\IOSshell\

Cruise Number      : 20-25

Cast Range         :    1-  70

File Name Extension: ios

------------------------------------------------------------------------------

MISCELLANEOUS SUMMARY

------------------------------------------------------------------------------

Total Number of Casts Found                   :           38

The Following Casts are Missing               : 6,8-12,18,23-25,31-38,43-45,47

-48,58-64,66,69

The Following Casts have the Wrong Consec #   :

The Following Casts have Blank Station #'s    :

------------------------------------------------------------------------------

CAST LIMITS

------------------------------------------------------------------------------

Max  Water

Event Station  Latitude   Longitude          Date          Depth Depth

----- ------- ---------- -----------  -------------------- ----- -----

First     1 SA 3    48 35.59 N 123 29.96 W  UTC 2000/09/05 01:05   215     0

Last     70 NOMADPH 51 22.83 N 135 28.35 W  UTC 2000/09/17 16:54   503     0

North    70 NOMADPH 51 22.83 N 135 28.35 W  UTC 2000/09/17 16:54   503     0

South     2 P 1     48 34.51 N 125 30.05 W  UTC 2000/09/05 14:07   111     0

East      1 SA 3    48 35.59 N 123 29.96 W  UTC 2000/09/05 01:05   215     0

West     65 MP26    49 59.69 N 145  0.30 W  UTC 2000/09/14 22:07   403  4228

Shallow    3 P 2     48 35.98 N 125 59.96 W  UTC 2000/09/05 16:48   111     0

Deep     57 MP26    50  0.06 N 144 59.82 W  UTC 2000/09/13 20:44  4258  4227

------------------------------------------------------------------------------

SPEED SUMMARY

------------------------------------------------------------------------------

The Following Casts Have Cruising Speeds:

Between     0 and     5 knots: 5-13,16-26,30-39,41,46-50,56-68

Between     5 and    10 knots: 2-4,14-15,27-29,40,42,51-55,70

Between    10 and    15 knots:

Between    15 and    20 knots:

Between    20 and    25 knots:

Between    25 and    30 knots:

Above      30 knots          :

------------------------------------------------------------------------------

CHANNEL SUMMARY

------------------------------------------------------------------------------

The Following are the Channels For Casts: 1-20,29-57

Pressure

Temperature:Primary

Temperature:Secondary

Conductivity:Primary

Conductivity:Secondary

Chl_fluorescence:Seapoint

Chl_fluorescence:Seatech

Transmissivity

Irradiance (PAR)

Surface Irradiance (PAR)

Salinity:T0:C0

Flag

The Following are the Channels For Casts: 21-28

Pressure

Temperature:Primary

Temperature:Secondary

Conductivity:Primary

Conductivity:Secondary

Transmissivity

Salinity:T0:C0

Flag

The Following are the Channels For Casts: 65-67

Pressure

Temperature:Primary

Temperature:Secondary

Conductivity:Primary

Conductivity:Secondary

Chl_fluorescence:Seapoint

Transmissivity

Salinity:T0:C0

Flag

The Following are the Channels For Casts: 68

Pressure

Temperature:Primary

Temperature:Secondary

Conductivity:Primary

Conductivity:Secondary

Chl_fluorescence:Seapoint

Transmissivity

Fluorometer:0

Fluorometer:1

Salinity:T0:C0

Flag

The Following are the Channels For Casts: 70

Pressure

Temperature:Primary

Temperature:Secondary

Conductivity:Primary

Conductivity:Secondary

Chl_fluorescence:Seapoint

Transmissivity

ph

Fluorometer:1

Salinity:T0:C0

Flag

------------------------------------------------------------------------------

DATA LIMITS over ALL CASTS

------------------------------------------------------------------------------

Channel                       Minimum Evnt Station        Maximum Evnt Station

---------------------- -------------- ---- ------- -------------- ---- -------

Pressure                        -0.98   26 P13           4258.606   57 MP26

Temperature:Primary            1.5001   57 MP26           98.9762   54 MP25

Temperature:Secondary          1.4998   57 MP26           16.3924   17 P8

Conductivity:Primary       -0.548E-03   21 P11           4.412875   54 MP25

Conductivity:Secondary      -0.550096   21 P11           4.136962   19 P9

Chl_fluorescence:Seapo     0.1301E-16   29 P16              14.87    2 P 1

Chl_fluorescence:Seate     0.8674E-18   13 P4              0.9653   29 P16

Transmissivity                      0   21 P11           70.33174   13 P4

Irradiance (PAR)                 1.54    7 P4                4017   56 MP26

Surface Irradiance (PA          -8838    5 P4                   0    1 SA 3

Salinity:T0:C0               0.79E-02   21 P11            34.6862   57 MP26

Flag                                0    1 SA 3                 0    1 SA 3

Fluorometer:0                 2.57387   68 OE1            2.78632   68 OE1

Fluorometer:1                 2.66422   70 NOMADPH        2.85836   70 NOMADPH

ph                              7.334   70 NOMADPH          8.119   70 NOMADPH

------------------------------------------------------------------------------

ADMINISTRATION SUMMARY

------------------------------------------------------------------------------

The Following are the Admin Items For Casts: 1-70

Cruise   : 2000-25

Agency   : IOS

Project  : LineP, Sept., 2000

Scientist: H. Freeland

Platform : J.P. Tully

------------------------------------------------------------------------------

INSTRUMENT SUMMARY

------------------------------------------------------------------------------

The Following Instrument Was Used For Casts: 1-20,29-57

Instrument Type         : Sea-Bird CTD

Instrument Model        : SBE-911plus

Instrument Serial Number: ctd 550 or 585

Sensor   1 : temperature # primary

Sensor   2 : conductivity # primary

Sensor   3 : pressure # 75636

Sensor   4 : temperature # secondary

Sensor   5 : conductivity # secondary

Sensor   6 : Seapoint

Sensor   7 : transmissometer # 333DR

Sensor   8 : fluorometer # sea tech

Sensor   9 : irradiance # 4565

Sensor  10 : surface # degrees =

The Following Instrument Was Used For Casts: 21-28

Instrument Type         : Sea-Bird CTD

Instrument Model        : SBE-911plus

Instrument Serial Number: ctd 550 or 585

Sensor   1 : temperature # primary

Sensor   2 : conductivity # primary

Sensor   3 : pressure # 77511

Sensor   4 : temperature # secondary

Sensor   5 : conductivity # secondary

Sensor   6 : transmissometer # 192D

Sensor   7 : surface # degrees =

The Following Instrument Was Used For Casts: 65-67

Instrument Type         : Sea-Bird CTD

Instrument Model        : SBE-911plus

Instrument Serial Number: ctd 550 or 585

Sensor   1 : temperature # primary

Sensor   2 : conductivity # primary

Sensor   3 : pressure # 77511

Sensor   4 : temperature # secondary

Sensor   5 : conductivity # secondary

Sensor   6 : Seapoint

Sensor   7 : transmissometer # 192D

Sensor   8 : surface # degrees =

The Following Instrument Was Used For Casts: 68

Instrument Type         : Sea-Bird CTD

Instrument Model        : SBE-911plus

Instrument Serial Number: ctd 550 or 585

Sensor   1 : temperature # primary

Sensor   2 : conductivity # primary

Sensor   3 : pressure # 77511

Sensor   4 : temperature # secondary

Sensor   5 : conductivity # secondary

Sensor   6 : Seapoint

Sensor   7 : transmissometer # 192D

Sensor   8 : userpoly # 180293

Sensor   9 : userpoly

Sensor  10 : surface # degrees =

The Following Instrument Was Used For Casts: 70

Instrument Type         : Sea-Bird CTD

Instrument Model        : SBE-911plus

Instrument Serial Number: ctd 550 or 585

Sensor   1 : temperature # primary

Sensor   2 : conductivity # primary

Sensor   3 : pressure # 77511

Sensor   4 : temperature # secondary

Sensor   5 : conductivity # secondary

Sensor   6 : Seapoint

Sensor   7 : transmissometer # 192D

Sensor   8 : pH # 180339

Sensor   9 : userpoly # 180293

Sensor  10 : surface # degrees =

------------------------------------------------------------------------------

PROCESSING HISTORY SUMMARY

------------------------------------------------------------------------------

The Following are the Processing Steps For Casts: 1-70

SBE_IOS

------------------------------------------------------------------------------

CALIBRATION SUMMARY

------------------------------------------------------------------------------

The Following are the Calibration Coefficients

For Casts: 1-70

$TABLE: RAW CHANNELS

!   Name Units Fmla Pad    Coefficients

!   ---- ----- ---- ------ ------------

$END

$TABLE: CALCULATED CHANNELS

!   Name Units Fmla Pad    Coefficients

!   ---- ----- ---- ------ ------------

$END

$TABLE: CORRECTED CHANNELS

!   Name Units Fmla Pad    Coefficients

!   ---- ----- ---- ------ ------------

$END

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Report Complete

$TABLE: CORRECTED CHANNELS

!   Name Units Fmla Pad    Coefficients

!   ---- ----- ---- ------ ------------

$END

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Report Complete

------------------------------------------------------------------------------

Input parameter file: C:\windows\TEMP\iosshell\IS192.inp

Log file: D:\LineP_00\process\IOSshell\summary.log

************************************************

*  Program: HSUMMARY                           *

*  Version: 1.2         Date: April 12, 2000   *

*                                              *

*  Copyright 2000 Government of Canada         *

*  Department of Fisheries and Oceans          *

*                                              *

*  Time of run:  2000/09/19 12:11:00           *

************************************************

Input file is in ASCII format.

Input path = D:\LineP_00\process\IOSshell\

Cruise year,cruise number =   20,25

First and last cast number =    1,  70

filename extension = ios

Summary listing will be produced.

1

1

Cruise  2000-

Cast no:   1     Station  SA 3                 Project  LineP, Sept.

Latitude  48 35.59 N          Longitude   123 29.96 W

Start date   5/ 9/ 0          Start time     1: 5 UTC

Maximum depth    215          No. of records    19476

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250001.DAT

Cast no:   2     Station  P 1                  Project  LineP, Sept.

Latitude  48 34.51 N          Longitude   125 30.05 W

Start date   5/ 9/ 0          Start time    14: 7 UTC

Maximum depth    111          No. of records     6550

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250002.DAT

Cast no:   3     Station  P 2                  Project  LineP, Sept.

Latitude  48 35.98 N          Longitude   125 59.96 W

Start date   5/ 9/ 0          Start time    16:48 UTC

Maximum depth    111          No. of records     8861

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250003.DAT

Cast no:   4     Station  P 3                  Project  LineP, Sept.

Latitude  48 37.54 N          Longitude   126 19.92 W

Start date   5/ 9/ 0          Start time    18:41 UTC

Maximum depth    808          No. of records    39316

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250004.DAT

Cast no:   5     Station  P4                   Project  LineP, Sept.

Latitude  48 39.03 N          Longitude   126 39.94 W

Start date   5/ 9/ 0          Start time    23: 9 UTC

Maximum depth    202          No. of records    15289

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250005.DAT

1

Cruise  2000-

Cast no:   7     Station  P4                   Project  LineP, Sept.

Latitude  48 39.04 N          Longitude   126 40.02 W

Start date   6/ 9/ 0          Start time     8:18 UTC

Maximum depth   1303          No. of records    70127

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250007.DAT

Cast no:  13     Station  P4                   Project  LineP, Sept.

Latitude  48 39.09 N          Longitude   126 39.98 W

Start date   6/ 9/ 0          Start time    18: 5 UTC

Maximum depth    401          No. of records    25289

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\202500013.DAT

Cast no:  14     Station  P5                   Project  LineP, Sept.

Latitude  48 41.56 N          Longitude   127 10.19 W

Start date   6/ 9/ 0          Start time    20:47 UTC

Maximum depth   2111          No. of records    87989

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250014.DAT

Cast no:  15     Station  P6                   Project  LineP, Sept.

Latitude  48 44.64 N          Longitude   127 39.94 W

Start date   7/ 9/ 0          Start time     0:10 UTC

Maximum depth   2573          No. of records   124192

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250015.DAT

Cast no:  16     Station  P7                   Project  LineP, Sept.

Latitude  48 46.58 N          Longitude   128 10.06 W

Start date   7/ 9/ 0          Start time     4:15 UTC

Maximum depth   2539          No. of records   118646

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250016.DAT

1

Cruise  2000-

Cast no:  17     Station  P8                   Project  LineP, Sept.

Latitude  48 48.99 N          Longitude   128 39.97 W

Start date   7/ 9/ 0          Start time     8: 2 UTC

Maximum depth   2544          No. of records   122530

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250017.DAT

Cast no:  19     Station  P9                   Project  LineP, Sept.

Latitude  48 51.46 N          Longitude   129  9.95 W

Start date   7/ 9/ 0          Start time    12:50 UTC

Maximum depth   2356          No. of records   111880

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250019.DAT

Cast no:  20     Station  P10                  Project  LineP, Sept.

Latitude  48 53.55 N          Longitude   129 40.05 W

Start date   7/ 9/ 0          Start time    17: 5 UTC

Maximum depth   2682          No. of records   125023

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250020.DAT

Cast no:  21     Station  P11                  Project  LineP, Sept.

Latitude  48 55.96 N          Longitude   130 10.11 W

Start date   7/ 9/ 0          Start time    22:11 UTC

Maximum depth   2702          No. of records    94959

No. of channels    8          Channel desc   P    %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250021.DAT

Cast no:  22     Station  P12                  Project  LineP, Sept.

Latitude  48 58.24 N          Longitude   130 39.83 W

Start date   1/ 1/**          Start time     0:11 UTC

Maximum depth   1505          No. of records    53635

No. of channels    8          Channel desc   P    %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250022.DAT

1

Cruise  2000-

Cast no:  26     Station  P13                  Project  LineP, Sept.

Latitude  49  2.64 N          Longitude   131 40.07 W

Start date   8/ 9/ 0          Start time    21: 5 UTC

Maximum depth   1504          No. of records    56150

No. of channels    8          Channel desc   P    %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250026.DAT

Cast no:  27     Station  P14                  Project  LineP, Sept.

Latitude  49  7.44 N          Longitude   132 40.05 W

Start date   9/ 9/ 0          Start time     3:55 UTC

Maximum depth   1506          No. of records    47871

No. of channels    8          Channel desc   P    %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250027.DAT

Cast no:  28     Station  P15                  Project  LineP, Sept.

Latitude  49 12.02 N          Longitude   133 39.89 W

Start date   9/ 9/ 0          Start time     9:50 UTC

Maximum depth   1503          No. of records    54843

No. of channels    8          Channel desc   P    %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250028.DAT

Cast no:  29     Station  P16                  Project  LineP, Sept.

Latitude  49 17.03 N          Longitude   134 39.86 W

Start date   9/ 9/ 0          Start time    16:14 UTC

Maximum depth   3673          No. of records   181604

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250029.DAT

Cast no:  30     Station  P16                  Project  LineP, Sept.

Latitude  49 17.05 N          Longitude   134 40.07 W

Start date   1/ 1/**          Start time     0:14 UTC

Maximum depth    403          No. of records    26625

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250030.DAT

1

Cruise  2000-

Cast no:  39     Station  P17                  Project  LineP, Sept.

Latitude  49 21.08 N          Longitude   135 39.94 W

Start date  10/ 9/ 0          Start time    17:51 UTC

Maximum depth   1502          No. of records    72555

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250039.DAT

Cast no:  40     Station  P18                  Project  LineP, Sept.

Latitude  49 25.98 N          Longitude   136 40.19 W

Start date  10/ 9/ 0          Start time    23:10 UTC

Maximum depth    152          No. of records     6731

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250040.DAT

Cast no:  41     Station  P18                  Project  LineP, Sept.

Latitude  49 26.02 N          Longitude   136 40.35 W

Start date  10/ 9/ 0          Start time    23:24 UTC

Maximum depth   1503          No. of records    60162

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250041.DAT

Cast no:  42     Station  P19                  Project  LineP, Sept.

Latitude  49 29.98 N          Longitude   137 39.94 W

Start date  11/ 9/ 0          Start time     5:10 UTC

Maximum depth   1504          No. of records    62471

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250042.DAT

Cast no:  46     Station  P 20                 Project  LineP, Sept.

Latitude  49 33.94 N          Longitude   138 40.01 W

Start date  11/ 9/ 0          Start time    13:34 UTC

Maximum depth   4030          No. of records   210602

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250046.DAT

1

Cruise  2000-

Cast no:  49     Station  P 20                 Project  LineP, Sept.

Latitude  49 34.05 N          Longitude   138 39.96 W

Start date  11/ 9/ 0          Start time    21:19 UTC

Maximum depth    403          No. of records    19257

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250049.DAT

Cast no:  50     Station  P 21                 Project  LineP, Sept.

Latitude  49 37.94 N          Longitude   139 39.98 W

Start date  12/ 9/ 0          Start time    14:35 UTC

Maximum depth   1502          No. of records    68497

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250050.DAT

Cast no:  51     Station  MP22                 Project  LineP, Sept.

Latitude  49 41.91 N          Longitude   140 39.95 W

Start date  12/ 9/ 0          Start time    19:43 UTC

Maximum depth   1504          No. of records    64629

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250051.DAT

Cast no:  52     Station  MP23                 Project  LineP, Sept.

Latitude  49 45.90 N          Longitude   141 40.05 W

Start date  13/ 9/ 0          Start time     0:50 UTC

Maximum depth   1502          No. of records    71623

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250052.DAT

Cast no:  53     Station  MP24                 Project  LineP, Sept.

Latitude  49 50.21 N          Longitude   142 39.97 W

Start date  13/ 9/ 0          Start time     5:55 UTC

Maximum depth   1501          No. of records    68540

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250053.DAT

1

Cruise  2000-

Cast no:  54     Station  MP25                 Project  LineP, Sept.

Latitude  49 59.96 N          Longitude   143 36.32 W

Start date  13/ 9/ 0          Start time    11: 2 UTC

Maximum depth   1503          No. of records    75674

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250054.DAT

Cast no:  55     Station  MP35                 Project  LineP, Sept.

Latitude  50  0.03 N          Longitude   144 18.31 W

Start date  13/ 9/ 0          Start time    14:59 UTC

Maximum depth   1501          No. of records    72704

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250055.DAT

Cast no:  56     Station  MP26                 Project  LineP, Sept.

Latitude  50  0.05 N          Longitude   144 59.95 W

Start date  13/ 9/ 0          Start time    20:24 UTC

Maximum depth    202          No. of records    12537

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250056.DAT

Cast no:  57     Station  MP26                 Project  LineP, Sept.

Latitude  50  0.06 N          Longitude   144 59.82 W

Start date  13/ 9/ 0          Start time    20:44 UTC

Maximum depth   4258          No. of records   200259

No. of channels   12          Channel desc   P      %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250057.DAT

Cast no:  65     Station  MP26                 Project  LineP, Sept.

Latitude  49 59.69 N          Longitude   145  0.30 W

Start date  14/ 9/ 0          Start time    22: 7 UTC

Maximum depth    403          No. of records    21511

No. of channels    9          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250065.DAT

1

Cruise  2000-

Cast no:  67     Station  MP26                 Project  LineP, Sept.

Latitude  50  0.04 N          Longitude   144 59.79 W

Start date  15/ 9/ 0          Start time     8: 9 UTC

Maximum depth    405          No. of records    18032

No. of channels    9          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250066.DAT

Cast no:  68     Station  OE1                  Project  LineP, Sept.

Latitude  51  0.03 N          Longitude   144  0.30 W

Start date  16/ 9/ 0          Start time     5: 7 UTC

Maximum depth   1002          No. of records    45643

No. of channels   11          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\2025OE1.DAT

Cast no:  70     Station  NOMA                 Project  LineP, Sept.

Latitude  51 22.83 N          Longitude   135 28.35 W

Start date  17/ 9/ 0          Start time    16:54 UTC

Maximum depth    503          No. of records    28800

No. of channels   11          Channel desc   P     %

Comments:

SBE HEADER

Sea-Bird SBE 9 Raw Data File:

FileName = C:\data\2000-25\20250070.DAT

1

Cruise

Start of run: 2000/09/19 12:11:00.42

End of run:   2000/09/19 12:11:18.32

*** Summary Report ***                     19-SEP-2000

Page   1

Filename          Event  Station  Latitude    Longitude    Date       Time

----------------  -----  -------  ----------  -----------  ---------- ---------

20250001.thn   0001  SA 3     48 35.59 N  123 29.96 W  2000/09/05 01:05 UTC

20250001.BOT    0001  SA 3     48 35.59 N  123 29.96 W  2000/09/05 01:05 UTC

20250002.thn   0002  P 1      48 34.51 N  125 30.05 W  2000/09/05 14:07 UTC

20250003.thn   0003  P 2      48 35.98 N  125 59.96 W  2000/09/05 16:48 UTC

20250004.thn   0004  P 3      48 37.54 N  126 19.92 W  2000/09/05 18:41 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250004.BOT    0004  P 3      48 37.54 N  126 19.92 W  2000/09/05 18:41 UTC

20250005.thn   0005  P4       48 39.03 N  126 39.94 W  2000/09/05 23:09 UTC

20250007.thn   0007  P4       48 39.04 N  126 40.02 W  2000/09/06 08:18 UTC

20250007.BOT    0007  P4       48 39.04 N  126 40.02 W  2000/09/06 08:18 UTC

20250013.thn   0013  P4       48 39.09 N  126 39.98 W  2000/09/06 18:05 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250013.BOT    0013  P4       48 39.09 N  126 39.98 W  2000/09/06 18:05 UTC

20250014.thn   0014  P5       48 41.56 N  127 10.19 W  2000/09/06 20:47 UTC

20250014.BOT   -9991  o       ***  0.00    ***  0.00    ****/00/00***:00 o

20250015.thn   0015  P6       48 44.64 N  127 39.94 W  2000/09/07 00:10 UTC

20250015.BOT    0015  P6       48 44.64 N  127 39.94 W  2000/09/07 00:10 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250016.thn   0016  P7       48 46.58 N  128 10.06 W  2000/09/07 04:15 UTC

20250016.BOT    0016  P7       48 46.58 N  128 10.06 W  2000/09/07 04:15 UTC

20250017.thn   0017  P8       48 48.99 N  128 39.97 W  2000/09/07 08:02 UTC

20250017.BOT    0017  P8       48 48.99 N  128 39.97 W  2000/09/07 08:02 UTC

20250019.thn   0019  P9       48 51.46 N  129  9.95 W  2000/09/07 12:50 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250019.BOT    0019  P9       48 51.46 N  129  9.95 W  2000/09/07 12:50 UTC

20250020.thn   0020  P10      48 53.55 N  129 40.05 W  2000/09/07 17:05 UTC

20250020.BOT    0020  P10      48 53.55 N  129 40.05 W  2000/09/07 17:05 UTC

20250021.thn   0021  P11      48 55.96 N  130 10.11 W  2000/09/07 22:11 UTC

20250022.thn   0022  P12      48 58.24 N  130 39.83 W  2000/09/08 10:31 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250026.thn   0026  P13      49  2.64 N  131 40.07 W  2000/09/08 21:05 UTC

20250027.thn   0027  P14      49  7.44 N  132 40.05 W  2000/09/09 03:55 UTC

20250028.thn   0028  P15      49 12.02 N  133 39.89 W  2000/09/09 09:50 UTC

20250029.thn   0029  P16      49 17.03 N  134 39.86 W  2000/09/09 16:14 UTC

20250029.BOT    0029  P15      49 17.03 N  134 39.86 W  2000/09/09 16:14 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250030.thn   0030  P16      49 17.05 N  134 40.07 W  2000/09/09 19:58 UTC

20250030.BOT    0030  P16      49 17.05 N  134 40.07 W  2000/09/09 19:58 UTC

20250039.thn   0039  P17      49 21.08 N  135 39.94 W  2000/09/10 17:51 UTC

20250039.BOT    0039  P17      49 21.08 N  135 39.94 W  2000/09/10 17:51 UTC

20250040.thn   0040  P18      49 25.98 N  136 40.19 W  2000/09/10 23:10 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250041.thn   0041  P18      49 26.02 N  136 40.35 W  2000/09/10 23:24 UTC

20250041.BOT    0041  P18      49 26.02 N  136 40.35 W  2000/09/10 23:24 UTC

20250042.thn   0042  P19      49 29.98 N  137 39.94 W  2000/09/11 05:10 UTC

20250042.BOT    0042  P19      49 29.98 N  137 39.94 W  2000/09/11 05:10 UTC

20250046.thn   0046  P 20     49 33.94 N  138 40.01 W  2000/09/11 13:34 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250046.BOT    0046  P 20     49 33.94 N  138 40.01 W  2000/09/11 13:34 UTC

20250049.thn   0049  P 20     49 34.05 N  138 39.96 W  2000/09/11 21:19 UTC

20250049.BOT    0049  P 20     49 34.05 N  138 39.96 W  2000/09/11 21:19 UTC

20250050.thn   0050  P 21     49 37.94 N  139 39.98 W  2000/09/12 14:35 UTC

20250050.BOT    0050  P 21     49 37.94 N  139 39.98 W  2000/09/12 14:35 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250051.thn   0051  MP22     49 41.91 N  140 39.95 W  2000/09/12 19:43 UTC

20250051.BOT    0051  MP22     49 41.91 N  140 39.95 W  2000/09/12 19:43 UTC

20250052.thn   0052  MP23     49 45.90 N  141 40.05 W  2000/09/13 00:50 UTC

20250052.BOT    0052  MP23     49 45.90 N  141 40.05 W  2000/09/13 00:50 UTC

20250053.thn   0053  MP24     49 50.21 N  142 39.97 W  2000/09/13 05:55 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250053.BOT    0053  MP24     49 50.21 N  142 39.97 W  2000/09/13 05:55 UTC

*** Summary Report ***                     19-SEP-2000

Page   2

Filename          Event  Station  Latitude    Longitude    Date       Time

----------------  -----  -------  ----------  -----------  ---------- ---------

20250054.thn   0054  MP25     49 59.96 N  143 36.32 W  2000/09/13 11:02 UTC

20250054.BOT    0054  MP25     49 59.96 N  143 36.32 W  2000/09/13 11:02 UTC

20250055.thn   0055  MP35     50  0.03 N  144 18.31 W  2000/09/13 14:59 UTC

20250055.BOT    0055  MP35     50  0.03 N  144 18.31 W  2000/09/13 14:59 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250056.thn   0056  MP26     50  0.05 N  144 59.95 W  2000/09/13 20:24 UTC

20250057.thn   0057  MP26     50  0.06 N  144 59.82 W  2000/09/13 20:44 UTC

20250057.BOT    0057  MP26     50  0.06 N  144 59.82 W  2000/09/13 20:44 UTC

20250065.thn   0065  MP26     49 59.69 N  145  0.30 W  2000/09/14 22:07 UTC

20250065.BOT    0065  MP26     49 59.69 N  145  0.30 W  2000/09/14 22:07 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

20250067.thn   0067  MP26     50  0.04 N  144 59.79 W  2000/09/15 08:09 UTC

20250067.BOT    0067  MP26     50  0.04 N  144 59.79 W  2000/09/15 08:09 UTC

20250068.thn   0068  OE1      51  0.03 N  144  0.30 W  2000/09/16 05:07 UTC

20250068.BOT    0068  OE1      51  0.03 N  144  0.30 W  2000/09/16 05:07 UTC

20250070.thn   0070  NOMADP   51 22.83 N  135 28.35 W  2000/09/17 16:54 UTC

----------------  -----  -------  ----------  -----------  ---------- ---------

2000-25 Notes on reprocessing – see also updated Processing Report

General Notes

There were 2 fluorometers on this cruise, one pumped, the other not. In the original processing the fluorometer data was not recalibrated and was not filtered.

When reprocessed in August 2001 it was believed that the 10X cable was used for both instruments. However, further investigations in March 2002 show that the values at Station Papa are too high and that the chlorophyll values from bottles at stations P4, P16, P20 ,P26 and OE1 are much lower than those from the fluorometer. This lead to the conclusion that the 10X cable was only used for the first cast and that the 30X cable was used for all subsequent casts. The calibrations applied in August 2001 were (–0.045μg/l, 1) for #2228 and (–0.06μg/l, 15 for #2229). In March 2022 for casts #2 through 75 all fluorometer values were divided by 3 using calibration file 2025FL2.ccf.

Problems were found with the unpumped fluorometer (#2229) for casts #29 through 57. The data is clearly bad and the channel was removed from those casts. For casts #65 to 75 the pumped fluorometer was removed from the CTD and the configuration file was changed so that the unpumped instrument was then listed as SeaPoint with a 10X gain. This may be confusing since that was how the pumped instrument is listed at the beginning of the cruise.

# FILES FOR IOS_LIB ARCHIVE

The output files from CTDEDIT (EDT) were not saved so the usual method of reprocessing can not be used; this will make no difference to the fluorometer data since only T and S were cleaned and no records were removed in the editing process. For Archive purposes the following steps were applied:

REMOVE was run on the IOS files to strip all channels except PRESSURE, and the two FLUOROMETER channels.

DELETE routine was rerun using the same parameters that were used in the original processing.

Both fluorometer channels were then filtered using a median filter size 11.

Bin-average was used with 1m bins and interpolated values for empty bins as in the original processing.

Merge was used to combine all channels from the original CTD files (except fluorometer channels) and the fluorometer channels from the new AVG files.

Those files were then recalibrated using 2025FL.ccf.

Remove was used to delete the data from the #2229 fluorometer from casts 29-57.

The new files were renamed CTD for placement in the archive.

The CHE files from the IOS library archive were recalibrated using 2025FL.ccf and renamed CHE.

As explained above files (CTD and CHE) for casts #2 through 75 were recalibrated in March 2002 using 2025FL2.ccf.

# FILES FOR Angelica Pena

Files were prepared for the use of Angelica Pena using the following steps:

DELETE was run on the IOS files using Delete Low Drop Rate parameters 0.3m/s and 15 points. This will remove a little more data than was done in the original processing.

A median filter size 11 was applied to both fluorometer channels.

Recalibration was done using 2025FL.ccf to correct fluorometer data.

NOTE SALINITY HAS NOT BEEN RECALIBRATED IN THESE FILES. The method used in the original processing is complicated and was not important to the researcher.

Bin-average was then run using 0.25m bins and interpolation was not used for empty bins.

Reorder was used to fix formats and remove unnecessary channels.

Casts #21-28 were deleted since there was no fluorometer data.

The unpumped FL data was removed from casts #29-57 since it was bad.

The rosette files saved were not suitable for this analysis (some channels were missing and SPAR channel was empty and name not in Parameter list) so they were reconverted. The fluorometer calibrations were applied using new configuration files 2025fl1.con, 2025FL2.con….20225FL5.con. The files were then converted to IOS SHELL format, and sent to Angelica Pena for her use.

As explained above files for casts #2 through 75 were recalibrated in March 2002 using 2025FL2.ccf.

# Moving files to new archive. April 17, 2003 – Joe Linguanti

- Program HEADER EDIT was used to change non-standard channel names and units and add additional metadata to the CTD and Rosette files.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files .CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.* “.

- The loop file was obtained from Frank Whitney. The spreadsheet file was converted to IOS Header format with meta-data added to it. The original .XLS file is in the DOC directory. Bernard’s original loop file is in the HISTORY directory.

- The thermosalinograph files were copied from the TRACKOBS directory in the OSAP Data Archive.

- Program HEADER EDIT was  used to change non-standard channel names and units and add additional metadata to the files.

- IOS Header Date and Time channels were added.

- Channels were re-ordered

- Program Clean was used to set the start and end times and the geographical area.