## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 31-Mar-2004 | Rosette files were copied from Bernard’s disk. |

PROCESSING NOTES

Cruise: 2000-27

Agency: OSAP

Location: Strait of Georgia / Juan de Fuca

Project: Strait of Georgia

Party Chief: Diane Masson

Platform: CCGS VECTOR

Date: 28 August 2000 –2 September 2000

Processed by: Germaine Gatien

Date of Processing: 19 January 2001 –23 January 2001

Number of original CTD casts: 79

Number of casts processed: 76

INSTRUMENT SUMMARY

A SeaBird Model SBE 911+ CTD  (#0550) was used.

SUMMARY OF QUALITY AND CONCERNS

The data was generally in good order.

PROCESSING SUMMARY

1. Seasave - This step was completed at sea; the raw data files have extension DAT.

2. Preliminary Steps

The Log Book was obtained.

Salinity calibration data was obtained.

The cruise summary sheet was completed.

The configuration file was obtained and the calibration constants were checked.

The sensor history was found.

- Conversion of Raw Data

There was difficulty converting this data. For all casts the ITS-90 calibration coefficients for the secondary temperature were entered in the ITS-68 section (a,b,c…rather than g,h,…). The primary temperature calibrations were entered in the usual way. The corrected file is named 2027CTD1.CON.

For casts #1-6 a problem was encountered with the software. The system is expecting NMEA positions to be logged, but cannot find them. This was handled by editing 2027CTD1.CON to turn off the NMEA feature; this con file was named 2027CTD2.CON.

The raw data was converted using conversion file 2027CTD2.con for casts #1 to #6 and 2027CTD1.con for casts #7 to #81.

An initial examination of the data suggests that the sensors tracked each other reasonably well.

4. ALIGNCTD

It is not clear whether the secondary conductivity sensor has been aligned by the deck unit. As found for other cruises when the same equipment was used, the noise in the salinity appears to be primarily associated with the temperature channel. For those cruises the primary sensors were chosen for archiving. It was decided to skip the alignment step for this data set.

5. WILDEDIT

Program WILDEDIT was used to remove spikes in Pressure.  Parameters used were:

Pass 1    Std Dev = 2

Pass 2    Std Dev = 5

Points per block = 50

6. CELLTM

The conductivity cell thermal mass correction was done for both channels (alpha = 0.03; 1/beta = 9.0.)

7. STRIP

The salinity channel was stripped from the CNV files so that DERIVE doesn’t create a 2nd set of salinity channels.

7. DERIVE

Program DERIVE was run twice:

- on all casts to calculate primary and secondary salinity.

- on all casts to calculate the differences between primary and secondary channels for temperature, conductivity and salinity. These were placed in a test directory and will not be archived.

8. Test Plots and Channel Check

A sample of casts (#3,45,54 and 69) was plotted to check for agreement between the pairs of T and C sensors. The descent rate looks quite good at these stations. The differences in T, S and C are on the order of –0.001 to –0.005C, -0.01units and –0.001units, respectively. The temperature differences were extremely high particularly at the beginning of the cruise; the secondary temperature is much noisier than the primary. The conductivity and salinity differences are very similar to those observed during 2000-26.

The two fluorometry channels and the transmissivity data look reasonable.

9.  Conversion to IOS Headers

The IOSSHELL routine was used to convert SEA-Bird 911+ data to IOS Headers. A header error in cast #37 was fixed to enable conversion.

The ROS files were converted to IOS files, and the extensions were changed to BOT.

10. Checking Headers

A header summary was produced and a number of errors found and corrected. The file names of the first two casts had been reversed; the positions and times are consistent with the station names so the file names were changed to match those. The station names were wrong for 10 casts.

The surface check was run. The average surface pressure is 1.58db.

The header check was produced and no errors were found.

The cruise track was plotted and looks reasonable.

11. Test Plots

Profiles of all casts were examined for any evidence of problems with the processing. Casts #2 and 4 show great variability between up and down casts, but the two sets of sensors track reasonably well and the area is one known for a lot of structure.

- DELETE

The following DELETE parameters were used:

Surface Record Removal: Last Press Min

Maximum Surface Pressure (relative): 20.00

Surface Swell Pressure Tolerance: 1.0

Swells deleted. Warning message if pressure difference of 2.00

Drop rates < 0.3m/s (calculated over 15 points) was deleted.

Sample interval =  .04 seconds.

COMMENTS ON WARNINGS

Most warnings refer to surface and upcasts only. All mid-depth warnings were investigated and problems were discovered with the following casts:

#30 – There was an initial descent to about 15m when it was discovered that the pump was not on. The CTD was then brought to the surface and lowered again with the pump on. To ensure that DELETE selects the proper data an editor was used to remove the bad data (1870 records) from the beginning of the IOS file and then DELETE was rerun.

#39  - This file contained only surface data so was not processed any further.

All DEL files were copied to EDT files.

- DETAILED EDITING

Page plots were produced using T0,S0 for all casts. These plots were examined for spikes and instabilities and used to guide the use of CTDEDIT.

CTDEDIT was used to clean noise in S and T near the surface and bottom for casts: 7,11,12, 15,20-22,24,27,28,30,32,33,35,37,42,44,47-51,60,61,63-66,68,70,72,75,77,78,81.

CTDEDIT was used for more extensive cleaning in casts: 1,2,5,6,36,53,67,69,71, 79.

Note was made of the editing details in the relevant files. The edited files were copied to EDT files so that a complete set of files exists with either edited data or data that does not require editing.

Plots of temperature and salinity vs depth for the BOT files were examined to check for any bad values in these files. No obvious bad points were found although there is a fairly large range of observed values associated with bottles due to the fact that this is an area of large currents and great variability.

14. BIN AVERAGE

The following Bin Average values were used for EDT files:

Bin channel = pressure

Averaging interval = 1.000

Minimum bin value =   .000

Average value will be used

Interpolated values are NOT used for empty bins

The same values were used for the BOT files except that the Bin Channel = Bottle Number.

15. Intercomparisons

Previous experience with these sensors – These sensors have been used extensively since the last calibration and the comparison with bottles is as follows:

| Mission | Start Date | Sensors 2173/2023 | Sensors 1766/2371 |
| --- | --- | --- | --- |
| 2000-10 | 30/5/2000 | High by 0.0058 (used for part of cruise) | Low by 0.0035 |
| 2000-24 | 24/7/2000 | High by 0.003 (few bottles – deeper 2 give 0.006) | Low by 0.006 (few bottles deeper 2 give 0.007) |
| 2000-26 | 21/8/2000 | High by 0.005 | Low by 0.005 |
| 2000-25 | 4/9/2000 | High by 0.004 (varied during cruise) | Low by 0.007 (varied during cruise) |

COMPARE – Comparison with the rosette bottle salinities produced a very noisy result. When individual casts were examined and bottles used only if the temperature and salinity gradients were low the results improved dramatically. The primary salinities were found to be high by 0.0042units and the secondary were low by 0.005units. This is consistent with results on previous cruises.

Historic ranges - Plots were examined with historic ranges superimposed for casts #6,11 and 51 and all data fell within the ranges.

- Recalibration

File 2027rcal.ccf was used to lower the primary salinity by 0.00042units and to raise the secondary salinity by 0.005units. The recalibration was applied to the cast files, bottle files and the SAM files. The latter were used to re-run COMPARE with output 2027com1.xls. The results show that the recalibration was successful; average differences between bottles and CTD values are within 0.0001units for both channels.

17. Final Plots

THIN and DERIVE were run to obtain values for tables and page plots were prepared using the edited and recalibrated data.

- REORDER

REORDER was run to ensure that the number of decimals reflect the precision and to remove the following channels from all casts: Secondary Temperature, Secondary Salinity, Conductivity:Primary, Conductivity:Secondary and Flag.

For the bottle files only the two conductivity channels and flag were removed.

19. Producing final files

a.) The final CTD files were renamed CTD; the rosette files were renamed RAC.

b.) A cross-reference listing was produced.

c.) The sensor history was updated.

### Particulars

1.2 These two cast file names were reversed. The positions and times are right for the station named. The files were renamed correctly.

2.Large wire angle

6. Log notes problem with software, cast 7 is a repeat of this station using DOS software. Data looks ok.

8,9 – Wrong station names

13 – No upcast data recorded; not a rosette cast.

14 – Delete. New cast (#15) was started for the same site.

18 – Stopped during upcast.

19 – Continuation of upcast.  The hydro files for 18 and 19 were merged as 20270018.hyd.

28 – File name may be wrong in header

30 – CTD went to about 15m, then returned to top and started again because the pump wasn’t on.

39 – Only surface data. No mention of problem in the CTD log.  Deleted.

40,41 – Files not saved.

65 – Ran pumps 5 minutes at surface to clear mud. There were no bottles for the previous cast so the mud will not have affected calibration data.

# Institute of Ocean Sciences

CRUISE SUMMARY

Cruise ID#: 2000-27

Dates: Start: 28 August 2001 End: 2 September 2000

Location: Strait of Georgia / Juan de Fuca Strait

Vessel: CCGS VECTOR

Chief Scientist: Diane Masson

| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |
| --- | --- | --- | --- | --- | --- |
| 1 | SEABIRD | 911+ | 0550 | Yes | yes |

Institute of Ocean Sciences

# CTD Calibration Information

Make/Model/Serial#:	SEABIRD/911+/	0550

Cruise ID#:		2000-27

| Calibration Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sensor |  | Pre-Cruise |  | Post Cruise |  |
| Name | S/N | Date | Location | Date | Location |
| Temperature | 2023 | 07Dec1999 | Factory |  |  |
| Conductivity | 2173 | 05May2000 | “ |  |  |
| Secondary Temp. | 2371 | 16June1999 | “ |  |  |
| Secondary Cond. | 1766 | 28Dec2000 | “ |  |  |
| Transmissometer | CST333DR |  | IOS |  |  |
| Fluorometer:pumped | 2228 |  |  |  |  |
| Fluorometer:unpumped | 2229 |  | ? |  |  |
| Pressure Sensor | 75636 | 4June1999 | Factory |  |  |

Sensor Calibration Notes:

The configuration file used is attached; this includes the sensor calibrations.