# PROCESSING NOTES

Cruise: 2000-38

Agency: PBS

Project: Beam

Geographic Area: Strait of Georgia / Juan de Fuca Strait

Chief Scientist: Beamish R.

Platform: W. E. Ricker

Date of Cruise: 7 July 2000- 24 July 2000

Processed by: Germaine Gatien

Date of Processing: 23 May 2005 – 24 May 2005

Number of original casts:  87 (1 empty, 44 downcasts, 42 upcasts)

Number of casts processed: 27

# INSTRUMENT SUMMARY

One CTD was used during this cruise.

Instrument Type:  Guildline 8715 CTD

Instrument Serial Number: 53501

# SUMMARY OF QUALITY AND CONCERNS

There was no calibration sampling available. In previous use (in early 1998) the salinity was found to be within ±0.005psu for most samples. This instrument is generally quite stable, so an estimate of ±0.01psu is reasonable for this cruise. Only 3 decimal places will be archived for salinity.

The conductivity was bad for a number of casts; these will not be archived. A few other casts have small patches of bad data which were removed using a graphical editor. While there remain some doubts about the quality of the rest of the data in those casts, they look close to the upcasts and the data fall within the local climatology so they will be archived.

# PROCESSING SUMMARY

- PRELIMINARY STEPS

There was no Log Book available, but there is a spreadsheet that contains some data, positions, station names and a few comments.

The cruise summary sheet was completed.

A cruise track plot was made and it looks reasonable.

- CONVERSION OF RAW DATA

- The files were converted using the IOSSHELL routine to convert GUILDLINE files.

- CALIBRATION

- All data were calibrated using file 2000-38cal.ccf which was based on file 9818cal.ccf.

- The header summary and header check were examined. There are no station names in the headers.

- There were separate downcast and upcast files. Upcast files will not be processed further.

- Station names are available for some casts in the spreadsheet “Together Ricker July.xls”. Where it is clear, station names were entered in the headers for the downcasts. At the same time, the year in the start and stop times were changed from 1980 to 2000.

- The surface check for the downcast files gave an average surface pressure of +1.3m, but there is a lot of variability. The pressure is unreliable during the soak period of the downcasts as the pressure sensor equilibrates. Individual upcast files were examined and these show that the pressure is reading within 1db of zero when the conductivity is near “out-of-water” values, with some pressures a little high, others a little low, so there is no systematic error.

- It was noticed at this point that a number of casts have bad conductivity data; this was noted in the spreadsheet for some of these casts. Casts 37 through 52, 64 and 72 through 76 have bad or suspicious data. The pressure and temperature may be ok at depth, but the temperature falls outside the local climatology for the top 15db to 40db of the cast. Given that the historic range is very broad, there is little comfort in finding some of the data within the ranges. Those casts will not be processed further. There are a few other casts that have bad data very close to the surface; these may be ok below that but will be examined carefully later.

- DESPIKE

- CLEAN was used to remove pad values by interpolation.

- The following ADVANCED DESPIKE parameters were used:

- CHANNEL        FIT   OVER-  MIN       MAX       MIN         MAX         SPIKE

- WIDTH LAP    VALUE   VALUE  STDDEV  STDDEV     TOL.

- -------------------- ----- ----- ------- ------- ------- ------- -----

- Pres 		55     5        -2.00       4000.00	.10000        999.000    3.00

- Temp   	55     5        -2.00          15.00	.00500        999.000    3.00

- Cond    	55     5           .00            1.50	.00010        999.000    3.00

- A few plots were examined to ensure that salinity spikes were removed successfully, and they were.

- TIME COMPENSATION

- Temp. probe Dist (m):   .0700  Sample period (sec):   .0400

- DELETE

- The following DELETE parameters were used:

- Surface Record Removal: Low Sal & Last Press Min

- Maximum Surface Pressure (relative):      10.00

- Minimum Surface Salinity:     5.000

- Surface Swell Pressure Tolerance:       1.00

- Pressure filtered over width:   11

- Swells deleted. Warning message if pressure difference of      2.00

- Drop rates<    .30m/s (calculated over  11 points) will be deleted

- Drop rate applies in the range    10db to 10db less than the maximum pressure

- Sample interval =       .04 seconds.

- There were no warnings.

- FILTER

- A median filter with filter width 15 was applied to the salinity channel.

- CTDEDIT

CTDEDIT was used to edit temperature and salinity. Most casts required only light editing.

Cast #2 had sections of bad data – the conductivity is clearly bad and there are doubts about pressure, so the records were removed from those sections.

Casts #6, 58 & 87 required no editing.

All other casts required light editing.

Details were added as comments to the headers.

9.   BIN AVERAGE

The following Bin Average values were used:

Bin channel = pressure

Averaging interval =      1.000

Minimum bin value =       .000

Average value will be used

Interpolated values are NOT used for empty bins

10.Final Plots

Profile plots, with historic ranges superimposed, were examined on-screen. Problems were found in casts 66 and 78 and so those casts will not be processed further. All other data were found to fall within the local climatology, but cast #2 looks odd. Given there were problems with bad conductivity at several levels it was decided to not process this cast further.

The bin-averaged files were thinned to standard depths and then put through DERIVED QUANTITIES to calculate theta, gamma, depth, SVA and speed of sound.

Page plots were produced.

11. Quality Control Tests

There were no bottle data available.

There were no repeat downcasts, but there were a few casts that were close together (21, 31 and 33; 62 and 68). These were plotted together. The only suspicious plot was cast #33 which looked quite different from #21 and #31 from 45db to 55db. However the upcast of #33 has the same feature that makes the downcast look different, so these data are probably fine.

This CTD was used for 4 cruises during 2000, but there was no salinity sampling available for any of those cruises. Only one other processed cruise could be found during which this instrument was used; that was 9818. There was some salinity calibration sampling and the salinity was found to be within ±0.005psu for most samples. Generally Guildline CTDs are quite stable, so we might expect that the salinity is within ±0.01psu for this cruise.

12.	REMOVE CHANNELS

The following channel was removed from all casts: Conductivity_Ratio.

13.	PRODUCE FINAL FILES

a.) HEADEDIT was used to fix channel names and formats and mission i.d. and to add the following comments:

There was no salinity calibration sampling.

The last calibration on file was from 1998. This is probably the

right one for this data set, but it is not certain.

Calibration sampling in 1998 showed that most samples were within +/-

0.005psu, so a reasonable estimate for this cruise is +/-0.01psu.

b.) The final files were renamed *.ctd. The Standard Check routine and HEADEDIT were rerun until all problems were resolved.

Note that the salinity format (F8.3) is non-standard to indicate the limitations in the quality of that data.

c.) A cross-reference listing was produced.

PARTICULARS

2 – bad conductivity near the top, suspicious salinity at depth.

37-52, 64, 66, 72, 74, 76, 83 – Bad conductivity data in large part of profile; pressure and temperature may also have problems.

66 – Bad in top 7db and T and S suspicious in top 20db.

78 – Conductivity bad in top 6db and all data outside historic ranges from 20 to 60db.

81 - Conductivity data bad in some parts and suspicious in others.

Institute of Ocean Sciences

CRUISE SUMMARY

Cruise ID#:	2000-38

Dates: 	Start:	7 July 2000	    End:	24 July 2000

Location:	Strait of Georgia / Juan de Fuca Strait

Vessel:		W. E. Ricker

Scientific Party Chief:    Beamish R.

| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |
| --- | --- | --- | --- | --- | --- |
| 1 | Guildline | 8715 | 53501 | No | yes |

Institute of Ocean Sciences

CTD Calibration Information

Make/Model/Serial#:	Guildline /8715/53501

Cruise ID#:	2000-38

| Calibration Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sensor |  | Pre-Cruise |  | Post Cruise |  |
| Name | S/N | Date | Location | Date | Location |
| Pressure |  |  |  |  |  |
| Temperature |  | 7 Aug 98 | I.O.S. |  |  |
| Conductivity |  | 7 Aug 98 | I.O.S. |  |  |

| Calibration Coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel | Formula | Coefficients |  |  |  |  |  |
| Name | # | C1 | C2 | C3 | C4 | C5 | C6 |
| Temperature | 10 | .0027318 | .999875 |  |  |  |  |
| Conductivity | 10 | .0000503156 | .999729 |  |  |  |  |