## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 24J June 2021 | Replaced DIC and TA; error found in original analysis. G.G.. |
| 16-Aug-2017 | Changed suffix of 2001-29-surface0001.HED underway file to PCO2. Suspect suffix was HED because Header Edit was the last program used on this file and listed in the History section. |
| 13-Jan-2014 | Added underway pCO2 data from Sophia Johannessen’s Excel files prepared for The Carbon Dioxide Information Analysis Center (CDIAC). The file is located in the cruise .DOC directory. |
| 13-Mar-2013 | Added Iron profile files with cast numbers 8xxx from Keith Johnson’s spreadsheet file which can be found in the cruise .DOC directory. |
| December 7, 2004 | Cast 0008.che – bottle salinity at sample #20 was supposed to be sample #29. Sample #29 was replaced with a pad value. J.L. |
| April 16, 2003 | Moved files to the new archive. |
| March 19, 2002 | SBE oxygen data was calibrated in CTD,CHE, RAC files using 2129oxy.ccf. Formats were changed and channels added as needed. See oxygen_calib.doc. A note was put in headers that oxygen is to be considered ±5%. |

PROCESSING NOTES

Cruise: 2001-29

Agency: OSAP

Location: NE Pacific

Project: Line P

Party Chief: Ron Perkin

Platform: CCGS John P. Tully

Date: August 21, 2001 – September 5, 2001

Processed by: Germaine Gatien

Date of Processing: September 17, 2001 – September 24, 2001

Number of original CTD casts: 26

Number of casts processed: 26

INSTRUMENT SUMMARY

A SeaBird Model SBE 911+ CTD (#0550) was mounted with SeaTech Transmissometer 192D, Seapoint Fluorometer #2229, Oxygen SBE43 sensor #0047 and PAR sensor #4615. The deck unit was a Seabird 11+ (serial number 0424).

SUMMARY OF QUALITY AND CONCERNS

The temperature sensor differences increased with depth and the conductivity sensor differences showed a temporal drift that is probably caused by the secondary sensor.

While the descent rate was very noisy, it was generally quite high minimizing shed wakes.

PAR and Fluorescence are unedited except that records obviously corrupted by shed wakes have been removed. There are spikes in the FL channel at depth that are not present in the other channels.

PROCESSING SUMMARY

1. Seasave - This step was completed at sea; the raw data files have extension DAT.

2. Preliminary Steps

The Log Book was obtained. Salinity calibration data was obtained.

The cruise summary sheet was completed.  The configuration files were obtained and the calibration constants were checked. The sensor history was found.

- Conversion of Raw Data

The raw data was converted using configuration file 0550ctd.con. This file differs from the other configuration files only in details that do not concern the conversion. The data from file 21290015.DAT could not be converted; fixdat.exe did not help. A file 21290015.BAK was renamed *.DAT and converted and produced data that looks reasonable.

A preliminary check shows all expected channels present. The PAR sensor was used only for casts: #1,3, 4,5,6,13,22 and 31.

Dark values were checked for the fluorometer and are generally about +0.11g/l below 500db. These values are consistent with the use of the 10X cable which is believed to be the one used for this cruise.

The temperature and salinity traces track reasonably well and up and downcasts are similar. The differences in salinity appears to be about 0.008units.

4. STRIP

The salinity channel was stripped from the CNV files so that DERIVE doesn’t create a 2nd set of salinity channels.

5. ALIGNCTD

The deck unit did not advance the secondary conductivity channel.

Tests on cast #6 and 28 show that an alignment of the primary conductivity channels by -0.033s (net alignment +0.040s) and secondary conductivity channel by +0.055s minimizes salinity spikes.  All casts were realigned using those values.

A study was made of the alignment of the SBEOXY channel. . The manual suggests advancing the oxygen channel for a pumped instrument by +2s to +5s. This equipment has been used on two previous cruises 2001-08 and 2001-20 when advancement by +8s and +10s respectively were found optimal. It is possible that differences in plumbing account for the variations in advancement. It is known that the external instruments were attached in a different manner during those two cruises.

Casts #6 and 28 were studied in detail by plotting the up and downcasts together displaying just oxygen and primary temperature. Getting the two traces to overlie each other is the method suggested in the SeaBird manual. In shallow water this might be reasonable although local effects might cause real changes and in deep water the effect of internal waves would cause the traces to not overlie each other. So deep casts were chosen and the offset of downcast and upcast temperature was compared and taken to be the standard so that the internal wave problem is allowed for. Casts with little rosette sampling were selected to simplify the comparison. An advancement of 5s led to optimal results and was applied to all casts.

6. WILDEDIT

Program WILDEDIT was used to remove spikes in all channels except scan number.  Parameters used were:

Pass 1    Std Dev = 2		Pass 2    Std Dev = 5		Points per block = 50

7. CELLTM

Tests were run on cast #6 to determine the optimal parameter choice. Combinations tested were (α,1/β) = (2,7),(2,9),(2.5,9) (3,7),(3,8),(3,9),(3,11). The choice of (3,9) produced the best results. The conductivity cell thermal mass correction was done for both channels (α = 0.03; 1/beta = 9.0.)

8. DERIVE

Program DERIVE was run twice:

- on all casts to calculate primary and secondary salinity.

- on all casts to calculate the differences between primary and secondary channels for temperature, conductivity and salinity and to calculate the descent rate. These were placed in a test directory and will not be archived.

9. Test Plots and Channel Check

A sample of casts was plotted to check for agreement between the pairs of T and C sensors. All expected channels were present.

The descent rate was often very noisy but the average rate was kept reasonably high for most casts.

The differences in temperature, conductivity and salinity were extremely noisy and the values that follow are very rough averages:

| Cast # | Max. Press | T1-T0 | C1-C0 | S1-S0 | Descent Rate |
| --- | --- | --- | --- | --- | --- |
| 8 | 1300 | +0.0002 | +0.0002 | +0.002 | Very noisy |
| 23 | 3150 | +0.001 | +0.0005 | +0.0045 | Extremely noisy |
| 38 | 3600 | +0.0012 | +0.00055 | +0.0055 | High, noisy |
| 41 | 3970 | +0.0014 | +0.00065 | +0.0065 | High, noisy |

Because of the suggestion of temporal variation more casts were checked at 1000db, 2000db, 3000db and 4000db. At all depths there was a gradual increase in the salinity difference with time and a decrease with depth. For example at 1000db the difference was about +0.002 for cast #8 and +0.0075 for cast #41 whereas at 3000db the difference was +0.0045 for cast #23 and +0.007 for cast #41.

Temperature differences increase with depth, but there is no time-dependence whereas conductivity variations are time-dependent but do not vary much with depth.

10.  Conversion to IOS Headers

The IOSSHELL routine was used to convert SEA-Bird 911+ data to IOS Headers.

The ROS files were converted to IOS files, and the extensions were changed to BOT.

11. Checking Headers

A header summary and header check were produced and no errors were found.

The surface check was run. The average surface pressure is 1.9db.

The cruise track was plotted and looks reasonable.

- DELETE

CLEAN was run to replace pad values in pressure with interpolated values.

The following DELETE parameters were used:

Surface Record Removal: Last Press Min

Maximum Surface Pressure (relative): 10.00   Pressure Tolerance: 1.0

Pressure filtered over 15 points

Swells deleted. Warning message if pressure difference of 2.00

Drop rates < 0.3m/s (calculated over 15 points) was deleted.  Sample interval =  .04 seconds.

COMMENTS ON WARNINGS: The only warnings pertained to upcast data.

Maximum pressure check: No problems noted.

All DEL files were copied to EDT files.

13. COMPARE

The BOT files were examined for errors and none were found.

COMPARE was run using SAL and BOT files. The details of the comparison were checked for indications of errors in the hydro files. The scatter is larger than usual, so it is not clear what to consider outliers.

The primary salinity was found to be high by 0.006units and the secondary high by 0.011units.  The variations with file pair number suggest that the difference, S0-S(bottle), is fairly flat, while S1-S(bottle) may be increasing with time. Once again the scatter makes this a rough judgment.

Following up on the observations of section 9 COMPARE was used to analyze the differences between the pairs of temperature and conductivity sensors, and the resulting salinities.  The bottle files (averaged on bottle number) were used for the comparison:

•the conductivity difference was found to be independent of pressure, but increasing with time.

•the temperature difference shows a marked increase with pressure; this is not something that is usually analyzed, but is unexpected. The scatter was very large in the analysis of (T1-T0) vs. time, but the trend line is flat.

•the salinity difference clearly increases with time changing by about 0.01units. The few deep casts available show a pressure-dependence below 2000db.

It appears that the secondary salinity is drifting significantly with time due to a conductivity drift. The depth dependence in the salinity may be caused by a drift in one of the temperature sensors, but there are not enough bottles to determine which sensor is at fault.

- DETAILED EDITING

All casts were examined for differences between the pairs of sensors and differences between up and down casts for evidence of problems with the processing. None were found.

The primary sensors were chosen for further processing since they are closest to the bottles and there is less suggestion of depth or time-dependence than for the secondary.

Page plots were produced using T0,S0 for all casts. These plots were examined for spikes and instabilities and used to guide the use of CTDEDIT. All casts were edited using CTDEDIT; this editing was very light for casts #1,40 and 42. Casts #22 and 23 were edited very heavily as they were corrupted by many shed wakes. Note was made of the editing details in the relevant files. Edited files were copied to *.EDT

15. SPECIAL FLUOROMETER PROCESSING (FILTER AND BIN AVERAGE)

A median filter with fixed size 11 was applied to the SeaPoint fluorometer channel to reduce spikiness.  BIN AVERAGE with an average interval of 0.25m was applied to the output of the filter. These files were then stored in a separate directory for Angelica Peña.

16. BIN AVERAGE

The following Bin Average values were used for EDT files:

Bin channel = pressure

Averaging interval = 1.000

Minimum bin value =   .000

Average value will be used

Interpolated values are NOT used for empty bins

The same values were used for the BOT files except that the Bin Channel = Bottle Number.

After averaging, page plots were examined on screen to check whether further editing was needed.

17. Other comparisons

Previous experience with these sensors –

All sensors have been recalibrated since their last use. It is noted that the secondary conductivity sensor has shown calibration drift with time over the past two years.

Historic ranges – Deep casts were plotted with the historic ranges superimposed. All data fell within those ranges except for cast #16 for which the temperatures at about 1500db are slightly above the range maxima. (See plots in Processing Section of Report.)

18. Recalibration

File 2129rcal.ccf was used to recalibrate the AVG files (including those in the IOS directory, the HYDRO directory and the FLUOR directory) as follows:

- primary salinity –  offset: -0.006units

- secondary salinity –  offset: -0.011units

- Chl_fluorescence -  offset: -0.11g/l

The SAM files were recalibrated and named COR; these files were used to rerun COMPARE. The results (2129com2.xls) indicate that the deep primary salinity values after recalibration were within 0.003units of the bottles. (See plots in Processing Section of Report.)

19. REMOVE and REORDER

The following channels were removed from all casts: Scan_Number, Secondary Temperature, Secondary Salinity, Conductivity:Primary, Conductivity:Secondary and Flag.

The PAR channel was removed for casts #: 2,8,16-21,23-30,38-42.

The channels were reordered and data format corrected as needed.

The channels for the rosette files were also reordered but only Conductivity:Primary, Conductivity:Secondary and Flag channels were removed from all casts.

20. Final Plots

THIN and DERIVE were run to obtain values for tables and page plots were prepared using the edited data

21. Producing final files

a.) The final files were renamed CTD.

b.) A cross-reference listing was produced.

### Particulars

15. Cast converted using 21290015.BAK. Very noisy descent rate. Many records removed with editor.

18. Two stops during downcast

19. CTD touched bottom

22. Very noisy descent rate. Many records removed in editing.

23. Extremely noisy descent rate. Many records removed in editing.

# Institute of Ocean Sciences

CRUISE SUMMARY

Cruise ID#: 2001- 29

Dates: Start: 21 August 2001 End: 5 September 2001

Location: NE Pacific

Vessel: CCGS John P. Tully

Party Chief: Ron Perkin

| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |
| --- | --- | --- | --- | --- | --- |
| 1 | SEABIRD | 911+ | 0550 | Yes | Yes |

# Institute of Ocean Sciences

# CTD Calibration Information

Make/Model/Serial#:	SEABIRD/911+/	0550

Cruise ID#:		2001-29

| Calibration Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sensor |  | Pre-Cruise |  | Post Cruise |  |
| Name | S/N | Date | Location | Date | Location |
| Temperature | 2668 | 18/07/01 | Factory |  |  |
| Conductivity | 2102 | 17/07/01 | “ |  |  |
| Secondary Temp. | 2374 | 18/07/01 | “ |  |  |
| Secondary Cond. | 2128 | 17/07/01 | “ |  |  |
| Fluorometer -pumped | 2229 | 01/07/01 | IOS |  |  |
| SBE43 Oxygen sensor | 0047 | 16/05/01 | ? |  |  |
| Transmissometer | 192DR | 26/05/01 | IOS |  |  |
| PAR | 4615 | 15/12/00 |  |  |  |
| Pressure Sensor | 75636 | 04/06/99 | Factory |  |  |

Sensor Calibration Notes:

The configuration file used is attached; this includes the sensor calibrations.

# Moving files to new archive. April 16, 2003 – Joe Linguanti

- Program HEADER EDIT was used to change non-standard channel names and units and add additional metadata to the CTD and Rosette files.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files .CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.* “.

- The loop file given to me by Bernard was replaced with one given to me by Frank. I added two new columns to Frank’s spreadsheet, Sample_Method and Pressure. The latitude and longitude values were computed to 3 decimal places. The spreadsheet file was then converted to IOS Header format with meta-data added to it. The original .XLS file is in the DOC directory. Bernard’s original loop file is in the HISTORY directory.