File List for SeaBird Data

Cruise Number: _2002-36____

This document describes the contents of folder created by or added to by G. Gatien. There are many other folders for this cruise.

[osapcommon.ctd.2002-36.doc]

2002-36_proc.doc - processing notes

2002-36_file.doc - this document

2236rcal1.ccf – calibration control file for 1st oxygen calibration

2236rcal2.ccf – calibration control file for 2nd oxygen calibration

2236header.txt – header file used to add comments to the headers of the CTD files

[osapcommon.ctd.2002-36.ctd data]

*.bl - original files from SeaSave

*.con - 		“

*.dat -      	“

*.hdr -		“

[osapcommon.ctd.2002-36.convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[osapcommon.ctd.2002-36.derive]

*.cnv - data after running ALIGNCTD, WILDEDIT, CELLTM and DERIVE (for S0, S1)

[osapcommon.ctd.2002-36.ios]

*.avg - data after metre-averaging using BINAVE

*.cln – *.ios files with event numbers added

*.ctd - final files

*.del – after DELETE, downcast data for 1-111, upcast for 81(called 9981)

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT.

Delete.log - log from routine DELETE

[osapcommon.ctd.2002-36.hydro]

*.bot  - files created by converting *.ros files to IOS HEADERS

*.oxy – oxygen calibration data provided by Bernard Minkley

*.rac – rosette files averaged by bottle number

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity calibration files provided by Bernard Minkley

*.sam – bot files with sample numbers added; input to COMPARE

2236comp.csv & .xls  - files for comparison of CTD salinity and bottles

36-41combo-DOX.xls – spreadsheet combining oxygen comparison for 2002-36 and 2002-41

2236oxy.csv & .xls – files for 1st comparison of SBE-DOX data and titrated oxygen data

2236oxy2.csv & .xls – files for 2nd comparison of SBE-DOX data and titrated oxygen data

2236oxy3.csv & .xls – files for 3rd comparison of SBE-DOX data and titrated oxygen data

2236chl.csv – comparison of chlorophyll and fluorescence

2002-36chldat.csv – chlorophyll data from Melanie Quenneville