File List for SeaBird Data

Cruise Number: _2002-41____

[osapcommon.ctd.2002-41.doc]

2002-41_proc.doc - processing notes

2002-41_file.doc - this document

2241rcal1.ccf – calibration control file for 1st oxygen calibration

2241rcal2.ccf – calibration control file for 2nd oxygen calibration and primary salinity calibration

2241header.txt – header file used to add comments to the headers of the CTD files

[osapcommon.ctd.2002-41.ctd data]

*.bl - original files from SeaSave

*.con - 		“

*.dat -      	“

*.hdr -		“

[osapcommon.ctd.2002-41.convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[osapcommon.ctd.2002-41.derive]

*.cnv - data after running ALIGNCTD, WILDEDIT, CELLTM and DERIVE (for S0, S1)

[osapcommon.ctd.2002-41.ios]

*.avg - data after metre-averaging using BINAVE

*.cln – *.ios files with event numbers added

*.ctd - final files

*.del – after DELETE, downcast data for 1-111, upcast for 81(called 9981)

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT.

Delete.log - log from routine DELETE

[osapcommon.ctd.2002-41.hydro]

*.bot  - files created by converting *.ros files to IOS HEADERS

*.oxy – oxygen calibration data provided by Bernard Minkley

*.rac – rosette files averaged by bottle number

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity calibration files provided by Bernard Minkley

*.sam – *.bot files with sample numbers added

2241comp.csv & .xls  - files for comparison of CTD salinity and bottles

36-41combo-DOX.xls – spreadsheet combining oxygen comparison for 2002-41 and 2002-41

2241oxy.csv & .xls – files for 1st comparison of SBE-DOX data and titrated oxygen data

2241oxy2.csv & .xls – files for 2nd comparison of SBE-DOX data and titrated oxygen data

2241oxy3.csv & .xls – files for 3rd comparison of SBE-DOX data and titrated oxygen data