Processing Notes for the Year 2002 Nanoose Bay Data	29 July, 2026

Ron Perkin

.hex and .con files were inspected then used to create .cnv files using the Sea Bird software and the recommended processing for the SBE19 using the following batch file. The new Seabird software would not process this data; I had to use the "oldseasoft" directory.

cd d:\nanoose\oldseasoft

echo use Ctrl-Break to interrupt processing

datcnv -i%1 -oraw -c%1

if errorlevel 1 goto abort

filter -iraw -o%1

if errorlevel 1 goto abort

alignctd -i%1 -o%1

if errorlevel 1 goto abort

loopedit -i%1 -o%1

if errorlevel 1 goto abort

split -i%1 -oddown -ouup

if errorlevel 1 goto abort

binavg -i%1 -o%1 -d

if errorlevel 1 goto abort

derive -i%1 -o%1 -c%1

if errorlevel 1 goto abort

strip -i%1 -o%1

if errorlevel 1 goto abort

goto end

:abort

echo batch process interrupted by user

:end

.cnv files were converted to .ios files.

The old Visual Basic program was converted to .NET format and positions were added from the .act files. Coordinates were taken from the .act files as distance in metres, x' and y', along range 2 which was inclined at an angle of 11.2º south of east. Conversion into easting and northing coordinates was done according to (output was .ctd files):

X = x'cos(11.2) + y'sin(11.2)

Y = -x'sin(11.2) + y'cos(11.2)

Conversion to lat/long was done according to:

Lat = 49º (21.05' +Y*60/111120)

Long = 124º + (9.55 - X*60/72387)

The data were inspected in CTDedit and small corrections were made to:

N2012301.ctd bad cast; discard

N2102301.ctd, a few glitches caused by rapid temperature change.

N2091001.ctd, 102 m deleted at the bottom.

N2090401.ctd, a few glitches caused by rapid temperature change.

N2082901.ctd, a few glitches caused by rapid temperature change.

N2021301.ctd, a few glitches caused by rapid temperature change.

The data was exported to a .o4x file(station name was added) and was inspected in Ocean Data View. Outliers were removed and the time sequence was contoured.

Fixed  .ctd files. Posted with the IOS sequential numbering.

Made .o4x with station numbers and imported into ODV.

Figure 1