Cruise 2003-11:  May/June Line P 2003

CTD/Rosette:

Casts 1 to 9 done with CTD 0585 and deck unit 0508. Mid-ship winch.

Casts 1, 2, 4, 5, 6  were done with PAR sensor ON. The PAR sensor was not used after this.

Cast 4 has the wrong cruise number (2003-12) in header.

Casts 10 and 11 done with CTD 0550 and deck unit 0508.

Change to stbd side winch.

Both casts have lots of spikes.

Cast 11: no bottle file

Casts 12 to the end: done with CTD 0550 and deck unit 0619.

Cast 12 crashed at the bottom. Cast 13 is the upcast for that station. Lots of spikes in cast 12. Casts 13 and on don’t seem to have spikes.

Cast 24 crashed at the bottom. Have to re-boot the computer. Cast 24 has lots of spikes. Cast 25 is the up-cast for that station.

Rosette cast 33: problem with the bottles/bottle file. Will need salinity samples to determine where the bottles were tripped. Bottle file has only 23 entries, but all 24 bottles were closed.

From the salinity samples, it looks like a bottle was closed at 1000m, but it did not register in the bottle file.

Cast 39: wrong station name in file. Is P20, should be P23.

Cast 42: error light turned on during cast, has spikes.

Casts 47 and 48 have spikes.

Before cast 49: stbd-winch cable is re-terminated, 850 m of cable removed.

Cast 53 crashed at the bottom. Cast 54 is the upcast. The information in the header is in the wrong place (longitude for latitude, etc.)

Casts 53 to 55 have spikes.

Just before cast 59: go back to mid-ship winch. (re-terminated)

Casts 95 and 96 have to bottles closed. (we did not want any samples).

Bongos:

We realised at P16 that there is a problem with the quick freezer. P16 frozen in the big freezer, and it is the first “for sure” frozen sample.

Flowmeter side at Argo 2 remained unfrozen for 2 days before being put in freezer.

DNA bongos done at P1 and P26.

Argo floats:

Float 948:	Wake up time (UTC)	1612	29 May 2003

Deployment			1632	29 May 2003

48°44.67	127°39.19

Corresponding CTD cast: 	2003-11-0010	P6

Float 949:	Argos ID number: 1383A8B

Wake up time (UTC)	0334	2 June 2003

Deployment			0405	2 June 2003

49°00.37	139°29.45

Corresponding CTD cast:	2003-11-0036	Argo 1

Float 954: 	Argos ID number: 1383AD4

Wake up time (UTC)	0455	8 June 2003

Deployment			0921	8 June 2003

49°59.56	146°59.55

Corresponding CTD cast:	2003-11-0061	Argo 2

Float 955: 	Argos ID number: 1383AE1

Wake up time (UTC)	0346	9 June 2003

Deployment			0428	9 June 2003

52°00.20	147°00.14

Corresponding CTD cast:	2003-11-0064	Argo 3

Float 959: 	Argos ID number: 1386926

Wake up time (UTC)	1247	10 June 2003

Deployment			1339	10 June 2003

53°30.19	140°59.49

Corresponding CTD cast:	2003-11-0068	Argo 4

Float 958: 	Argos ID number: 1386913

Wake up time (UTC)	0351	11 June 2003

Deployment			0506	11 June 2003

53°30.19	138°00.15

Corresponding CTD cast:	2003-11-0071	Argo 5

Float 950: 	Argos ID number: 1383A98

Wake up time (UTC)	1714	11 June 2003

Deployment			1936	11 June 2003

53°30.31	134°59.81

Corresponding CTD cast:	2003-11-0074	Argo 6