File List for SeaBird Data        Cruise Number: _2003-12____

[work2.ctd.2003-12] *.bl - original files from SeaSave

*.con - 		“

*.dat -     	“

*.hdr –		“

[work2.ctd.2003-12.convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[work2.ctd.2003-12.derive]

*.cnv - data after running ALIGNCTD, WILDEDIT, CELLTM and DERIVE (for S0, S1)

[work2.ctd.2003-12.doc]

2003-12-rcal.ccf – calibration control file with offset for primary salinity

2003-12_proc.doc - processing notes

2003-12_file.doc - this document

[work2.ctd.2003-12.hydro]

*.bot – rosette files after conversion to IOS Headers and event #s added

*.che – rac files merged with chemistry data

*.chl – extracted chlorophyll data with quality flag channel

*.oxy – oxygen bottle data

*.oxyq – oxygen files wit quality flag channel added

*.rac – averaged, recalibrated bottle files with sample numbers

*.ros – original converted bottle files in SeaBird format

*.sal  - salinity bottle data

*.salq – sal files with quality flag channel added

*.sam – bot files with sample number added

*.sac – sam files after recalibration

addsamp.csv – spreadsheet with cast #, bottle #, sample #

2003-12-chlcomp.csv – input to COMPARE for CHL and fluorescence

2003-12-chlcomp.xls – comparison of CHL and fluorescence

2003-12chlarc.xls – titrated chlorophyll data provided by M. Quenneville

2003-12chl.csv – titrated chlorophyll data spreadsheet in format used to create individual files

2003-12-salcomp.csv & 2003-12-salcomp1.csv – input for salinity comparison

2003-12-salcomp.xls & 2003-12-salcomp1.xls – salinity comparison before & after recalibration

2003-12-LOOPSAL.xls – loop salinity data

[work2.ctd.2003-12.ios]

*.avg - data after metre-averaging using BINAVE

*.ctd - final files

*.del – after DELETE, downcast data for 1-111, upcast for 81(called 9981)

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios - data after conversion to IOS HEADERS

*.log  - log files created by CTDEDIT

*.rev – reverse of 2 files after SHIFT and before DELETE, casts 62, 72 – upcast used for archive

Delete.log - log from routine DELETE

Folders [2003-12-VEMCO],[aanderaa], [oxygen], [thermo], [thermosalinograph] – created by others, details unknown