Cruise Plan & Itinerary:

Cruise #IOS-2003-24 CCGS Vector   June 16 - June 21, 2003

Area of Operation: Juan de Fuca Strait and Strait of Georgia

Summary of Objectives: CTD, bottle sampling, sediment coring

Specific objectives:

- CTD/rosette along standard Juan de Fuca and Strait of Georgia lines (76 stations). The water from the rosette will be used for dissolved oxygen, nutrients and total and dissolved organic carbon measurements.

- Collect sediment box cores at stations GVRD-1, GVRD-2 and GVRD-6

Chief Scientist: 	Sophia Johannessen - phone (250) 363-6616

Additional personnel:

|  | Bernard Minkley (IOS) David Paton (IOS) Sheila Toews (IOS) Lizette Beauchemin (contractor) Cynthia Wright (contractor) Sébastien Mancini (student - ISITV) Maxime Lanoelle (student - ISITV) |
| --- | --- |

Ship’s equipment required:

GPS positioning

Depth sounder

Loran C

Radar

Deck machinery required:

CTD/Rosette winch

Sediment core winch

Other equipment to be loaded:

Laboratory instruments and computers

CTD/rosette

Freezers

Sediment corer, wide block

Provisional schedule:

Monday, June 16: Load the equipment at IOS.  When loading is completed and equipment tested, sail to the mouth of  Juan de Fuca Strait and start CTD stations. A first test station will be done in Saanich Inlet near the Met buoy.

Monday - Friday (June 16-20): proceed with the list of stations, including the three sediment core stations.

Friday, June 20: Complete the stations in the northern Strait of Georgia. As soon as completed, sail back to Pat Bay and unload material.

Station list:

CTD/R Stations:

| Sta | Lat | Lat_mn | Lon | Lon_mn | Depth | ### Activity |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 49 | 20.30 | 124 | 12.10 | 247 | CTD |
| 2 | 49 | 24.10 | 124 | 9.30 | 289 | CTD Ox |
| 3 | 49 | 26.60 | 124 | 20.20 | 327 | CTD Ox |
| 4 | 49 | 24.50 | 124 | 22.10 | 344 | CTD |
| 5 | 49 | 27.40 | 124 | 30.80 | 315 | CTD |
| 6 | 49 | 30.60 | 124 | 27.80 | 192 | CTD Ox |
| 7 | 49 | 34.40 | 124 | 24.20 | 357 | CTD |
| 8 | 49 | 39.30 | 124 | 33.00 | 355 | CTD |
| 9 | 49 | 35.50 | 124 | 38.30 | 170 | CTD Ox |
| 10 | 49 | 40.70 | 124 | 47.30 | 95 | CTD |
| 11 | 49 | 42.40 | 124 | 43.40 | 289 | CTD |
| 12 | 49 | 43.60 | 124 | 40.80 | 357 | CTD Ox |
| 13 | 49 | 55.30 | 124 | 55.20 | 219 | CTD |
| 14 | 49 | 53.00 | 124 | 59.60 | 311 | CTD Ox |
| 15 | 49 | 51.60 | 125 | 1.60 | 143 | CTD |
| 16 | 49 | 57.70 | 125 | 8.80 | 146 | CTD Ox |
| 17 | 49 | 58.80 | 125 | 4.30 | 265 | CTD |
| 18 | 49 | 59.20 | 124 | 58.90 | 146 | CTD |
| 19 | 50 | 1.20 | 124 | 52.90 | 283 | CTD |
| 20 | 49 | 47.20 | 124 | 32.30 | 311 | CTD |
| 23 | 50 | 15.0 | 125 | 23.0 | 240 | CTD Ox |
| 22 | 49 | 40.20 | 124 | 16.30 | 358 | CTD |
| 21 | 50 | 2.0 | 125 | 13.6 | 72 | CTD |
| 24 | 49 | 30.30 | 124 | 6.00 | 430 | CTD |
| 25 | 49 | 27.70 | 124 | 7.50 | 320 | CTD |
| 26 | 49 | 14.30 | 123 | 50.80 | 256 | CTD |
| 27 | 49 | 19.10 | 123 | 48.00 | 347 | CTD Ox |
| 28 | 49 | 24.10 | 123 | 45.30 | 134 | CTD |
| 37 | 49 | 13.60 | 123 | 20.70 | 219 | CTD |
| 38 | 49 | 12.00 | 123 | 26.40 | 296 | CTD |
| 39 | 49 | 9.80 | 123 | 33.00 | 388 | CTD Ox |
| 40 | 49 | 8.60 | 123 | 36.80 | 146 | CTD |
| 41 | 49 | 3.30 | 123 | 22.30 | 247 | CTD |
| 42 | 49 | 1.80 | 123 | 26.20 | 326 | CTD Ox |
| 43 | 49 | 0.20 | 123 | 30.10 | 187 | CTD |
| 44 | 48 | 56.80 | 123 | 6.00 | 119 | CTD |
| 45 | 48 | 53.90 | 123 | 8.40 | 128 | CTD |
| 46 | 48 | 51.40 | 123 | 10.80 | 179 | CTD Ox |
| 47 | 48 | 49.30 | 123 | 6.20 | 188 | CTD |
| 48 | 48 | 51.70 | 123 | 3.20 | 220 | CTD |
| 49 | 48 | 55.00 | 122 | 59.60 | 132 | CTD |
| 50 | 48 | 57.70 | 122 | 56.10 | 26 | CTD |
| 51 | 48 | 46.70 | 122 | 51.60 | 155 | CTD |
| 52 | 48 | 38.90 | 122 | 43.30 | 91 | CTD |
| 53 | 48 | 33.20 | 122 | 45.00 | 73 | CTD |
| 54 | 48 | 26.60 | 122 | 44.30 | 77 | CTD |
| 56 | 48 | 46.40 | 123 | 1.60 | 214 | CTD Ox |
| 57 | 48 | 44.00 | 123 | 8.10 | 141 | CTD |
| 58 | 48 | 43.00 | 123 | 14.30 | 338 | CTD |
| 59 | 48 | 37.80 | 123 | 14.60 | 227 | CTD Ox |
| 60 | 48 | 33.90 | 123 | 12.80 | 298 | CTD |
| 61 | 48 | 29.20 | 123 | 9.20 | 274 | CTD |
| 62 | 48 | 22.80 | 123 | 2.60 | 150 | CTD Ox |
| 63 | 48 | 14.60 | 122 | 58.50 | 154 | CTD |
| 64 | 48 | 12.80 | 123 | 5.80 | 97 | CTD |
| 65 | 48 | 15.90 | 123 | 9.80 | 121 | CTD Ox |
| 66 | 48 | 19.30 | 123 | 14.10 | 104 | CTD |
| 67 | 48 | 22.80 | 123 | 18.10 | 106 | CTD |
| 68 | 48 | 12.20 | 123 | 43.00 | 137 | CTD |
| 69 | 48 | 15.60 | 123 | 43.20 | 174 | CTD Ox |
| 70 | 48 | 19.20 | 123 | 43.20 | 154 | CTD |
| 71 | 48 | 22.20 | 123 | 59.40 | 115 | CTD |
| 72 | 48 | 18.60 | 124 | 4.00 | 192 | CTD Ox |
| 73 | 48 | 15.00 | 124 | 6.70 | 157 | CTD |
| 74 | 48 | 25.10 | 124 | 35.60 | 192 | CTD |
| 75 | 48 | 28.20 | 124 | 32.80 | 230 | CTD Ox |
| 76 | 48 | 31.40 | 124 | 30.00 | 93 | CTD |
| 101 | 48 | 25.40 | 124 | 45.14 | 143 | CTD |
| 102 | 48 | 30.00 | 124 | 44.00 | 260 | CTD Ox |
| 103 | 48 | 33.00 | 124 | 43.00 | 125 | CTD |
| 104 | 48 | 20.00 | 123 | 18.00 | 70 | CTD |
| 105 | 48 | 11.00 | 123 | 19.00 | 80 | CTD |
| ADC | 48 | 14.00 | 123 | 18.00 | 113 | CTD Ox |
| GVRD-1 | 49 | 35.50 | 124 | 38.30 | 170 | Core (stn 9) |
| GVRD-2 | 49 | 19.87 | 123 | 18.50 |  | Core |
| GVRD-6 | 48 | 56.20 | 123 | 18.80 |  | Core |

GVRD-1

GVRD-2

GVRD-6

Sediment Core