CTD data from Saanich Inlet, August, 2003

Instrument was Plankton Dynamics’ SBE 19 Seacat serial number 1294 with pump #050424.  In addition to conductivity, temperature, and pressure, a fluorometer (Wetstar #  WS3s-715P) and PAR (Biospherical  QSP-L #4615 initially, then #4564) were mounted with a specially made Y-connector.

Some difficulty was encountered in getting the system to work as configured initially. It would appear that there are data for those casts, but they need work. The cast files are interspersed between test files.  After Aug 12 the data are more readable.

CTD casts were done on July 17 (a test cast), then on August 6, 7, 8, 9, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 25, and 26. On Aug 9 and 11 two casts were done each day, one with the PAR unplugged, the other with the fluorometer unplugged. CTD casts were generally done early in the morning (i.e., around 0730). The instrument was started, held at the surface for 2 minutes, then hand-lowered to 20m, held at 20m for a few seconds, then brought back to the surface. The data were dumped periodically, but not discarded from the CTD until August 18. Then different people downloaded the data, thus the various naming conventions.

List of files and casts:

| Cast number/filename | Date | Time | Data |
| --- | --- | --- | --- |
| Saanich1.hex | July 17 | 0840 | Profile- fluorometer no PAR |
| Saanich2.hex | July 17 |  | Profile- fluorometer no PAR |
| 1294-017Saan.cnv | Aug 6 |  | See records 1757-2751 |
| 1294-017Saan.cnv | Aug 7 |  | See records 3389-3675 or 3676 -7384 |
| cast0000.hex | Aug 8 | 0729 | Needs work |
| cast0007.hex | Aug 9 | 0732 | “ |
| Cast0008.hex | Aug 9 | 0739 | “ |
| Cast0009.hex | Aug 11 | 0719 | “ |
| Cast0010.hex | Aug 11 | 0727 | “ |
| Cast0020.hex | Aug 12 | 0728 | Profile- no PAR |
| Cast0022.hex | Aug 13 | 0814 | Profile- PAR + fluor |
| Cast0023.hex | Aug 14 | 0733 |  |
| Cast0024.hex | Aug 14 | 0734 | “ |
| Cast0025.hex | Aug 15 | 0720 | “ |
| Cast0026.hex | Aug 16 | 0728 | “ |
| Cast0027.hex | Aug 17 | 0720 | “ |
| Cast0028.hex | Aug 18 | 0736 | “ |
| 1294-0029.hex | Aug 19 | 1010 | “ |
| 1294-0030.hex | Aug 20 | 1006 | “ |
| 1294-0032.hex | Aug 21 | 0956 | “ |
| 1294-0033.hex | Aug 21 | 1005 | “ |
| 1294-0035.hex | Aug 21 | 1115 | “ |
| SI082700.hex | Aug 22 | 0934 | “ |
| SI082701.hex | Aug 25 | 1027 | “ |
| SI082702.hex | Aug 26 | 0907 | “ |
| SI082703.hex | Aug 26 | 0931 | “ |