File List for SeaBird Data          Cruise Number: _2005-08____

[osapcommon\cruise_data\2005-08\ctd]

*.bl - original files from SeaSave

*.con - 		“

*.dat -      	“

*.hdr -		“

[osapcommon\cruise_data\2005-08\ctd\convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[osapcommon\cruise_data\2005-08\ctd\derive]

*.cnv - data after running ALIGNCTD, WILDEDIT, CELLTM and DERIVE (for S0, S1)

[osapcommon\cruise_data\ctd\2005-08\ctd\doc]

2005-08-ctd.con – configuration file used for all casts

2005-08-recal1.ccf – calibration control file for recalibration of DO and salinity

2005-08-recal2.ccf – calibration control file for 2nd recalibration of DO

2005-08_proc.doc - processing notes

2005-08_file.doc - this document

2005-08-bot-hdr.txt – header file comments used for CHE files

2005-08-hdr.txt – header file comments used for CTD downcast files

[osapcommon\cruise_data\2005-08\ctd\hydro]

*.add – bottle from analyst, converted, edited, with flags

*.bot  - *.ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.mrg – after addition of oxygen and salinity bottle data to BOT files

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted by spreadsheet

*.sam – bottle files with sample numbers added

addsamp.csv – file containing cast #, bottle number and sample number

2005-08-sal-comp1.csv & xls – initial salinity COMPARE file

2005-08-sal-comp2.csv & .xls - salinity COMPARE files after recalibration of S

2005-08-dox-comp1.csv & .xls – initial comparison of titrated DO and CTD DO

2005-08-dox-comp2.csv & .xls – comparison of titrated DO and CTD DO after recal

2005-08-dox-comp3.csv & .xls – comparison of titrated DO and downcast CTD DO

2005-08-dox-comp4.csv & .xls – comparison of titrated DO and CTD DO after final recal

[osapcommon\cruise_data\2005-08\ctd\ios]

*.avg - data after metre-averaging using BINAVE

*.ctd - final files

*.del – after DELETE, downcast data for 1-111, upcast for 81(called 9981)

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT.

Delete.log - log from routine DELETE

[osapcommon\cruise_data\2005-08\Moorings]

[osapcommon\cruise_data\2005-08\Oxyen] Original oxygen bottle files from analyst

[osapcommon\cruise_data\2005-08\Photos]

[osapcommon\cruise_data\2005-08\Salinity] Original oxygen bottle files from analyst

[osapcommon\cruise_data\2005-08\TSG]

*.hex and con – original files from sea

*.cnv – Converted files

[osapcommon\cruise_data\2005-08\TSG\DOC]

2005-08-TSG-recal.ccf – calibration control file for TSG

2005-08-TSG-CTD-comp.xls – comparison of TSG and surface CTD data

2005-08-TSG-CTD-comp-wellmixed.xls – subset of above file - 8 well-mixed casts

2005-08-therm-hdr.txt – file with comment for header of final TSG file

[osapcommon\cruise_data\2005-08\TSG\IOS]

*.atc – after adding header details and time and date channels to *.ios files

*.ed1 and *.log – output of editor used to remove spikes

*.ios – files after conversion from CSV to IOS headers

*.tob – final data file after REMOVE and HEAD EDIT