File List for SeaBird Data          Cruise Number: _2005-09____

[osapcommon\cruise_data\2005-09\Bin1] files created at sea

[osapcommon\cruise_data\2005-09\CTD]

*.bl, con, dat and hdr - original files from SeaSave

[osapcommon\cruise_data\2005-09\CTD convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[osapcommon\cruise_data\2005-09\CTD\derive]

*.cnv - data after running ALIGNCTD, WILDEDIT, and DERIVE (for S0, S1)

[osapcommon\cruise_data\2005-09\CTD\doc]

2005-09-recal1.ccf – calibration control file for recalibration of SAL and DOX

2005-09-recal2.ccf – calibration control file for final recalibration of downcast DOX

2005-09-hdr.txt – header comments for downcast files

2005-09-bot-hdr.txt – header comments for bottle files

2005-09_file.doc - this document

2005-09_proc.doc - processing notes

2005-09-ctd.con – configuration file used in processing

[osapcommon\cruise_data\2005-09\CTD\hydro]

*.add – oxygen calibration data with flags –with a few corrections to data provided by analyst

*.bot  - files created by converting *.ros files to IOS HEADERS

*.che – final bottle files

*.mrg – merge of sal, add and bottle-averaged rosette files

*.oxy – 2 files that were edited from originals, to enable conversion to ADD files

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted by spreadsheet from analyst

*.sam – bottle files with sample numbers added

addsamp.csv – file containing cast #, bottle number and sample number

2005-09 salinity.csv – edited version of spreadsheet from analyst

2005-09-sal-comp1.csv & .xls - salinity COMPARE files

2005-09-dox-comp1.csv & .xls – dissolved oxygen COMPARE files – before recalibration

2005-09-dox-comp2.csv & .xls – dissolved oxygen COMPARE files – after initial recalibration

2005-09-dox-comp3.csv & .xls – dissolved oxygen COMPARE files – downcast vs bottles

[osapcommon\cruise_data\2005-09\CTD\ios]

*.avg - data after metre-averaging using BINAVE

*.ctd - final files

*.del – after DELETE

*.edt - data after editing has been done (an EDT file was prepared even if no editing was required.)

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT.

Delete.log - log from routine DELETE

[osapcommon\cruise_data\2005-09\Oxygen] 	files from analyst

[osapcommon\cruise_data\2005-09\Salinity]	files from analyst

[osapcommon\cruise_data\2005-09\SVP conversion]	files converted at sea