File List for SeaBird Data          Cruise Number: _2005-10____

[osapcommon\cruise_data\2005-10\ctd]

*.bl - original files from SeaSave

*.con - 		“

*.dat -      	“

*.hdr -		“

[osapcommon\cruise_data\2005-10\ctd\convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[osapcommon\cruise_data\2005-10\ctd\derive]

*.cnv - data after running ALIGNCTD, WILDEDIT, CELLTM and DERIVE (for S0, S1)

[osapcommon\cruise_data\ctd\2005-10\ctd\doc]

2005-10-ctd.con – configuration file used for all casts

2005-10-recal1.ccf – calibration control file for recalibration of DO and salinity

2005-10-recal2.ccf – calibration control file for 2nd recalibration of DO

2005-10_proc.doc - processing notes

2005-10_file.doc - this document

2005-10-bot-hdr.txt – header file comments used for CHE files

2005-10-hdr.txt – header file comments used for CTD downcast files

[osapcommon\cruise_data\2005-10\ctd\hydro]

*.add – bottle from analyst, converted, edited, with flags

*.bot  - *.ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.chl – extracted chlorophyll files converted from analyst’s spreadsheet

*.mrg – after addition of oxygen and salinity bottle data to BOT files

*.nuts – nutrient files converted from analyst’s spreadsheet

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted by spreadsheet

*.sam – bottle files with sample numbers added

addsamp.csv – file containing cast #, bottle number and sample number

2005-10-chl-fl.csv & xls – fluorescence/CHL COMPARE file

2005-10-sal-comp1.csv & xls – initial salinity COMPARE file

2005-10-sal-comp2.csv & .xls - salinity COMPARE files after recalibration of S

2005-10-dox-comp1.csv & .xls – initial comparison of titrated DO and CTD DO

2005-10-dox-comp2.csv & .xls – comparison of titrated DO and CTD DO after recal

2005-10-dox-comp3.csv & .xls – comparison of titrated DO and downcast CTD DO

2005-10-dox-comp4.csv & .xls – comparison of titrated DO and CTD DO after final recal

[osapcommon\cruise_data\2005-10\ctd\ios]

*.avg - data after metre-averaging using BINAVE

*.ctd - final files

*.del – after DELETE, downcast data for 1-111, upcast for 81(called 9981)

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT.

Delete.log - log from routine DELETE

[osapcommon\cruise_data\2005-10\DOC] documents – other than those produced in processing

[osapcommon\cruise_data\2005-10\Nutrients] Original nutrient bottle files from analyst

[osapcommon\cruise_data\2005-10\Oxyen] Original oxygen bottle files from analyst