## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |

## PROCESSING NOTES

Cruise: 2005-36

Agency: OSD

Location: Strait of Georgia

Project: Strait of Georgia Pollution Transport

Party Chief: Ross P.

Platform: Vector

Date: September 17, 2005 – September 19, 2005

Processed by: Germaine Gatien

Date of Processing: 19 May 2006 – 24 May 2006

Number of original CTD casts: 4

Number of CTD casts processed: 4

# INSTRUMENT SUMMARY

A SeaBird Model SBE 911+ CTD (#0443) was mounted with a Chelsea/Seatech transmissometer (#498DR), a PAR sensor (#4565), a surface PAR sensor (#16504) and an Altimeter OA-916D (#1024). A SeaBird dissolved oxygen sensor (#0766) was mounted on the primary pump and a Seapoint Fluorometer (#2356) with a 10X cable was mounted on the secondary pump. The deck unit was a model 911 (#0508). The salinometer was an 8400B model Autosal (S/N 68572).

# SUMMARY OF QUALITY AND CONCERNS

These 4 casts were taken right after cruise 2005-25 using the same equipment. There was no calibration sampling and little data so the results of 2005-25 were applied throughout.

# PROCESSING SUMMARY

##### Seasave

This step was completed at sea; the raw data files have extension DAT. The casts were run at the end of cruise 2005-25 and the original files names were 2005-25-0091 through 2005-25-0094. The casts were later assigned their own cruise number, so the file names were changed to 2005-36-0091.* through 2005-36-0094.* (Later in the processing these were changed to 2005-35-0001 to 2005-36-0004.)

##### Preliminary Steps

The Log Book was obtained.

The cruise summary sheet was completed.

The histories of the conductivity, DO and pressure sensors were obtained.

The configuration files are exactly the same as the one used for 2005-25. One was saved as 2005-36-ctd.con.

##### Conversion of Raw Data

The files were converted to CNV files

A few casts were examined and all expected channels were found and looked reasonable.

The descent rate was occasionally low but generally steady.

The two temperature channels sometimes show somewhat different response to sharp features. The down and upcast temperature and conductivity channels are reasonably close, though the two channels vary more on the upcast than the downcast.

Transmissivity looked fine with good agreement between downcast and upcast.

The dissolved oxygen had the usual sort of offset.

The fluorescence, PAR and Surface PAR look ok.

The altimetry is often extremely noisy but generally looks reliable near the bottom.

##### WILDEDIT

Program WILDEDIT was used to remove spikes from the pressure, temperature and conductivity channels only.  Parameters used were:

Pass 1    Std Dev = 2	Pass 2    Std Dev = 5 	Points per block = 50

##### CELLTM

Based on the results of 2005-25, CELLTM was run using (0.02, 7) for both channels.

##### DERIVE

Program DERIVE was run twice:

on all casts to calculate primary and secondary salinity.

on all casts to calculate the differences between primary and secondary channels for temperature, conductivity and salinity. These were placed in a test directory and will not be archived.

##### Test Plots and Channel Check

A few casts were plotted to check for agreement between the pairs of T and C sensors. The differences were extraordinarily noisy, but looked roughly like those of 2005-25.

##### Conversion to IOS Headers

The IOSSHELL routine was used to convert SEA-Bird 911+ CNV files to IOS Headers. The event numbers and file names were changed from 91 through 94 to 1 through 4. The Bottom Depth as recorded in the log book was added to the headers. CLEAN was run to replace pad values in the Pressure channel using linear interpolation based on scan number.

##### Checking Headers

The header check and header summary were run. No errors were found. The log times are in local time (PDT), so differ from the header times by about 7 hours as expected.

The average surface pressure is less <0. During 2005-25 a decision all pressures were increased by 0.4db. Most of negative pressures are associated with data gathered with the pump turned off. Pressures of -0.6 correspond to out-of-water readings, while data from -0.4db appear to be “in water”, so adding 0.4db looks appropriate. A further offset of +.4db will be applied to this data.

All casts were examined to ensure that the algorithm for the altimetry worked well and it did.

##### Other Comparisons

Previous experience with these sensors –

- The primary conductivity sensors were used for 2005-11, -12, -21, -22 and -25. From the latter three cruises there were a lot of bottles and the salinity was found to high by 0.0006 and 0.0005 and 0.0006psu.  The secondary sensors were used for 2005-21, -22 and -25 when they produced salinity high by 0.0027, 0.0021 and 0.0032psu.

- During the most recent uses of this DO sensor the results were:

CTD-BOT = 1.3705 DOX-CTD + 0.049   (2005-10) SoG/JdeF

CTD-BOT = 1.4103 DOX-CTD + 0.0742 (2005-14) SoG/JdeF

CTD-BOT = 1.4255 DOX-CTD + 0.1063 (2005-21) offshore

CTD-BOT = 1.4191 DOX-CTD + 0.0128 (2005-22) offshore

CTD-BOT = 1.3941 DOX-CTD + 0.1253 (2005-14) SoG/JdeF

- The pressure sensor was believed to be reading a little low during 2005-22, but no recalibration was applied. During 2005-25 an offset of +0.4db was applied.

Historic ranges – The temperatures for casts #3 and 4 fell entirely within the local climatology ranges but for casts #1 and 2 they were far outside the historic limits. This is likely because that region is not represented in the climatology. This was confirmed by doing a search for data in the DATA_LIB archive for a small window around these casts, and no other casts were found.

##### Post-cruise calibration – During the processing of 2005-25 a study of the post-cruise calibration for the DO sensor was found to be in reasonable agreement with the calibration found for bottles.

##### 11. SHIFT

Fluorescence

Based on the results of other cruises all data was shifted by +24records. (Output: SHFFL)

Dissolved Oxygen

Based on the results of 2005-25 a shift of +140 records was applied to the DO channel for all casts.

Conductivity

Based on the results of 2005-25 shifts of -0.4 for the primary conductivity and +0.4 for the secondary were applied to all casts. (Output: *.SHFC0 and SHFC1).

One cast was examined to ensure these shifts improved the data and they did.

##### DELETE

The following DELETE parameters were used:

Surface Record Removal: Last Press Min and Low Salinity

Maximum Surface Pressure (relative): 10.00  Minimum Salinity: 5

Surface Pressure Tolerance: 1.0                  Pressure filtered over 15 points

Swells deleted. Warning message if pressure difference of 2.00

Drop rates <   0.30m/s (calculated over 11 points) will be deleted.

Drop rate applies in the range    10db to 10db less than the maximum pressure

Sample interval = 0.042 seconds. (taken from header)

COMMENTS ON WARNINGS: None

##### DETAILED EDITING

The primary sensors were chosen for further processing since those were found best for 2005-25.

Page plots were produced using (T0, S0). These were used to guide the editing. On-screen plots of descent rate and pump status were also used. The descent rate was generally very steady.

Casts 2, 3 and 4 required some light editing.

Note was made of the editing details in the headers of the relevant files.

##### Initial Recalibration and Fluorescence Filter

No recalibration will be applied to the salinity channel.

From 2005-25 we have the following equation for recalibration of DOX:

CTD-BOT = 1.3941 * DOX-CTD + 0.1253

File 2005-25-recal1.ccf was prepared to apply the above equation to the DO channel and to add 0.4db to the pressure channel in the EDT files. (Output:COR)

A median filter, fixed size=11, was applied to the fluorescence channel in the COR files to reduce spikiness. One cast was examined before and after this step and showed that the filter was effective. (Output: FIL)

##### BIN AVERAGE of CTD files

The following Bin Average values were applied to the FIL files (output AVG):

Bin channel = pressure		Averaging interval = 1.000

Minimum bin value =   .000	Average value will be used.

Interpolated values are NOT used for empty bins.

After averaging, page plots were examined on screen and no further editing was deemed necessary.

##### Final DO comparison and calibration

During 2005-25 it was found that a further correction to CTD DO values was needed:

CTD DOX (Corrected) = CTD DOX + 0.0005*Pressure - 0.1395

That correction was applied to the bin-averaged files.

The dissolved oxygen data in the CTD files should be considered:

- ±0.4ml/l in the top 150m

- ±0.1ml/l below 150m.

##### FINAL CTD files steps (REMOVE and HEADEDIT)

For all casts the following channels were removed: Scan_Number, Temperature:Secondary, Salinity:T1:C1, Conductivity:Primary, Conductivity:Secondary, Oxygen:Voltage:SBE, Descent_Rate, Status:Pump, Altimeter and Flag channels. (Output: *.REM)

CHANGE UNITS was used to derive Oxygen:Dissolved:SBE in umol/kg.

REORDER was used to get the two SBE DO channels together.

HEADER EDIT was used to fix formats and channel names and to add the following comments:

There was no calibration sampling for this cruise and only 4 casts.

The results of cruise 2005-25 were used since that cruise had the

same equipment and occurred just before this one.

Transmissivity and fluorescence are nominal and unedited except that

some records were removed in editing temperature and salinity.

The SBE dissolved oxygen data in the CTD files should be considered

•	±0.4ml/l from   0 -  150m

•	±0.1ml/l below 150m

The Standards Check routine was run and HEADEDIT adjusted until no further problems were found. The files were named CTD.

##### Cross-reference lists were produced for the final CTD and CHE file.

The sensor histories were updated.

# Institute of Ocean Sciences

CRUISE SUMMARY

| Cruise ID#: 2005-36 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Dates: Start: 17 Sept. 2005 End: 19 Sept. 2005 |  |  |  |  |  |  |
| Location: SoG/JdeF |  |  |  |  |  |  |
| Vessel: Vector Party Chief: Ross P. |  |  |  |  |  |  |
| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |  |
| 1 | SEABIRD | 911+ | 0443 | Yes | Yes |  |

Institute of Ocean Sciences

# CTD Calibration Information

Make/Model/Serial#:	SEABIRD/911+/0443         Cruise ID#:		2005-36

| Sensor |  | Pre-Cruise |  | Post Cruise |  |
| --- | --- | --- | --- | --- | --- |
| Name | S/N | Date | Location | Date | Location |
| Temperature | 4484 | 19/03/05 | Factory “ |  |  |
| Conductivity | 3038 | 03/05/05 | “ |  |  |
| Secondary Temp. | 2106 | 08/07/05 | “ |  |  |
| Secondary Cond. | 1729 | 12/07/05 | “ |  |  |
| Transmissometer | 498DR | 22/05/05 | Factory |  |  |
| SBE 43 DO sensor | 0766 | 16/11/04 | Factory |  |  |
| Altimeter | 1024 | ? | ? |  |  |
| PAR | 4565 | 23/12/04 |  |  |  |
| Surface PAR | 16504 | 02/01/04 |  |  |  |
| Fluorometer | 2356 |  | IOS |  |  |
| Pressure Sensor | 63507 | 25/10/2004 | Factory |  |  |