CSS Vector

Deck Lay Out: Thomson/Juhasz Cruise

Cruise ID: 2006 - 10

Date : There are 2 Legs :

May 29-Jun.5/6-Jun.12, 2006

| A: | Aft A-Frame Required |
| --- | --- |
| B: | Not Required-Please Remove |
| C: | Capstan Required |
| D: | a) May 29 – app. Jun. 5/6 Work Winch 329 #1306 c/w 1000m x 1/2” (3x19) wire b) Jun. 5/6 – Jun. 12 CTD Winch 329 #1307 c/w 3000m x .316" x 1cdr cable Swap at IOS |
| E: | Work Winch 331 #1231 c/w 2400m x 1/2” (3x19) wire |
| F: | Work Winch 420 #1309 c/w 1000m x 3/8” (7x19) wire Remove May 29-Jun. 5/6 Replace Jun. 5/6 – Jun. 12 Swap at IOS |
| G: | Aft Crane Required |
| H: | Hydro Winch 410 #1536 c/w 800m x 5/32" (3x19) wire |
| I: | Hydro Davit Required |
| J: | Bow Crane Required |
| K: | Not Required |
| L: | Not Required |
| M: | Not Required |
| N: | Not Required |
| O: | Not Required |

Please see attached comments.

Comments :

a) Phil Lobb, please load a CTD Block, a hydro block. Also load an Omron counter with each winch. We will also need one for sure, potentially 2, Osco blocks from the ship we will be lowering app. 5000-6000 lb anchors on 1/2” wire over these blocks suspended from the aft A-Frame.

b) Phil Lobb, please reterminate with new fittings and load test both the 1/2” wire winches, a) lift and hold 5000 lbs on winch hydraulics, and b) stop wire on manual brake and suspend 10000 lbs on termination. I will come and help with these tests.

Please ensure correct hydraulic flow and pressure is applied to winches on the ship.

c) The Vector, Captain and Bosun, for the first week, May 29 – Jun. 5/6, the cruise is dedicated to the deployment of 3 surface toroid buoy moorings for the ECOHAB project. These moorings are bigger and heavier than usual for the Vector. We will require storage space on the upper deck for mooring equipment, i.e., toroid buoys and mooring hardware not in play. The main deck will be filled with the on going work, i.e., the mooring we are deploying, as well as the anchors for all three moorings. The second half of the cruise entails more conventional work, typical moorings and CTD profiling.

Thank you,  Tom