File List for SeaBird Data

Cruise Number: _2006-11____

[osapcommon\cruise_data\2006-11\chl] Original titrated chlorophyll bottle files from analyst

[osapcommon\cruise_data\2006-11\ctd]

*.bl - original files from SeaSave

*.con - 		“

*.dat -      	“

*.hdr -		“

[osapcommon\cruise_data\2006-11\ctd\convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[osapcommon\cruise_data\2006-11\ctd\derive]

*.cnv - data after running WILDEDIT, CELLTM and DERIVE (for S0, S1)

[osapcommon\cruise_data\ctd\2006-11\ctd\doc]

2006-11-recal1.ccf – calibration control file for recalibration of DO

2006-11-recal2.ccf – calibration control file for 2nd recalibration of DO

2006-11-ctd.con – configuration file used for all casts

2006-11_file.doc - this document

2006-11_proc.doc - processing notes

2006-11-nuts-comments.doc – notes from M.Quenneville on nutrient sample # errors

*hdr.txt – header files with comments for CTD and CHE files

[osapcommon\cruise_data\2006-11\ctd\hydro]

*.add – bottle from analyst, converted, edited, with flags

*.bot  - *.ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.chl – extracted chlorophyll files converted from analyst’s spreadsheet

*.ed1 and *.log – edited bot file and editing log file for 3 casts

*.mrg – after addition of oxygen and salinity bottle data to BOT files

*.nuts – nutrient files converted from analyst’s spreadsheet

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted by spreadsheet

*.sam – bottle files with sample numbers added

addsamp.csv – file containing cast #, bottle number and sample number

2006-11chlarc.csv – edited version of CHL spreadsheet

2006-11-nuts.csv – edited version of the nutrient spreadsheet

2006-11sal.csv – edited version of salinity spreadsheet

2006-11-chl-fl-comp1.csv & xls – fluorescence/CHL COMPARE file

2006-11-dox-comp1.csv & .xls – initial comparison of titrated DO and CTD DO

2006-11-dox-comp2.csv & .xls – comparison of titrated DO and CTD DO after 1st recal

2006-11-dox-comp3.csv & .xls – comparison of titrated DO and downcast CTD DO

2006-11-dox-comp4.csv & .xls – comparison of titrated DO and CTD DO after final recal

2006-11-sal-comp1.csv & xls – initial salinity COMPARE file

2006-11-sample-#s.xls – study of sample number records from rosette and CTD logs

[osapcommon\cruise_data\2006-11\ctd\ios]

*.avg - data after metre-averaging using BINAVE

*.ctd - final files

*.del – after DELETE, downcast data for 1-111, upcast for 81(called 9981)

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT.

Delete.log - log from routine DELETE

[osapcommon\cruise_data\2006-11\Nutrients] Original bottle nutrient spreadsheet from analyst

[osapcommon\cruise_data\2006-11\Oxyen] Original oxygen bottle files from analyst

[osapcommon\cruise_data\2006-11\Salinity] Original bottle salinity spreadsheet from analyst

[osapcommon\cruise_data\2006-11\TSG]

*.cnv – after SeaBird conversion

*.con – configuration files

*.hex – original data acquired at sea

[osapcommon\cruise_data\2006-11\TSG\doc]

2006-11-TSG-recal.ccf – calibration control file for TSG

2006-11-TSG.con – conversion file used in processing

2006-11-loop-chem.csv – all useful data from loop samples

2006-11-loop-nuts.csv – nutrient data from loop samples

2006-11sal-loop – salinity data from loop samples

2006-11ctd-surf.xls – extract of CTD data from 3m

2006-11loopchl.xls – titrated chlorophyll from loop samples

2006-11-TSG-CTD-comp.xls – comparison of CTD and TSG matching times

2006-11-TSG-CTD-wellmixed.xls – subset of above for CTD casts well-mixed at surface

2006-11-TSG-sal-chl-loop-comp.xls – comparison of loop salinity and CHL samples to TSG

CDOM-sal-plot.doc – plot of TSG CDOM fluorescence versus TSG salinity

2006-11-TSG-hdr.txt – file with comments for headers

[osapcommon\cruise_data\2006-11\TSG\IOS] – data after conversion to IOS Headers

*.atc – files after adding time channel

*.ed1,ed2,ed3 – file after 3 stages of editing

*.ios – files after conversion from CSV to IOS headers

*.log and *.lo& - log file for editing runs

*.tob – final data file after REMOVE and HEAD EDIT