File List for SeaBird Data          Cruise Number: 2006-24

[osapcommon\cruise_data\2006-24\aandera]

[osapcommon\cruise_data\2006-24\ADCP]

[osapcommon\cruise_data\2006-24\ctd files]

*.bl - original files from SeaSave

*.con - 		“

*.dat -      	“

*.hdr -		“

[osapcommon\cruise_data\2006-24\ctd files\convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[osapcommon\cruise_data\2006-24\ctd files\derive]

*.cnv - data after running ALIGNCTD, WILDEDIT, CELLTM and DERIVE (for S0, S1)

[osapcommon\cruise_data\ctd\2006-24\ctd files\doc]

2006-24-recal1-bot.ccf – calibration control file for salinity, pressure and DO in bottle files

2006-24-recal2.ccf – calibration control file for initial recalibration of DO and salinity

2006-24-recal3.ccf – calibration control file for final recalibration of DO

2006-24-recal-pr.ccf – calibration control file for recalibrating pressure

2006-24-0001.con– configuration files used in processing

2006-24_proc.doc - processing notes

2006-24_file.doc - this document

*hdr.txt – various files containing comments used for headers

[osapcommon\cruise_data\2006-24\ctd files\hydro]

*.add – bottle from analyst, converted, edited, with flags

*.bot  - *.ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.mrg – merge of sal, add, nuts and bottle-averaged rosette files

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted by spreadsheet

*.sam – bottle files with sample numbers added

2006-24-sal.csv – spreadsheet of non-loop salinity data

addsamp*.csv – file containing cast #, bottle number and sample number

2006-24-sal-comp1-csv &.xls – salinity COMPARE fileS

2006-24-dox-comp1.csv & .xls – initial DO COMPARE files

2006-24-dox-comp2.csv & .xls – DO COMPARE files after 1st recalibration

2006-24-dox-comp3.csv & .xls – DO COMPARE files downcast vs upcast bottles

2006-24-dox-comp4.csv & .xls – DO COMPARE files after 2nd recalibration

[osapcommon\cruise_data\2006-24\ctd files\ios]

*.avg - data after metre-averaging using BINAVE

*.ctd - final files

*.del – after DELETE, downcast data for 1-111, upcast for 81(called 9981)

*.edt - data after editing has been done

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT.

Delete.log - log from routine DELETE

[osapcommon\cruise_data\2006-24\Oxygen] Original bottle data from analyst

[osapcommon\cruise_data\2006-24\S4MD]

[osapcommon\cruise_data\2006-24\salinity] Original bottle salinity files from analyst

[osapcommon\cruise_data\2006-24\TSG files]

*.cnv – after SeaBird conversion

*.con – configuration files

*.hex – original data acquired at sea

[osapcommon\cruise_data\2006-24\TSG\doc]

2006-24-TSG-recal.ccf – calibration control file for TSG

2005-24-sal-loop.csv – salinity data from bottle analysis

2006-24ctd-tsg-surf.xls – comparison of TSG data and CTD data from 4db

2005-24-sal-loop-tsg-comp.xls – comparison of loop samples and TSG salinity

2006-24-hdr-TSG.txt – file with comments for headers

[osapcommon\cruise_data\2006-24\TSG Data\IOS] – data after conversion to IOS Headers

*.atc – files after adding time channel

*.ed1 and ed2 – output of CTDEDIT

*.ios – files after conversion from CSV to IOS headers

*.log & .lo& - log files from CTDEDIT

*.tob – final data file after REMOVE and HEAD EDIT

[osapcommon\cruise_data\2006-24\UcAT]