January, 2007

Processing of CFMETR Nanoose Range CTD data

- Seabird processing:

- Outliers are identified as data greater than 2 standard deviations from mean calculated over a 30 scans (15 s), two passes are made for each profile.

- A 2.5 s (5 scans) low-pass filter is applied to the pressure signal.

- A 2.0 s (4 scans) low-pass filter is applied to the conductivity signal.

- Temperature readings are advanced by 0.6 s.

- Descent rates less than 0.25 ms-1 are flagged.

- The downcast data have been averaged into 1 m intervals.

- IOS Shell processing:

- Conversion from Seabird to IOS header format.

- Salinity derived from pressure, temperature and conductivity.

- Conductivity and flag channels removed leaving three channels (pressure, temperature and salinity).

- Matlab processing;

- Latitude and longitude for each cast calculated from ~.act file and written into CTD file.

- Plots generated showing location map, temperature and salinity profiles, and salinity and temperature contours (see below).

The 113 files have been given a .CTD extension and put in directory:

\\Paciosfp1\OSD_Data_Archive\Cruise_Data\2006\2006-99