Cruise 2007-01:  Line P February 2007

CTD/Rosette:

72 casts done, 0001 to 0133

35 ‘real rosette casts’ (33  IOS plus 1 UBC plus 1 calibration cast)

32 CTD casts in Rosette with 5m Niskin

5 stand-alone CTD casts, 0053 to 0060

All the casts were done with deck unit 0425; Mid-ship winch.

GENERAL NOTES:

Casts with PAR sensor:  0009, 0025, 0036, 0046, 0067, 0069, 0072

Config file WITH par channel for all 0443CTD casts.

Cast which needs a change in station name:

0004:  Is P4, should be P2.

Cast 0001: Oxygen sensor plug not removed.  Affected Salinity data also.

Cast 0088: Latitude: longitude, Longitude: time, Time: Latitude.

Cast 0122: Large spikes in T and S at surface (beginning).

Casts 0001 to 0052:

CTD 0443 in rosette frame.

Cast 0053, P22:  Switch to stand alone CTD 0585.  The primary temperature sensor – 2663 – seemed way off, mainly compared to the TSG temperature.  The transmissometer quit at ~ 400dbar on the way down, and came back at ~1000 dbar on the way up.

Casts 0055 to 0060:  stand-alone CTD 0585 with primary temperature sensor 2106.  Fluorometer 2228 removed.  Transmissometer cable replaced.

Casts 0066 to 0128:

CTD 0443 in rosette frame.

Casts 0129 to 0133:

CTD 0585 in rosette frame for calibration.  New configuration file, different from the 0585 stand-alone CTD config file.