File List for SeaBird Data          Cruise Number: _2017-19____

[osdcommon\cruise_data\2017-19\ARCHIVE]

[osdcommon\cruise_data\2017-19\ARCHIVE\Bottle] *.CHE

[osdcommon\cruise_data\2017-19\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data\2017-19\ARCHIVE\DOC] most documents from RAW and PROCESSING

[osdcommon\cruise_data\2017-19\ARCHIVE\DOC\HYDRO] COMPARE files

[osdcommon\cruise_data\2017-19\ARCHIVE\DOC\TSG] all TSG documents

[osdcommon\cruise_data\2017-19\ARCHIVE\LOGS] any scanned logs or Raw SCS files

[osdcommon\cruise_data\2017-19\ARCHIVE\Underway] includes TOB and loop files

[osdcommon\cruise_data\2017-19\PROCESSING]

[osdcommon\cruise_data\2017-19\PROCESSING\CONVERT]	*.CNV – after conversion

[osdcommon\cruise_data\2017-19\PROCESSING\DERIVE]*.CNV – after convert/wildedit/celltm/derive

[osdcommon\cruise_data\2017-19\PROCESSING\DOC]

2017-19-recal1.ccf – calibration control files

2017-19-ctd*.xmlcon – configuration files used in processing

2017-19_file.doc - this document

2017-19_Processing_Report.doc - processing notes

2017-19-ctd-final.rpt – CTD file header check including channel summary

2017-19-ctd-xref.lis – cross reference list of CTD files

2017-19-ctd-altimeter.csv – report on header entries for altimetry

*hdr.txt – header file with comments for CTD files

[osdcommon\cruise_data\2017-19\PROCESSING\DOC\hydro]

2017-19chem.csv – bottle data other than SAL, DO and CHL

2017-19chl.csv – extracted chlorophyll data simplified from analyst’s s/s

2017-19DO.csv – nutrients simplified from analyst’s s/s

2017-19sal.csv – salinity data from Autosal, simplified from analyst’s s/s

2017-19-fl-chl-comp1.csv & .xlsx – comparison of extracted CHL and CTD Fluorometer

2017-19-dox-comp1.csv & .xlsx – initial comparison of titrated DO & CTD DO

2017-19-dox-comp2.csv & .xlsx –comparison of titrated DO & CTD DO after initial recal

2017-19-dox-comp3.csv & .xlsx – comparison of titrated DO & downcast CTD DO

2017-19-sal-comp1.csv & xlsx –initial comparison of rosette bottle and CTD salinity

2017-19-sal-comp2.csv & xlsx –comparison of rosette bottle and CTD salinity after recalibration

addsamp.csv – spreadsheet relating bottle numbers to sample numbers

2017-19-bottles-final.xlsx – summary of bottle data in CHE files

2017-19-che-final.rpt – final header check of bottle files

2017-19-che-xref.lis – cross-reference listing of bottle files

2017-19- altimeter-che.csv – altimetry and water depth information

*hdr.txt – header file with comments for CHE files

[osdcommon\cruise_data\2017-19\PROCESSING\HYDRO]

*.bot - ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.chl – extracted chlorophyll data converted from simplified version of spreadsheet

*.comb – bottle data other than DO and SAL converted from chemistry spreadsheet

*.ed1 & *.log* – output from CTDEDIT

*.mrg – after addition of oxygen and salinity bottle data to BOT files

*.mrgreo – after derivations and reordering

*.mrh – after addition of positions and station names and water depths

*.oxy – dissolved oxygen files converted from simplified version of analyst’s spreadsheet

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted from simplified version of analyst’s spreadsheet

*.sam – bottle files with sample numbers added

[osdcommon\cruise_data\2017-19\PROCESSING\IOS] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.clip – after removal of sub-surface soak records

*.cln - data after conversion to IOS HEADERS and a run of CLEAN

*.ctd - final files

*.del – after DELETE

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios – data after conversion to IOS SHELL format

*.log* - log files created by CTDEDIT

*.mrh – after addition of positions and station names and water depths

*.reo – after derivations and reordering

Delete.log - log from routine DELETE

[osdcommon\cruise_data\2017-19\RAW]

[osdcommon\cruise_data\2017-19\RAW\Chemistry] data from analysts

[osdcommon\cruise_data\2017-19\RAW\CTD] raw CTD data

[osdcommon\cruise_data\2017-19\RAW\CTD19+] raw CTD19 data

[osdcommon\cruise_data\2017-19\RAW\DOC] documents – hydro file

[osdcommon\cruise_data\2017-19\RAW\LOGS] logs

[osdcommon\cruise_data\2017-19\RAW\TSG] raw TSG data

[osdcommon\cruise_data\2017-19\RAW\XBT] XBT data

[osdcommon\cruise_data\2017-19\RAW\XCTD] raw XCTD data