File List for SeaBird Data     	Cruise Number: _2008-12____

[osdcommon\cruise_data_processing\2008-12\ARCHIVE]

[osdcommon\cruise_data_processing\2008-12\ARCHIVE\CTD]   *.CTD

[osdcommon\cruise_data_processing\2008-12\ARCHIVE\DOC] all documents from RAW and PROCESSING

[osdcommon\cruise_data_processing\2008-12\ARCHIVE\DOC\hydro] compare documents

[osdcommon\cruise_data_processing\2008-12\processing\]

*.hex – renamed original files from SeaSave

[osdcommon\cruise_data_processing\2008-12\PROCESSING\Angelica] special files for A. Peña

[osdcommon\cruise_data_processing\2008-12\ processing\convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[osdcommon\cruise_data_processing\2008-12\ processing\derive]

*.cnv - data after running WILDEDIT, WFILTER, CELLTM and DERIVE

[osdcommon\cruise_data_processing\2008-12\ processing\doc]

0404ctd.con – configuration file used for casts 50-377, with fluor & trans.

0404b ctd.con – configuration file with fluor, no transmissometer, not used

0404c ctd.con – configuration file used for casts 2-46, no fluor, no trans.

2008-12-file.doc - this document

2008-12-proc.doc - processing notes

2008-12-*.txt – text files for addition of comments to headers

2008-12-ctd-final.rpt – final header check report

*.xls – spreadsheets from analysts

[osdcommon\cruise_data_processing\2008-12\ processing\doc\hydro]

2008-12-sal-chl-comp.xls – comparison of CTD and bottle salinity and CHL

2008-12-bottles.xls – summary of all bottle sampling

[osdcommon\cruise_data_processing\2008-12\ processing\Howard]

special files used for fixing original hex files casts 2-46

[osdcommon\cruise_data_processing\2008-12\ processing\ios]

*.atc – files after addition of positions and correction of time

*.avg – bin-averaged files

*.ctd - final files

*.del – data after DELETE

*.delrev – data after DELETE run on reversed files

*.edt - data after editing has been done

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT

Delete.log - log from routine DELETE

[osdcommon\cruise_data_processing\2008-12\ raw\]

Raw data and chemistry files from analysts