XCTD

1. Personal name and affiliation

Shigeto NISHINO

Japan Agency for Marine-Earth Science and Technology (JAMSTEC)

2. Objective

Measurements of temperature and salinity profiles in the Arctic Ocean to understand water mass distributions and the ocean circulation, which are related to the sea-ice distribution, heat and freshwater transports, biogeochemical cycles, and their temporal variations in the changing Arctic climate system.

3. Parameters

Depth, temperature, and salinity (conductivity).

4. Methods

XCTD (eXpendable Conductivity – Temperature – Depth profiler, Tsurumi-Seiki Co., Ltd.) probes were launched by a hand launcher LM-3A (Tsurumi-Seiki Co., Ltd.) from the stern of the ship into the seawater to measure the vertical profiles of water temperature and salinity to a depth of 1000 m (or the bottom in case where the bottom depth is less than 1000 m). The data is communicated back to a digital data converter MK-130 (Tsurumi-Seiki Co., Ltd.) and a computer onboard the ship by a fine wire which breaks when the probe reaches its maximum depth.

According to the manufacturer’s nominal specifications, the range and accuracy of parameters measured by the XCTD are as follows;

Parameter	Range		Accuracy

Conductivity	0 ~ 60 [mS/cm]	+/- 0.03 [mS/cm]

Temperature	-2 ~ 35 [deg-C]	+/- 0.02 [deg-C]

Depth		0 ~ 1000 [m]	5 [m] or 2 [%] (either of them is major)

In this cruise, 107 XCTDs were launched into the Arctic Ocean, mainly into the Canada Basin. Three XCTDs (Cast No. 25, 48, and 66) were failed to obtain the data because of communication error. This error usually happens when we hit the Abort button just after the breaking of wire. This is because when we hit the Abort button, the compute tries to communicate with the XCTD probe, but the broken wire results in the communication error. Do not hit the Abort button. Cut the wire first. If you find the communication error in spite of not hitting the Abort button, wait for 20-30 seconds. The computer automatically saves the data. Another possibility for the communication error is related to a virus software. When the virus scan starts in the midst of measurement, the data transport may be disturbed by the virus scan. After removing the virus software, this error did not occur. In a heavy ice region, the ship should stop to drop the XCTD into open water not to be broken the wire by the sea-ice motion. The XCTD casts are listed in Table.

5. Preliminary Results

Figures 1a and 1b show vertical sections of temperature and salinity, respectively, along ~ 150W. For the illustrations, the XCTD data were combined with the CTD data. We found relatively warm and fresh surface water in the Northwind Ridge region enclosed by a red circle on the map of Figure 1c. There was a large area of open water (Figure 1e). Therefore, the warm and fresh water is thought to be largely influenced by sea-ice melt water. On the other hand, north of the region, the surface temperature decreases and salinity increases (Figures 1a and 1b), and the sea-ice covered the ocean (Figure 1d), suggesting a less influence of sea-ice melt water.

Figure 1. Vertical sections of (a) temperature and (b) salinity along ~ 150W, (c) map and trajectory for the illustrations of vertical sections, and photos taken at (d) 77 10’ N, 149 15’ W and (e) 75 41’ N, 156 30’ W. For the illustrations of (a), (b) and (c), the XCTD data were combined with the CTD data.

Figures 2a and 2b show temperature and salinity distributions, respectively, at a depth of 10m. For the illustrations, the XCTD data were combined with the CTD data. A region with higher temperature and lower salinity south of ~ 77N in the 150W line and south of ~ 75N in the 140W line corresponds to an open water region (Figure 2c). North of the region, sea-ice covers the ocean. The surface temperature and salinity distributions are very consistent with the sea-ice distribution.

(a)

(b)

(c)

Figure 2. (a) Temperature and (b) salinity distributions at a depth of 10 m, and (c) RADARSAT sea-ice image on 13 August 2008, derived from a public drive in CCGS Louis S. St-Laurent during the cruise. For the illustrations of (a) and (b), the XCTD data were combined with the CTD data.

Figures 3a and 3b show distributions of temperature on an isohaline surface of S = 32 and dynamic height from 1000 m to 100 m, the depth of which roughly corresponds to the level of S = 32 isohaline surface. For the illustrations, the XCTD data were combined with the CTD data. Furthermore, the data from R/V Mirai of JAMSTEC in 2004 (Shimada, 2004) were combined to compare our results with that of the western part of the Canada Basin. The S = 32 isohaline surface is located at the bottom of Pacific summer water (PSW), which spreads east of the Chukchi Plateau (Shimada et al., 2001; See also red allows in Figure 3b). West of the Chukchi Plateau, temperature minimum water occupies along the S = 32 isohaline surface, and the water is thought to be PSW that has been cooled in winter on its way to the Chukchi Abyssal Plain (Nishino et al., 2008). On the S = 32 isohaline surface, low temperature water was found not only in the Chukchi Abyssal Plain (western part of the Canada Basin) but also northern and eastern parts of the Canada Basin (Figure 3a). Based on the dynamic height, anticyclonic circulation (blue arrows in Figure 3b) seems to carry the lowest temperature water in the Chukchi Abyssal Plain into the northern and eastern parts of the Canada Basin. The temperature minimum water of S = 32 in the Chukchi Abyssal Plain has relatively high nutrients, because the water would have passed through the western Chukchi Sea where the nutrients are regenerated from the bottom of the shelf. Therefore, the water would play an important role in biological activities in the Canada Basin, if the sea-ice retreats to the northern part of the Canada Basin.

(a)

(b)

Figure 3. (a) Temperature on an isohaline surface of S = 32 and (b) dynamic height at a depth of 100 m relative to 1000 m. In (b), schematic circulation patterns on the Pacific summer water and its modified water by winter cooling are shown by red and blue arrows, respectively.

6. Data Management Policy

The XCTD observation is conducted under the program of JWACS (Joint Western Arctic Climate Studies) between JAMSTEC and DFO. Data will be shared among the participants of this cruise (LSSL2008-30). Sharing the data with third party shall be by mutual consent between JAMSTEC and IOS/DFO.

Science coordinators

JAMSTEC: Koji Shimada (shimadak@jamstec.go.jp) *1

IOS/DFO: Eddy Carmack (Eddy.Carmack@dfo-mpo.gc.ca)

*1 Now at Tokyo University of Marine Science and Technology; koji@kaiyodai.ac.jp

References

Nishino, S., K. Shimada, M. Itoh, M. Yamamoto-Kawai, and S. Chiba (2008), East-west differences in water mass, nutrient, and chlorophyll a distributions in the sea-ice reduction region of the western Arctic Ocean, J. Geophys. Res., in press.

Shimada, K. (2004), R/V Mirai Cruise Report MR04-05, edited by K. Shimada, S. Nishino, and M. Itoh, 110pp., JAMSTEC, Yokosuka, Japan.

Shimada, K., E.C. Carmack, K. Hatakeyama, and T. Takizawa (2001), Varieties of shallow temperature maximum waters in the western Canada Basin of the Arctic Ocean, Geophys. Res. Lett., 28, 3441–3444.

Table. 2008-30 XCTD Log

| Station | Month | Day | Time(UTC) | Latitude | Longitude | Depth | File name | Probe number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XCTD-001 | Jul | 22 | 01:37:04 | 70 59.3320N | 125 58.8160W | 401 | 200830-XCTD-001 | 07012631 |
| XCTD-002 | Jul | 22 | 05:57:26 | 71 26.2364N | 128 58.9958W | 111 | 200830-XCTD-002 | 07012638 |
| XCTD-003 | Jul | 22 | 08:19:54 | 71 39.0627N | 130 28.9011W | 291 | 200830-XCTD-003 | 07012634 |
| XCTD-004 | Jul | 22 | 20:17:00 | 71 34.6997N | 133 27.5859W | 1100 | 200830-XCTD-004 | 07012635 |
| XCTD-005 | Jul | 22 | 22:28:55 | 71 31.2196N | 134 58.0187W | 1600 | 200830-XCTD-005 | 07012636 |
| XCTD-006 | Jul | 23 | 00:44:50 | 71 32.1910N | 136 27.8907W | 1600 | 200830-XCTD-006 | 07012632 |
| XCTD-007 | Jul | 23 | 02:44:40 | 71 33.0685N | 137 58.0545W | 2600 | 200830-XCTD-007 | 07012633 |
| XCTD-008 | Jul | 23 | 09:37:17 | 71 17.5744N | 140 00.1257W | 2275 | 200830-XCTD-008 | 07022787 |
| XCTD-009 | Jul | 23 | 21:00:21 | 70 24.2843N | 140 00.2882W | 508 | 200830-XCTD-009 | 07022786 |
| XCTD-010 | Jul | 24 | 02:50:02 | 70 16.7763N | 140 28.4651W | 160 | 200830-XCTD-010 | 07022785 |
| XCTD-011 | Jul | 24 | 03:57:41 | 70 28.9957N | 140 47.8913W | 510 | 200830-XCTD-011 | 07022788 |
| XCTD-012 | Jul | 24 | 04:24:04 | 70 35.6199N | 140 59.1193W | 1050 | 200830-XCTD-012 | 07022790 |
| XCTD-013 | Jul | 24 | 04:54:57 | 70 41.3533N | 141 09.6001W | 1100 | 200830-XCTD-013 | 07022789 |
| XCTD-014 | Jul | 24 | 05:43:42 | 70 51.1745N | 141 24.7164W | 2014 | 200830-XCTD-014 | 07033322 |
| XCTD-015 | Jul | 24 | 06:51:21 | 71 05.9214N | 141 50.2811W | 2498 | 200830-XCTD-015 | 07033323 |
| XCTD-016 | Jul | 24 | 08:42:27 | 71 29.2854N | 142 32.4353W | 2875 | 200830-XCTD-016 | 07033324 |
| XCTD-017 | Jul | 24 | 10:33:46 | 71 54.0458N | 143 15.4283W | 3000 | 200830-XCTD-017 | 07033327 |
| XCTD-018 | Jul | 24 | 12:32:01 | 72 18.0571N | 143 58.6571W | 3204 | 200830-XCTD-018 | 07033326 |
| XCTD-019 | Jul | 25 | 00:29:59 | 72 29.5563N | 146 12.3447W | 3400 | 200830-XCTD-019 | 07033325 |
| XCTD-020 | Jul | 25 | 02:29:00 | 72 18.0288N | 147 42.1693W | 3500 | 200830-XCTD-020 | 07022654 |
| XCTD-021 | Jul | 25 | 04:59:20 | 72 06.1120N | 149 11.7738W | 3400 | 200830-XCTD-021 | 07022653 |
| XCTD-022 | Jul | 25 | 11:29:29 | 71 48.8198N | 150 38.8310W | 2525 | 200830-XCTD-022 | 07022652 |
| XCTD-023 | Jul | 26 | 06:19:31 | 72 14.9677N | 150 22.0014W | 3500 | 200830-XCTD-023 | 07022651 |
| XCTD-024 | Jul | 26 | 07:25:20 | 72 29.8832N | 149 59.9132W | 3655 | 200803-XCTD-024 | 07022650 |
| XCTD-025 | Jul | 26 | 08:26:14 | 72 44.6301N | 149 59.6209W | 3665 | Error: no file | 07022649 |
| XCTD-026 | Jul | 26 | 14:31:10 | 73 30.2873N | 150 00.2353W | 3728 | 200830-XCTD-026 | 07022648 |
| XCTD-027 | Jul | 26 | 21:43:50 | 74 14.8062N | 148 52.6575W | 3773 | 200830-XCTD-027 | 07022682 |
| XCTD-028 | Jul | 27 | 03:39:08 | 74 34.6676N | 148 50.7080W | 3800 | 200830-XCTD-028 | 07022683 |
| XCTD-029 | Jul | 29 | 00:36:43 | 75 08.1276N | 151 40.7598W | 3830 | 200830-XCTD-029 | 07022684 |
| XCTD-030 | Jul | 29 | 02:10:36 | 75 12.7227N | 152 29.5652W | 3738 | 200830-XCTD-030 | 07022687 |
| XCTD-031 | Jul | 29 | 08:41:42 | 75 22.3169N | 153 57.2506W | 3847 | 200830-XCTD-031 | 07022690 |
| XCTD-032 | Jul | 29 | 10:04:10 | 75 26.6050N | 154 34.4416W | 3838 | 200830-XCTD-032 | 07022686 |
| XCTD-033 | Jul | 29 | 15:01:47 | 75 37.4973N | 155 53.0003W | 2883 | 200830-XCTD-033 | 07022689 |
| XCTD-034 | Jul | 29 | 15:24:48 | 75 37.8361N | 155 58.1670W | 2470 | 200830-XCTD-034 | 07022685 |
| XCTD-035 | Jul | 29 | 16:00:26 | 75 38.1414N | 156 08.0241W | 2032 | 200830-XCTD-035 | 07022693 |
| XCTD-036 | Jul | 29 | 19:51:11 | 75 42.5469N | 156 44.9696W | 1110 | 200830-XCTD-036 | 07022688 |
| XCTD-037 | Jul | 29 | 23:20:41 | 75 40.8785N | 155 59.0408W | 2350 | 200830-XCTD-037 | 07022692 |
| XCTD-038 | Jul | 29 | 23:53:18 | 75 42.0071N | 155 40.5835W | 3666 | 200830-XCTD-038 | 07022691 |
| XCTD-039 | Jul | 30 | 00:19:50 | 75 43.0046N | 155 21.3479W | 3842 | 200830-XCTD-039 | 08048177 |
| XCTD-040 | Jul | 30 | 01:16:42 | 75 44.9724N | 154 39.9922W | 3837 | 200830-XCTD-040 | 08048178 |
| XCTD-041 | Jul | 30 | 02:13:25 | 75 47.5712N | 154 01.0570W | 3836 | 200830-XCTD-041 | 08048179 |
| XCTD-042 | Jul | 30 | 03:08:50 | 75 49.6017N | 153 21.7657W | 3835 | 200830-XCTD-042 | 08048182 |
| XCTD-043 | Jul | 30 | 04:02:24 | 75 53.5371N | 152 43.5592W | 3834 | 200830-XCTD-043 | 08048181 |
| XCTD-044 | Jul | 30 | 04:58:41 | 75 54.5824N | 151 59.8184W | 3831 | 200830-XCTD-044 | 08048180 |
| XCTD-045 | Jul | 30 | 05:47:01 | 75 55.7894N | 151 21.9770W | 3829 | 200830-XCTD-045 | 08048183 |
| XCTD-046 | Jul | 30 | 13:06:27 | 76 29.7679N | 150 00.2432W | 3891 | 200830-XCTD-046 | 08048184 |
| XCTD-047 | Jul | 30 | 22:36:27 | 77 21.0931N | 148 22.3070W | 3821 | 200830-XCTD-047 | 08048185 |
| XCTD-048 | Jul | 31 | 08:50:00 | 77 51.0000N | 148 20.0000W | 3822 | Error: no file | 08048186 |
| XCTD-048 | Jul | 31 | 09:13:17 | 77 51.5341N | 148 20.4556W | 3821 | 200830-XCTD-048 | 08048187 |
| XCTD-049 | Aug | 2 | 02:46:46 | 78 05.1155N | 151 03.6273W | 3830 | 200830-XCTD-049 | 08048188 |
| XCTD-050 | Aug | 2 | 03:58:30 | 78 09.5133N | 151 49.7121W | 3834 | 200830-XCTD-050 | 08048212 |
| XCTD-051 | Aug | 2 | 05:06:08 | 78 14.5826N | 152 31.3195W | 3650 | 200830-XCTD-051 | 08048211 |
| XCTD-052 | Aug | 2 | 05:41:49 | 78 16.2959N | 152 56.2482W | 3314 | 200830-XCTD-052 | 08048208 |
| XCTD-053 | Aug | 2 | 21:40:27 | 78 22.7386N | 153 24.9442W | 1804 | 200830-XCTD-053 | 08048209 |
| XCTD-054 | Aug | 2 | 22:20:11 | 78 27.2053N | 153 11.5732W | 2786 | 200830-XCTD-054 | 08048206 |
| XCTD-055 | Aug | 2 | 22:39:44 | 78 28.7001N | 152 57.4424W | 3494 | 200830-XCTD-055 | 08048203 |
| XCTD-056 | Aug | 2 | 23:30:32 | 78 36.5575N | 152 27.6839W | 3823 | 200830-XCTD-056 | 08048205 |
| XCTD-057 | Aug | 3 | 00:39:18 | 78 43.7739N | 151 41.7788W | 3822 | 200830-XCTD-057 | 08048202 |
| XCTD-058 | Aug | 3 | 02:06:30 | 78 49.9148N | 151 02.8120W | 3830 | 200830-XCTD-058 | 08048201 |
| XCTD-059 | Aug | 3 | 11:46:57 | 79 30.1020N | 149 59.7120W | 3820 | 200830-XCTD-059 | 08048204 |
| XCTD-060 | Aug | 4 | 01:36:47 | 80 20.0849N | 150 11.9722W | 3806 | 200830-XCTD-060 | 08048207 |
| XCTD-061 | Aug | 4 | 03:56:12 | 80 39.6592N | 149 59.4932W | 3800 | 200830-XCTD-061 | 08048210 |
| XCTD-062 | Aug | 4 | 10:39:14 | 81 06.2235N | 149 48.1921W | 3793 | 200830-XCTD-062 | 08048191 |
| XCTD-063 | Aug | 4 | 12:22:44 | 81 14.5010N | 150 01.7611W | 3794 | 200830-XCTD-063 | 08048190 |
| XCTD-064 | Aug | 4 | 15:02:58 | 81 30.3961N | 149 49.3218W | 3473 | 200830-XCTD-064 | 08048189 |
| XCTD-065 | Aug | 4 | 17:26:04 | 81 44.8635N | 149 57.0828W | 3388 | 200830-XCTD-065 | 08048192 |
| XCTD-066 | Aug | 5 | 08:25:00 | 82 14.0000N | 149 06.0000W | 3085 | Error: no file | 08048195 |
| XCTD-066 | Aug | 5 | 08:35:02 | 82 14.8477N | 149 06.5991W | 3085 | 200830-XCTD-066 | 08048196 |
| XCTD-067 | Aug | 5 | 11:00:11 | 82 29.4205N | 150 00.2798W | 3057 | 200830-XCTD-067 | 08048199 |
| XCTD-068 | Aug | 5 | 12:39:10 | 82 44.8028N | 150 00.4995W | 2747 | 200830-XCTD-068 | 08048193 |
| XCTD-069 | Aug | 6 | 01:06:49 | 82 51.3379N | 148 27.9855W | 2754 | 200830-XCTD-069 | 08048194 |
| XCTD-070 | Aug | 6 | 03:14:40 | 82 41.6791N | 146 55.4166W | 2716 | 200830-XCTD-070 | 08048197 |
| XCTD-071 | Aug | 6 | 05:19:02 | 82 33.2314N | 145 21.3063W | 2599 | 200830-XCTD-072 | 08048200 |
| XCTD-072 | Aug | 6 | 07:43:30 | 82 25.0183N | 143 51.8429W | 3065 | 200830-XCTD-072 | 08048198 |
| XCTD-073 | Aug | 6 | 10:36:37 | 82 12.1585N | 141 53.6764W | 2938 | 200830-XCTD-073 | 07012330 |
| XCTD-074 | Aug | 7 | 00:17:39 | 81 45.0393N | 140 01.6944W | 3676 | 200830-XCTD-074 | 07012331 |
| XCTD-075 | Aug | 7 | 02:19:39 | 81 30.1460N | 140 02.5447W | 3749 | 200830-XCTD-075 | 07012332 |
| XCTD-076 | Aug | 7 | 04:21:39 | 81 15.0327N | 140 02.4305W | 3755 | 200830-XCTD-076 | 07012334 |
| XCTD-077 | Aug | 7 | 11:30:53 | 80 45.0074N | 140 06.5006W | 3756 | 200830-XCTD-077 | 07012335 |
| XCTD-078 | Aug | 7 | 13:27:53 | 80 30.2099N | 139 58.8717W | 3759 | 200830-XCTD-078 | 07012333 |
| XCTD-079 | Aug | 7 | 15:17:52 | 80 15.3445N | 139 58.8784W | 3763 | 200830-XCTD-079 | 07012336 |
| XCTD-080 | Aug | 7 | 23:20:46 | 79 39.9666N | 140 00.4996W | 3766 | 200830-XCTD-080 | 07012337 |
| XCTD-081 | Aug | 8 | 02:04:39 | 79 20.9682N | 139 47.2455W | 3733 | 200830-XCTD-081 | 07012338 |
| XCTD-082 | Aug | 8 | 12:26:42 | 78 30.6552N | 140 06.6942W | 3773 | 200830-XCTD-082 | 07012343 |
| XCTD-083 | Aug | 9 | 04:37:15 | 77 35.8110N | 140 57.2985W | 3755 | 200830-XCTD-083 | 07012342 |
| XCTD-084 | Aug | 10 | 09:39:32 | 76 50.9504N | 138 29.9093W | 3694 | 200830-XCTD-084 | 07012341 |
| XCTD-085 | Aug | 10 | 12:14:35 | 76 41.1102N | 136 58.2792W | 3648 | 200830-XCTD-085 | 07012604 |
| XCTD-086 | Aug | 10 | 22:33:28 | 76 18.9194N | 134 15.2954W | 3407 | 200830-XCTD-086 | 07012605 |
| XCTD-087 | Aug | 11 | 16:19:31 | 76 02.8895N | 133 09.9712W | 3180 | 200830-XCTD-087 | 07012601 |
| XCTD-088 | Aug | 11 | 19:49:54 | 76 01.3222N | 135 25.9633W | 3484 | 200830-XCTD-088 | 07012602 |
| XCTD-089 | Aug | 11 | 22:40:45 | 76 01.4263N | 137 42.2007W | 3691 | 200830-XCTD-089 | 07012603 |
| XCTD-090 | Aug | 12 | 21:35:49 | 75 30.6117N | 140 20.0578W | 3697 | 200830-XCTD-090 | 07012606 |
| XCTD-091 | Aug | 13 | 04:54:28 | 74 39.6560N | 141 37.3119W | 3686 | 200830-XCTD-091 | 07012598 |
| XCTD-092 | Aug | 13 | 12:01:27 | 74 09.2825N | 141 43.1507W | 3644 | 200830-XCTD-092 | 07012599 |
| XCTD-093 | Aug | 14 | 21:26:56 | 73 44.6836N | 139 01.9916W | 3381 | 200830-XCTD-093 | 07012595 |
| XCTD-094 | Aug | 15 | 03:05:46 | 73 13.9451N | 138 58.1196W | 3189 | 200830-XCTD-094 | 07012596 |
| XCTD-095 | Aug | 15 | 09:36:39 | 72 52.1575N | 141 31.7930W | 3326 | 200830-XCTD-095 | 07012600 |
| XCTD-096 | Aug | 15 | 11:34:45 | 72 44.2080N | 143 05.8604W | 3381 | 200830-XCTD-096 | 07012597 |
| XCTD-097 | Aug | 15 | 13:39:40 | 72 36.0040N | 144 41.9187W | 3436 | 200830-XCTD-097 | 07022647 |
| XCTD-098 | Aug | 15 | 20:07:41 | 72 21.4703N | 142 59.2143W | 3293 | 200830-XCTD-098 | 07022646 |
| XCTD-099 | Aug | 15 | 22:10:29 | 72 06.9032N | 141 15.9701W | 3946 | 200830-XCTD-099 | 07022696 |
| XCTD-100 | Aug | 16 | 02:07:38 | 71 39.8264N | 138 10.9783W | 2254 | 200830-XCTD-100 | 07021633 |
| XCTD-101 | Aug | 16 | 04:12:16 | 71 25.2484N | 136 32.1256W | 1633 | 200830-XCTD-101 | 07022695 |
| XCTD-102 | Aug | 16 | 07:31:00 | 71 07.4820N | 134 34.7110W | 530 | 200830-XCTD-102 | 07022643 |
| XCTD-103 | Aug | 19 | 15:17:25 | 72 21.6317N | 138 01.1504W | 2522 | 200830-XCTD-103 | 07022644 |
| XCTD-104 | Aug | 19 | 17:48:41 | 72 42.4038N | 136 01.8648W | 2653 | 200830-XCTD-104 | 07022645 |
| XCTD-105 | Aug | 19 | 23:59:04 | 72 04.4863N | 132 58.5741W | 1741 | 200830-XCTD-105 | 07022698 |