## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |

## PROCESSING NOTES

Cruise: 2008-049

Agency: OSD

Location: Canadian Beaufort Shelf

Project: Northern Coastal Marine Studies on the Canadian Beaufort Shelf

Chief Scientist: Williams, W.

Platform: Nahidik

Date: 23 July 2008 – 19 August 2008

Processed by: Jeannette Bedard

Date of Processing: 14 March 2024 – 2 April 2024

Number of HEX files: 207		Number of CTD files processed: 206

Number of original TSG files: 0		Number of TSG files processed:  0

# INSTRUMENT SUMMARY

CTD #0415 was used for the entire cruse. Piggybacked instruments are listed in the table at the end of this report. No TSG files were found.

Seasave version 7.17a was used for acquisition.

# SUMMARY OF QUALITY AND CONCERNS

Only profile data has been processed. There was bottle sampling, but this data was not available at the time of processing.

Both the PAR sensor and ISUS nitrate sensors had issues during the cruise, their data was examined and determined to be bad. These data channels were removed.

# PROCESSING SUMMARY

The following steps were performed by Bill Williams and Sarah Zimmerman using the SeaBird software (cut and pasted from their report):

Three Seabird configuration files were used during the cruise.

Casts ?	  	SBE25_sn0415_10xFL_1xTurb_350m_23Jul2008.con		(file date 26 Jul 2008)

Casts ?	 	SBE25_sn0415_10xFL_1xTurb_1000m_23Jul2008.con		(file date 30 Jul 2008)

Casts ? 		SBE25_sn0415_10xFL_1xTurb_350m_NMEAPC_31Jul2008.con (file date 31 Jul 2008)

Sea-Bird’s Windows-based processing software, SBE Data Processing v 5.37e was used to produce 0.5db averaged downcast profiles.  The standard processing step as suggested by Seabird for the SBE25 CTD data were applied.

Following this, it looked like cast 052 and 187 needed some editing (April 2009).

Data Conversion

Data were converted from raw hex to ascii format, skipping over the surface soak scans specified for each station.

Bottle file average 16 scans (2 sec) using starting scan indicated in *.bl file (raw data file).

Bottle Summary

The scans associated with the bottle file are averaged.  Salinity, potential temperature and oxygen added to the output.

Wild Edit

Data spikes were removed from variables pressure, temperature, conductivity and oxygen voltage using the criteria of 10 standard deviations for pass 1, 20 standard deviations for pass 2, and 8 scans per block.

Cell Thermal Mass

A correction for the thermal mass of the conductivity sensor was applied using the suggested values of 0.04 and 8.0 for alpha and tau terms respectively.

Align

Oxygen voltage was advanced by 5 seconds relative to the other variables.

Filter

Data were filtered with a low pass filter 0.500 seconds for Pressure

Data were filtered with low pass filter of 0.030 seconds for conductivity.

Loop Edit

Pressure reversals were removed by identifying all data with velocities below 0m/s.

Surface soak was removed (should be gone using the starting scan specified in data convert) using min depth of 0 and max depth of 5db.

Derive

Calculated properties, salinity and oxygen were derived.  Oxygen used 2 second averaging of doc/dt and the Owens-Millard equations.

Bin Average

Data are averaged into 0.5 dbar bins.

Split

The files are separated between down and upcast profiles.

##### Conversion to IOS Header Format

The IOSSHELL routine was used to convert SEA-Bird CNV files to IOS Headers.

Encountered an error with cast 195, .cnv file only contains ‘dnahidik_2008_ctd_004.cnv’ – removed this file from further processing.

CLEAN was run to add event numbers and to replace pad values in the pressure channel with interpolated values based on record number.

Station names and depths were added using Merge: CSV file to Headers.

- Casts 6, 68 and 70 did not have a water depth from when the CTD was removed from the water. For these three the CTD ‘in the water’ value was used instead.

##### Checking Headers

- The header check report and the cross-reference check was run and turned up no problems.

- Surface check was run and found an average of 2.13db.

- Cast locations were plotted and added to the end of this report. There were too many casts to include station names and have the locations remain readable. A few casts appear to be on land, however it was assumed this was due to the resolution of the coastline data.

Added a depth channel using IOShell Derived Quantities—used .depth extension.

The altimeter and water depth readings from the headers of the .depth files were exported to spreadsheets.

A check value was calculated as follows:

CHECK VALUE = Max Depth Sampled + Altimetry Header - Water Depth from Header

The following casts did not have altimeter values in the header: 76, 95, 96 and 97.

26 casts have a check value>10m with the greatest 93.8m.

- 21 of these casts extended deeper than the max site depth entered in the logs. The max depth for these casts were replaced with max sample depth + altimeter value.

  - These casts were: 13, 14, 15, 29, 30, 31, 32, 39, 42, 49, 54, 78, 99, 160, 161, 162, 177, 187, 201 and 203.

  - Merge header was run a second time to correct these depths.

- The remaining 5 casts appeared to be where the cast was cut short, these were left as they are. These casts were: 53, 75, 92, 98, 131, and 166.

note: many of the max water depths from the logs are suspiciously rounded numbers like 100 or 45 m, suggesting they might be from very rough readings of charts.

##### Shift

- Skipped due to earlier SeaBird processing

##### DELETE

- Skipped due to earlier SeaBird processing

##### CLIP

- Not needed.

##### DETAILED EDITING

Only a few casts required editing. Minor salinity spikes were removed from casts 26, 35, 64, 75, 89 , 128 and 156. Notes about editing applied were added to the files.

Note that cast 92 only has a single bin of data, however it looks valid.

The edited files were copied to *.EDT.

After editing T-S plots were examined for all casts. No further editing was needed.

##### Corrections to Pressure, Salinity and Dissolved Oxygen Concentration

There is no indication of any problems and corrections were applied during the SeaBird processing. No recalibrations were applied.

##### Fluorescence Processing

Not applied as data was already binned.

##### BIN AVERAGE of CTD files

Bin averaging was done during the SeaBird processing.

##### Final CTD File Steps (REMOVE and HEADEDIT)

For all casts REMOVE was run to remove the following channels: Scan_Number, Oxygen:Voltage:SBE and Flag.

PAR – It was noted during the cruise that the PAR sensor was damaged. This data was examined and determined to be bad. This channel was removed.

ISUS nitrate meter – It was noted during the cruise that this sensor was not functioning due to an issue with the instrument’s mirror. This channel was removed.

HEADER EDIT was used to add comments about processing.

The Header Check was run; no problems were found. The Standards Check routine was run and a number of issues were found and addressed.

Profile and T-S plots were examined. No problems were found.

CRUISE SUMMARY – CTD

| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | SEABIRD | 25 | 0415 |  |  |  |
| Calibration Information - 1453 |  |  |  |  |  |  |
| Sensor |  |  | Pre-Cruise |  | Post Cruise |  |
| Name |  | S/N | Date | Location | Date | Location |
| Temperature |  | 4444 | 18Dec2007 |  |  |  |
| Conductivity |  | 3209 | 4Dec2007 |  |  |  |
| Pressure 1000dbar |  | 0603 |  |  |  |  |
| Pressure 350dbar |  | 0530 |  |  |  |  |
| Dissolved Oxygen |  | 1202 | 13Mar2008 |  |  |  |
| Seapoint Fluorometer |  | SCF2859 |  |  |  |  |
| Wetlabs Transmissometer |  | CST-1047DR |  |  |  |  |
| Optical Backscatter |  | 11074 |  |  |  |  |
| PAR sensor |  | 20273 |  |  |  |  |
| Altimeter |  | 41098 |  |  |  |  |
| Nitrate, ISUS |  | 121 |  |  |  |  |
| PDIM |  | 0113 |  |  |  |  |