March, 2008

Processing of CFMETR Nanoose Range CTD data

- Seabird processing:

- DatCnv used to convert .hex to .cnv files – channels:

        - Pressure, Strain Gauge [db]

        - Temperature (ITS-90, deg C]

        - Conductivity[S/m]

- Wild Edit - Outliers in the three channels are identified as data greater than 2 standard deviations from mean calculated over 30 scans (15 s at 2 Hz), two passes are made for each profile.

- Filter – A low-pass filter is applied to the pressure data (filter A has a 2.5 s (5 scans) time constant). A low-pass filter is applied to the conductivity (filter B has a 2.0 s (4 scans) time constant).

- Align CTD - Temperature readings are advanced by 0.6 s.

- Loop Edit - Descent rates less than 0.25 ms-1 are flagged.

- Bin Average - The downcast data have been averaged into 1 m intervals.

- IOS Shell processing:

- Conversion from Seabird ASCII files to IOS header format (.IOS ext).

- Salinity derived from pressure, temperature and conductivity (.der ext).

- Conductivity and flag channels removed leaving three channels (pressure, temperature and salinity(.trm ext).

- Matlab processing;

- Latitude and longitude for each cast calculated from ~.act file and written into CTD file.

- Plots generated showing location map, temperature and salinity profiles, and salinity and temperature contours (see below).

The 86 files have been given a .CTD extension and put in directory:

\\Paciosfp1\OSD_Data_Archive\Cruise_Data\2008\2008-99

CFMETR contact:	Terry.Berkley@forces.gc.ca	250-756-5004

Brian.Powell@forces.gc.ca	250-468-5080