File List for SeaBird Data          Cruise Number: _2009-59____

[osdcommon\cruise_data\2009-59\ARCHIVE]

[osdcommon\cruise_data\2009-59\ARCHIVE\Bottle] *.CHE

[osdcommon\cruise_data\2009-59\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data\2009-59\ARCHIVE\DOC] all documents from RAW and PROCESSING

[osdcommon\cruise_data\2009-59\ARCHIVE\DOC\HYDRO] COMPARE files and edited versions of    spreadsheets from analysts

[osdcommon\cruise_data\2009-59\PROCESSING]

[osdcommon\cruise_data\2009-59\PROCESSING\CONVERT]	*.CNV – after conversion

[osdcommon\cruise_data\2009-59\PROCESSING\DERIVE]*.CNV – after convert/wildedit/celltm/derive

[osdcommon\cruise_data\2009-59\PROCESSING\DOC]

2009-59-recal.ccf - calibration control file for recalibration of DO and Pressure

2009-59-ctd.con – configuration file used in processing

2009-59_file.doc - this document

2009-59_proc.doc - processing notes

2009-59-che-final.rpt – CTD file header check including channel summary

2009-59-ctd-final.rpt – CHE file header check including channel summary

2009-59-altimeter.csv – report on header entries for altimetry

*hdr.txt – header files with comments for CTD and CHE files

[osdcommon\cruise_data\2009-59\PROCESSING\DOC\hydro]

2009-59oxy.csv – nutrients simplified from analyst’s s/s

2009-59-dox-comp1.csv & .xls – initial comparison of titrated DO & CTD DO run early

2009-59-dox-comp2.csv & .xls – comparison of titrated DO and CTD DO after 1st recal

2009-59-dox-comp3.csv & .xls – comparison of titrated DO and downcast CTD DO

2009-59-altimeter-positions.csv – summary of header altimeter readings

addsamp.csv – spreadsheet relating bottle numbers to sample numbers

[osdcommon\cruise_data\2009-59\PROCESSING\HYDRO]

*.add – bottle from analyst after editing, formats fixed

*.bot  - *.ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.mrg – after addition of oxygen and salinity bottle data to BOT files

*.ros  - rosette files created by DATCNV, no other processing done.

*.sam – bottle files with sample numbers added

[osdcommon\cruise_data\2009-59\PROCESSING\IOS] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.cln - data after conversion to IOS HEADERS and CLEAN

*.ctd - final files

*.del – after DELETE

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.log - log files created by CTDEDIT

Delete.log - log from routine DELETE

______________________________________________________________________________

[osdcommon\cruise_data\2009-59\RAW]

[osdcommon\cruise_data\2009-59\RAW\2009-59-mooring]

[osdcommon\cruise_data\2009-59\RAW\Chemistry] data from analysts

[osdcommon\cruise_data\2009-59\RAW\CTD] raw CTD data

[osdcommon\cruise_data\2009-59\RAW\Main Lab 2009-59 ctd data]