## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |

## PROCESSING NOTES

Cruise: 2010-07

Agency: IOS, Ocean Sciences Division, Sidney, B.C

Location: Arctic Ocean's Canada Basin

Project: Joint Ocean Ice Study (JOIS including PACI and BGEP)

Party Chief: Williams, Bill.

Platform: Louis S. St. Laurent

Date: September 15 2010 – October 15 2010

Processed by: Initial by Sarah Zimmermann, subsequent processing by Joe Linguanti

Date of Processing: 27 March 2018 – 11 April 2018

Number of CTD files: 72 (casts 1-72)

Number of bottle files: 71 (casts 1-72, missing 46)

# CTD PROCESSING SUMMARY

Initial processing was completed by Sarah Zimmermann using standard Seabird processing steps and Matlab-based routines (see document 2010-07 CTD report Draft - MD 20101118.doc). Additional processing, for submission into the Ocean Sciences Division’s Data Archive, was completed by Joe Linguanti using IOSShell routines.

##### Preliminary Steps

The Log Book and rosette log sheets were obtained as well as the Cruise Report and draft Data Report.

Obtained additional information from Sarah on CTD data files (document 2010-07_CTD Data Files_2013-05-21.doc) and CTD Headers notes (document 2010-07 CTD Header Notes for Data Archive process v2018-03-27.docx).

##### Conversion to IOS Header format

After initial processing, 72 CNV files were exported from Matlab in standard Seabird format by Sarah.

The CNV files were converted to IOS Header format using the IOSShell routine, Convert Seabird ASCII Files. The converted files have extension name *.IOS.

A cross-reference list was produced using IOSShell routine, Cross Reference, to check that the date and location information was included for each file. In addition, a Cruise Summary report was produced using IOSShell routine, Header Check. This summary is quite useful for comparing casts for the entire cruise and verifying, among other information, the different kinds of instruments and sensors used during the cruise and for which casts.

The sensor information in the INSTRUMENT section of the header was checked and updated with the correct serial numbers and sensor names using the text editor UltraEdit.

- Reprocessing of PAR data

In preparing the files for the archive, Sarah discovered that the PAR data had the wrong calibration terms in the configuration file. Par data were only collected for a few casts and of those, only casts 1 and 2 had a signal. Sarah reprocessed the PAR data for these two casts with the correct terms from March 2007. However, Sarah did not run any of the post-Seabird processing steps on the rest of the properties in files. Therefore, the reprocessed PAR data needed to replace the erroneous PAR data in the previously processed files, which had the post-Seabird processing steps run on all the properties. The erroneous PAR data was removed from the original processed files and replaced with the reprocessed PAR data using the IOS Shell routine Merge.

The files with the reprocessed PAR data included have extension name *.PAR

##### Remove unwanted channels

IOSShell routine, Remove Channels, was run on the *.IOS files for all casts (with some exceptions) to remove the following channels:

Scan_Number [n/a], Depth:Salt_Water [metres], Temperature:Secondary [deg C (ITS90)],  Conductivity:Secondary [mS/cm], Oxygen:Voltage:SBE [volts], Nitrate_plus_Nitrite:ISUS [Volts] (except for casts 1, 2 and 5), PAR [uE/m^2/sec] (except for casts 1 and 2), Salinity:T1:C1 [PSS-78],  Temperature:Potential [deg C (ITS90)], Sigma:Theta [kg/m^3] and Speed:Sound [m/s].

The following files had additional channels removed or modified:

Cast 49. Fluorescence_CDOM:URU:Wetlabs, no data.

Cast 25. Nitrate_plus_Nitrite:ISUS, replaced with pad value after 600 dbar.

The files with channels removed have extension name *.REM.

##### Update Header

IOSShell routine, Header Edit, was used to fix formats, channel names and to add comments on processing, and collection and analysis methods to the *.REM files.

One channel name was changed:

- Fluorescence:URU:Seapoint changed to Fluorescence:Calibrated:Seapoint

Water samples showed a drift in the conductivity values over time therefore the conductivity data were calibrated in five groups of casts. For further changes and additions to the headers, see the five files “2010-07 CTD Header casts firstcast-lastcast.txt”, in the cruise .DOC directory for details. The difference between the files is the calibration values for conductivity.

The updated header files have extension name *.HED

- Reorder channels

IOSShell routine, Reorder, was run on the *.HED files to reorder the CTD channels in their traditional order.

The reordered files have extension name *.REO

- Remove Empty Channels

The IOSShell routine, Clean, was run on the .REO files with the “Remove Empty Channels” option selected.

The updated clean files have extension name *.CTD

Notes:

- Cast 25 - Nitrate data replaced with pad value after 600 dbar.

- Cast 49 - Fluorescence CDOM data removed. Data value pegged at maximum voltage of 4.95V. Cause unknown.

# CHEMISTRY PROCESSING SUMMARY

Sarah provided the chemistry spreadsheet file, “2010-07_LSSL_Chem_2018-04-04.xlsx”, for archiving.

##### Preliminary Steps

The worksheet, “2010-07_LSSL_CHEM”, in the spreadsheet file was prepared for conversion to IOS Header files.

- Transmissivity values are in units of %/25cm. An additional column was added with the values converted to units of %/meter using the formula %/meter = ((raw/100.0)^4)*100.0.

- ISUS Nitrate values were removed from all casts except 1, 2, 5 after 600dbars and 25 since instrument had been compromised.

- CTD CDOM values were removed from cast 49.

- Records at sample numbers 177, 234, 382 and 1545-1549 were removed since bottle either failed or sampling was skipped.

The modified chemistry spreadsheet file was renamed to “2010-07_LSSL_Chem_2018-04-11.xlsx. The worksheet, “2010-07_LSSL_CHEM”, was exported to CSV format to be used by the IOS Header conversion program.

##### Conversion to IOS Header format

The CSV file was converted to IOS Header format files using the IOSShell routine, Convert Spreadsheet Files. The converted files have extension name *.IOS2.

A cross-reference list was produced using IOSShell routine, Cross Reference, to check that the date and location information was included for each file.

- Reorder channels

IOSShell routine, Reorder, was used to reorder the channels so that the ISUS nitrate channel was placed before the bottle data and the Bottle Integrity Flag was place immediately following the Bottle Number.

The files have extension name *.REO

- Remove Empty Channels

The Spreadsheet Conversion programs exports all the water properties specified even if a property might not have been sampled for a particular cast. These are referred to as empty channels.  The IOSShell routine, Clean, was run on the .REO files with the “Remove Empty Channels” option selected.

The files with empty channels removed have extension name *.CLN.

- Sort on Pressure

The data records in the spreadsheet file were entered with pressure in ascending order (deepest to shallow). IOSShell routine, The Ocean Sciences Division’s Data Archive requires the pressure values to be descending order. The IOSShell routine, Sort, was used to sort the data records in the .CLN files with pressure descending.

The files have extension name *.SRT.

- Change alpha character flags

The quality flag columns in the spreadsheet contained the alpha character codes used pre 2013. I used program, Change_Flags (written in 2013), to change the alpha character flags in the .HED files to numeric values.

The files with numeric quality flags have extension name *.SRT

- Update Header

IOSShell routine, Header Edit, was used to add comments on processing, and collection and analysis methods, to the *.SRT files. See file “2010-07 CHE Header cast firstcast-lastcast.txt” in the cruise .DOC directory for details.

The updated header files have extension name *.CHE.

# MOVE FILES TO ARCHIVE

The CTD and chemistry files were copied to the Ocean Sciences Division’s digital on-line archive in folder Cruise_Data\2010\2010-07. The CTD files with extension names *.CTD and the chemistry files with extension names *.CHE were placed in the appropriate subfolders ‘CTD’ and ‘Bottles’ respectively.

All the documentation provided by Sarah and produced by Joe Linguanti was placed in the subfolder ‘DOC’.