File List for SeaBird Data     	Cruise Number: _2010-17____

[osdcommon\cruise_data_processing\2010-17\ARCHIVE]

[osdcommon\cruise_data_processing\2010-17\ARCHIVE\CTD]   *.CTD

[osdcommon\cruise_data_processing\2010-17\ARCHIVE\DOC] documents from RAW and PROCESSING

[osdcommon\cruise_data_processing\2010-17\processing\]

[osdcommon\cruise_data_processing\2010-17\ processing\convert]

*.cnv - data files after conversion using DATCNV, no other processing done.

[osdcommon\cruise_data_processing\2010-17\ processing\derive]

*.cnv - data after running WILDEDIT, WFILTER, CELLTM and DERIVE

[osdcommon\cruise_data_processing\2010-17\ processing\doc]

2010-17-ctd.con – configuration file used in processing

404 May19-2010-css.con – configuration file used in acquisition

2010-17-file.doc - this document

2010-17-proc.doc - processing notes

2010-17-hdr.txt – text files for addition of comments to headers

2010-17-ctd-10db.csv – thinned, bin-averaged upcast CTD data from ~10db

2010-17-bottle-data.xls – summary of sample data plus CTD data

2010-17-bottle-comparison.xls – comparison of CTD and bottle salinity

lat-long-depth.csv – spreadsheet used to add header info

2010-17-ctd-final.rpt – final header check report

[osdcommon\cruise_data_processing\2010-17\ processing\hydro]

*.avg – data from around 11.5m – one record per cast

*.revdel – upcast data

[osdcommon\cruise_data_processing\2010-17\ processing\ios]

*.atc – files after addition of positions and correction of time

*.avg – bin-averaged files

*.ctd - final files

*.del – data after DELETE – downcast data only

*.revdel – data after REVERSE and DELETE – upcast data only

*.edt - data after editing has been done

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT

*.mrh – files after addition of latitude, longitude, water depth to headers

Delete.log - log from routine DELETE

[osdcommon\cruise_data_processing\2010-17\ raw\]

Raw CTD data, chemistry data from analysts and cruise documents