Cruise 2010-36:  Ocean Acidification July 2010

CTD/Rosette:

Xx actual ‘casts’, but xx files, 0001 to 00xx.

0 stand-alone CTD casts.

xx ‘CTD’ casts with no sample taken.

All casts done with deck unit 0424, CTD 0506

All casts done with LARS (mid-ship station).

Seasave version 7.18c

GENERAL NOTES:

Problem casts:

Casts 0001/0002: same cast, computer crash.

Pressure spike on last record of cast 0001.

Cast 0006: Lost oxygen sensor at about 50 dbar on the upcast.

Casts 0030/0031: same cast, computer crash.  Forgot to turn pumps on for cast 0031.

Casts 0033/0034/0035: same cast, many computer crashes, some Niskins missing in .bl files.

Cast 0057: big shift in oxygen sensor data on downcast around 750 dbar.

Cast 0066: Niskin 1 not showing up in .bl file.

Casts 0089/0090/0091: same cast, computer crash.  Niskins 16 and 17 in two .bl files.

Casts 0094/0095: same cast, computer crash.  Niskin 17 in both .bl files.

Casts 0107/0108: same cast, computer crash.

Casts 0111/0113: same cast, computer crash (event 0112 was already taken by the Trace Metal Rosette).

Casts 0003, 0015, 0020: spike in Temp and Oxy.

Cast 0088: Oxygen sensor starting to get really low in OMZ

Out of order (User Input) rosette casts:

xx casts were user input casts: 0004, 0006, 0012, 0017, 0023, 0036, 0041, 0044, 0047, 0049, 0060, 0064, 0066, 0068, 0072, 0075, 0076, 0077, 0078, 0081, 0085, 0089/90/91, 0094/95, 0097, 0100, 0101, 104, 105, 107/108, 111/113, 0117, 0121, 0124, 0127, 0129, 0135, 0137, 0138, 0139, 0140

Headers:

Cast 0076: wrong depth in header.

PAR:

Casts with PAR sensor 4694:  0001, 0002, 0003, 0004, 0006, 0007, 0008, 0010, 0012, 0014, 0015, 0021, 0029, 0038, 0041, 0042, 0043, 0044, 0045, 0047, 0056, 0064, 0068, 0070, 0072, 0074, 0075, 0076, 0077, 0078, 0079, 0080, 0081, 0083, 0097, 0100,

pH:

Casts with pH sensor on: 0001, 0002, 0003, 0004, 0006, 0007, 0008, 0010, 0012, 0014, 0015, 0017, 0038, 0041, 0042, 0043, 0044, 0045, 0047, 0068, 0070, 0072, 0074, 0075, 0076, 0077, 0078, 0079, 0080, 0081, 0094/0095, 0097, 0100,

Niskins at CTD/Rosette casts:

Casts 0001/2: all Niskins closed at the surface just to checked, none sampled.

Cast 0139: Niskins 19 to 24 closed at the surface just to close then, not sampled, don’t need to be in CHE file.

Loops:

Loop taken for Chl, Nut, Sal at QCS1 (0006), AQ1 (0080),

TSG: