DATA ANALYSIS COMMENTS

Data quality is expressed with the following flags

----------------------------------------------------------------------------

1 = Sample for this measurement was drawn from water bottle but not analyzed

(not normally used).

2 = Acceptable measurement (not normally used).

3 = Questionable measurement (no problem observed in sampling or analysis,

but value is not trusted, nonetheless; includes outlyers).

4 = Bad measurement (known problem with sampling or analysis, but not

serious enough to completely discard the value).

5 = Not reported (lost sample; unredeemably bad measurement).

6 = Mean of replicate measurements.

7 = Manual chromatographic peak measurement.

8 = Irregular digital chromatographic peak integration.

9 = Sample not drawn for this measurement from this bottle (not normally

used).

----------------------------------------------------------------------------

Analysis methods:

-----------------

Chlorophyll samples were filtered onto 25mm GF/F filters and stored in glass scintillation vials at -20C prior to analysis. Samples were extracted in 90% acetone for 24 hours and analyzed in the lab on a Turner 10AU fluorometer which is calibrated annually with commercially pure chlorophyll a (Sigma). Fluorescence readings taken before and after acidification were used to calculate chlorophyll and phaeopigment concentrations (Holm-Hansen 1965). For full results, see file QF2011-45Nuts+Chl*.xls including a discussion of major analysis problems.

NOTE: Phaeopigment data are provided for information only and no quality flags are assigned to them.

Salinity samples were collected in glass bottles and analyzed on Guildline Autosal model 8400B, S/N 68572, at IOS. Salinometers are standardized with IAPSO standard seawater. Full results are in file QF2011-45SAL*.xls.

Nutrient samples were collected in plastic tubes and stored frozen and later analysed at IOS using an Astoria analyzer following methods described in Barwell-Clarke and Whitney (1996). File QF2011-45Nuts+Chll*.xls." includes the raw data and an analysis of duplicates.

References:

1. Barwell-Clarke, J. and Whitney, F. 1996. Institute of Ocean Sciences

Nutrient Methods and Analysis. Canadian Technical Report of Hydrography

and Ocean Sciences, No. 182, 43 pp.

2. Holm-Hansen, O., Lorenzen, C.J., Holmes, R.W., and Strickland J.D.H. 1965.

Fluorometric Determination of Chlorophyll. J.du Cons. Intl. Pour l’Epl.

De la Mer. 30:3-15.