File List for SeaBird Data          Cruise Number: _2012-01____

[osdcommon\cruise_data\2012-01\ARCHIVE]

[osdcommon\cruise_data\2012-01\ARCHIVE\Bottle] *.CHE

[osdcommon\cruise_data\2012-01\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data\2012-01\ARCHIVE\DOC] most documents from RAW and PROCESSING

[osdcommon\cruise_data\2012-01\ARCHIVE\DOC\HYDRO] COMPARE files

[osdcommon\cruise_data\2012-01\ARCHIVE\DOC\TSG] all TSG documents

[osdcommon\cruise_data\2012-01\ARCHIVE\LOOP] loop file

[osdcommon\cruise_data\2012-01\ARCHIVE\TSG] *.TOB

[osdcommon\cruise_data\2012-01\PROCESSING]

[osdcommon\cruise_data\2012-01\PROCESSING\Angelica] files for use of A. Peña

[osdcommon\cruise_data\2012-01\PROCESSING\CONVERT]	*.CNV – after conversion

[osdcommon\cruise_data\2012-01\PROCESSING\DERIVE]*.CNV – after convert/wildedit/celltm/derive

[osdcommon\cruise_data\2012-01\PROCESSING\DO test] initial calibration of SBE:DO

[osdcommon\cruise_data\2012-01\PROCESSING\DOC]

2012-01-recal*.ccf – calibration control files for all CTD and CHE files

2012-01-ctd*.xmlcon – configuration files used in processing before DO tests

2012-01-ctd*-new.xmlcon – configuration files with updated DO paramters

2012-01_file.doc - this document

2012-01_proc.doc - processing notes

2012-01-ctd-final.rpt – CHE file header check including channel summary

2012-01-ctd-altimeter.csv – report on header entries for altimetry

*hdr.txt – header files with comments for TSG, CTD and CHE files

[osdcommon\cruise_data\2012-01\PROCESSING\DOC\hydro]

2012-01-chl.csv – extracted chlorophyll data simplified from analyst’s s/s

2012-01-nuts.csv – nutrients simplified from analyst’s s/s

2012-01-oxy.csv – nutrients simplified from analyst’s s/s

2012-01-sal.csv – salinity data from Autosal, simplified from analyst’s s/s

2012-01-fl-chl-comp1.csv & .xls – comparison of extracted CHL and CTD Fluorescence

2012-01-do-cal.xls – initial calibration data and plot

2012-01-dox-comp1.csv & .xls – comparison of titrated DO & CTD DO after initial calib.

2012-01-dox-comp2.csv & .xls – comparison of titrated DO & downcast CTD DO

2012-01-sal-comp1.csv & xls –comparison of rosette bottle and CTD salinity

addsamp.csv – spreadsheet relating bottle numbers to sample numbers

2012-01-che-altimeter-bot.csv – report on header entries for altimetry from bottle files

2012-01-che-final.rpt – CTD file header check including channel summary

2012-01-bottles-final.xls – summary of bottle data in CHE files

[osdcommon\cruise_data\2012-01\PROCESSING\DOC\tsg]

2012-01-tsg-recal1.ccf – calibration control file for recalibration of all TSG files

2012-01-missing-tsg-posns.csv – ship position information to fix file #1

2012-01-rosette-tsg-comp.xls – comparison of TSG data to rosette bottle data

2012-01-CTD-TSG-comp.xls – comparison of CTD rosette data and TSG data

2012-01-tsg-loop-comp.xls –comparison of TSG with loop data.

2012-01-tsg.con – configuration file for conversion of TSG HEX files.

[osdcommon\cruise_data\2012-01\PROCESSING\HYDRO]

*.bot -*.ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.chl – extracted chlorophyll data converted from simplified version of spreadsheet

*.ed1 – editing output for 2 files

*.log – log files for the editing of 2 files

*.mrg – after addition of oxygen and salinity bottle data to BOT files

*.nut – nutrient files converted from simplified version of analyst’s spreadsheet

*.oxy – bottle from analyst after editing, formats fixed

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted from simplified version of analyst’s spreadsheet

*.sam – bottle files with sample numbers added

[osdcommon\cruise_data\2012-01\PROCESSING\IOS] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.cln - data after conversion to IOS HEADERS and a run of CLEAN

*.ctd - final files

*.del – after DELETE

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios – data after conversion to IOS SHELL format

*.log* - log files created by CTDEDIT

Delete.log - log from routine DELETE

[osdcommon\cruise_data\2012-01\PROCESSING\Loop] 5m rosette plus loop data

2012-01-che-surface.csv – file from Chief Scientist with 5m rosette data

2012-01-surface-loop.csv – combination of loop and rosette data

2012-01-loop-6linehdr.csv – above adapted for conversion to IOS Header format

2012-01-0001.ios – IOS Header version

2012-01-surface.loop – final loop file

[osdcommon\cruise_data\2012-01\PROCESSING\TSG] before conversion to IOS SHELL

*.cnv – files after Sea-Bird conversion

[osdcommon\cruise_data\2012-01\PROCESSING\TSG\IOS] after conversion to IOS SHELL

*.atc – after adding positions and times to headers and time/date channel

*.edt– after editing using CTDEDIT

*.ios – after conversion to IOS HEADERS

*.log – log files from editing

*.tob – final TSG files

______________________________________________________________________________

[osdcommon\cruise_data\2012-01\RAW]

[osdcommon\cruise_data\2012-01\RAW\Chemistry] data from analysts

[osdcommon\cruise_data\2012-01\RAW\CTD] raw CTD data

[osdcommon\cruise_data\2012-01\RAW\Documents] cruise documents

[osdcommon\cruise_data\2012-01\RAW\RMC] ship’s positions for part of cruise

[osdcommon\cruise_data\2012-01\RAW\TSG files] raw TSG data