File List for SeaBird Data          Cruise Number: _2012-12____

[osdcommon\cruise_data\2012-12\ARCHIVE]

[osdcommon\cruise_data\2012-12\ARCHIVE\Bottle] *.CHE

[osdcommon\cruise_data\2012-12\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data\2012-12\ARCHIVE\DOC] most documents from RAW and PROCESSING

[osdcommon\cruise_data\2012-12\ARCHIVE\DOC\HYDRO] COMPARE files

[osdcommon\cruise_data\2012-12\ARCHIVE\DOC\TSG] all TSG documents

[osdcommon\cruise_data\2012-12\ARCHIVE\LOOP] loop file

[osdcommon\cruise_data\2012-12\ARCHIVE\TSG] *.TOB

[osdcommon\cruise_data\2012-12\PROCESSING]

[osdcommon\cruise_data\2012-12\PROCESSING\Angelica] files for use of A. Peña

[osdcommon\cruise_data\2012-12\PROCESSING\CONVERT]	*.CNV – after conversion

[osdcommon\cruise_data\2012-12\PROCESSING\DERIVE]*.CNV – after convert/wildedit/celltm/derive

[osdcommon\cruise_data\2012-12\PROCESSING\DO test] initial calibration of SBE:DO

*.bot, *.ros, *.samavg, and 2012-DO-cal-study.xls

[osdcommon\cruise_data\2012-12\PROCESSING\DOC]

2012-12-recal1.ccf – calibration control file for all CTD and CHE files

2012-12-ctd.con – configuration file used in converting files for DO study

2012-12-ctd-new.con – updated configuration file used in converting files

2012-12_file.doc - this document

2012-12_proc.doc - processing notes

2012-12-ctd-final.rpt – CHE file header check including channel summary

2012-12-altimeter-ctd.csv – report on header entries for altimetry

*hdr.txt – header files with comments for TSG, CTD and CHE files

[osdcommon\cruise_data\2012-12\PROCESSING\DOC\hydro]

2012-12-che-altimeter.csv – report on header entries for altimetry from bottle files

2012-12_chl.csv – extracted chlorophyll data simplified from analyst’s s/s

2012-12dms.csv – DMS data simplified from analyst’s file

2012-12nuts.csv – nutrients simplified from analyst’s s/s

2012-12oxy.csv – nutrients simplified from analyst’s s/s

2012-12-sal.csv – salinity data from Autosal, simplified from analyst’s s/s

2012-12-Dual-fl-chl-comp1.csv & .xls – comparison of extracted CHL and CTD Fluorescence

2012-12-do-cal.xls – initial calibration data and plot

2012-12-dox-comp1.csv & .xls – comparison of titrated DO & CTD DO after initial calib.

2012-12-dox-comp2.csv & .xls – comparison of titrated DO & downcast CTD DO

2012-12-sal-comp1.csv & xls –comparison of rosette bottle and CTD salinity

2012-12-sal-comp2.csv & xls –comparison of rosette bottle and CTD salinity after recal

addsamp.csv – spreadsheet relating bottle numbers to sample numbers

2012-12-bottles-final.xls – summary of bottle data in CHE files

2012-12-che-final.rpt – header check of final CHE files

2012-12-bot-hdr.txt – header comments file

[osdcommon\cruise_data\2012-12\PROCESSING\DOC\tsg]

2012-12-tsg-recal1.ccf – calibration control file for recalibration of all TSG files

2012-12-tsg-final.rpt – header check on TOB files

2012-12-CTD-TSG-comp.xls – comparison of CTD rosette data and TSG data

2012-12-tsg-loop-rosette-comp.xls –comparison of TSG with loop data.

2012-12-tsg*.xmlcon – configuration files for conversion of TSG HEX files.

2012-12-hdr-tsg*.txt – header comment files

[osdcommon\cruise_data\2012-12\PROCESSING\FIX] files used to fix some errors in TR/FL

[osdcommon\cruise_data\2012-12\PROCESSING\HYDRO]

*.bot -*.ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.chl – extracted chlorophyll data converted from simplified version of spreadsheet

*.dms – DMS data converted from simplified version of spreadsheet

*.ed1 – editing output for 2 files

*.log  – log files for the editing of 2 files

*.mrg – after addition of oxygen and salinity bottle data to BOT files

*.nut – nutrient files converted from simplified version of analyst’s spreadsheet

*.oxy – bottle from analyst after editing, formats fixed

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted from simplified version of analyst’s spreadsheet

*.sam – bottle files with sample numbers added

[osdcommon\cruise_data\2012-12\PROCESSING\IOS] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.cln - data after conversion to IOS HEADERS and a run of CLEAN

*.ctd - final files

*.del – after DELETE

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios – data after conversion to IOS SHELL format

*.log* - log files created by CTDEDIT

Delete.log - log from routine DELETE

[osdcommon\cruise_data\2012-12\PROCESSING\Loop] 5m rosette plus loop data

2012-12-che-surface-loops.csv – file from Chief Scientist

2012-12-che-surface-loops-6linehdr.csv – above adapted for conversion to IOS Header format

2012-12-loop-hdr-comments.txt – header comments for the final loop file

2012-12-surface.loop – IOS Header version

*.cln*, *.ios*,*.atc*, *.mrg – used to add 2nd DO bottle column to previous version of loop file

[osdcommon\cruise_data\2012-12\PROCESSING\TSG] before conversion to IOS SHELL

*.cnv – files after Sea-Bird conversion

[osdcommon\cruise_data\2012-12\PROCESSING\TSG\IOS] after conversion to IOS SHELL

*.atc – after adding positions and times to headers and time/date channel

*.edt– after editing using CTDEDIT

*.ios – after conversion to IOS HEADERS

*.log – log files from editing

*.tob – final TSG files

______________________________________________________________________________

[osdcommon\cruise_data\2012-12\RAW]

[osdcommon\cruise_data\2012-12\RAW\Chemistry] data from analysts

[osdcommon\cruise_data\2012-12\RAW\CTD] raw CTD data

[osdcommon\cruise_data\2012-12\RAW\Files] cruise documents

[osdcommon\cruise_data\2012-12\RAW\Loop] data for loop file

[osdcommon\cruise_data\2012-12\RAW\TSG files] raw TSG data