File List for SeaBird Data          Cruise Number: _2012-50____

[osdcommon\cruise_data\2012-50\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data\2012-50\ARCHIVE\DOC] most documents from RAW and PROCESSING

____________________________________________________________________________

[osdcommon\cruise_data\2012-50\PROCESSING\CONVERT]	*.CNV – after conversion

[osdcommon\cruise_data\2012-50\PROCESSING\DERIVE]*.CNV – after convert/wildedit/align/celltm/derive

[osdcommon\cruise_data\2012-50\PROCESSING\DOC]

2012-50-ctd.SBE911-nopH.xmlcon – for conversion without pH sensor, old DO calibration

2012-50-ctd.SBE911-pH.xmlcon – for conversion with pH sensor, old DO calibration

2012-50-ctd.SBE911-nopH-new.xmlcon – for conversion without pH sensor, updated DO calibration

2012-50-ctd.SBE911-pH.xmlcon – for conversion with pH sensor, updated DO calibration

2012-50-ctd.SBE25.xmlcon – used to convert all SBE25 files

2012-50_file.doc - this document

2012-50_proc.doc - processing notes

2012-50-ctd-final.rpt –file header check including channel summary

2012-50-bottles_plus_CTD_data.xls – upcast bottle data with downcast CTD data from same level

2012-50-position3.csv – summary of information to be added to the headers

2012-50-ctd-bottle-comparisons.xls – comparison of CHL and salinity samples with CTD sensor data

2012-50-hdr*.txt – header comment files

2012-50-xref.lis – list of all CTD casts with positions and times

2012-50-sBE25-recal1.ccf – file to add small offset to pressure channel

2012-50-sBE911-recal1.ccf – file to add offset to DO concentration channel

[osdcommon\cruise_data\2012-50\PROCESSING\IOS\SBE25] after conversion to IOS SHELL

*.ctd – final files

*.del – Delete files from downcast

*.edt - data after editing has been done

*.ios - data after conversion to IOS HEADERS

*.mrh – after addition of positions and water depths

*.rev – reversed files after conductivity SHIFT run

*.revdel – Delete files from upcast

Del.log – log from routine DELETE – downcasts

Delrev.log – log from routine DELETE - upcasts

[osdcommon\cruise_data\2012-50\PROCESSING\IOS\SBE911] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.cln – after adding event numbers to IOS

*.ctd - final files

*.del– after DELETE

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios - data after conversion to IOS HEADERS

*.rev – reversed files after conductivity SHIFT run for file #337

*.log - log files created by CTDEDIT.

Delete.log - log from routine DELETE

Delrev.log – log from routine DELETE - upcasts

[osdcommon\cruise_data\2012-50\PROCESSING\pH] after conversion to IOS SHELL

*.ctdpH – like *.ctd but contains SBE pH sensor channel – not to go in IOS DATA ARCHIVE

2012-50-xref.lis – list of ctdpH files with times and positions

____________________________________________________________________________

[osdcommon\cruise_data\2012-50\RAW]

[osdcommon\cruise_data\2012-50\RAW\CTD] raw CTD data

[osdcommon\cruise_data\2012-50\RAW\Chemistry] bottle data spreadsheets from analysts