File List for SeaBird Data          Cruise Number: _2013-43____

[osdcommon\cruise_data\2013-43\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data\2013-43\ARCHIVE\DOC] most documents from PROCESSING\DOC folder

[osdcommon\cruise_data\2013-43\ARCHIVE\DOC\HYDRO] hydro bottle analyses + CTD data

[osdcommon\cruise_data\2013-43\PROCESSING\CONVERT]	*.CNV – after conversion

[osdcommon\cruise_data\2013-43\PROCESSING\DERIVE] *.CNV – after convert/wildedit/celltm/derive

[osdcommon\cruise_data\2013-43\PROCESSING\DOC]

2013-43-ctd.xmlcon – configuration file with pre-cruise pressure calibration

2013-43-ctd-post.xlmcon – configuration file used in processing with post-cruise pressure calibration

2013-43_file.doc - this document

2013-43_Processing_Report.doc - processing notes

2013-43 station info.xlsx – summary of station information provided by SAFE

2013-43-depths.csv – file with positions and depths in format for addition to headers

2013-43-ctd-final.rpt – CHE file header check including channel summary

2013-43-hdr.txt – header comment file

2013-43-xref.lis – list of all processed CTD casts with positions and times

2013-43-recal1.ccf – calibration control file for correction of DO

[osdcommon\cruise_data\2013-43\PROCESSING\DOC\hydro]

2013-43-bottles_plus_CTD_data.xlsx – combined bottle and CTD data including FL/CHL comparison

2013-43-dncast-*m-ctd.csv –downcast CTD data from 9 or 10m, as appropriate

2013-43-upcast-*m-ctd.csv –upcast CTD data from 9 or 10m, as appropriate

[osdcommon\cruise_data\2013-43\PROCESSING\IOS] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.cln – after adding event numbers to IOS

*.ctd - final files

*.del  and delrev– after DELETE on downcast and reversed upcast files, respectively

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios - data after conversion to IOS HEADERS

*.log - log files created by CTDEDIT.

*.mrh – after addition of depths and positions to CLN files

*.rev – reverse of shifted files (Before Delete)

Delete.log - log from routine DELETE on downcast files

Deleterev.log – log from routine DELETE on reversed upcast files

__________________________________________________________________________

[osdcommon\cruise_data\2013-43\RAW]

[osdcommon\cruise_data\2013-43\RAW\Chemistry] data from analysts

[osdcommon\cruise_data\2013-43\RAW\CTD] raw CTD data