File List for SeaBird Data

Cruise Number: _2018-098____

[osdcommon\cruise_data\2018-098\ARCHIVE]

[osdcommon\cruise_data\2018-098\ARCHIVE\Bottle] *.CHE

[osdcommon\cruise_data\2018-098\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data\2018-098\ARCHIVE\DOC] most documents from RAW and PROCESSING

[osdcommon\cruise_data\2018-098\ARCHIVE\DOC\HYDRO] COMPARE files and reports

[osdcommon\cruise_data\2018-098\ARCHIVE\LOGS] digital or scanned logs

---------------------------------------------------------------------------------------------------------------------------------------

[osdcommon\cruise_data\2018-098\PROCESSING]

[osdcommon\cruise_data\2018-098\PROCESSING\CONVERT]	*.CNV – after conversion with wrong DO sensor including 3 with post-cruise calibrations

[osdcommon\cruise_data\2018-098\PROCESSING\DO-CONVERT] *.CNV – after conversion with corrected

DO sensor parameters and including 3 with post-cruise calibrations

[osdcommon\cruise_data\2018-098\PROCESSING\DERIVE]*.CNV – after derive with wrong DO sensor

parameters

[osdcommon\cruise_data\2018-098\PROCESSING\DO-DERIVE]*.CNV – after derive with corrected DO sensor

parameters and including 3 with post-cruise calibrations

[osdcommon\cruise_data\2018-098\PROCESSING\DOC]

2018-098-recal1.ccf – calibration control file

2018-098-ctd-final.rpt – CTD file header check including channel summary

2018-098-ctd-xref.lis – cross reference list of CTD files

2018-098-altimeter-ctd.xls – report on header entries for altimetry

2018-098_File.doc - this document

2018-098_Processing_Report.doc - processing notes

2018-098-ctd.xmlcon – configuration file with correct DO sensor

2018-098-ctd-DO1202.xmlcon – configuration file with wrong DO sensor

2018-098-post.xmlcon – configuration file with post-cruise cals and correct DO sensor.

2018-098Bridge Log-CTD-casts.csv – event log

*.xml – calibration data for temp, cond and DO sensors, pre-cruise and post-cruise

*hdr.txt – header file with comments for CTD files

[osdcommon\cruise_data\2018-098\PROCESSING\DOC\hydro]

2018-098sal.csv – salinity data from Autosal, simplified from analyst’s s/s

2018-098Chl.csv –extracted chlorophyll data reorganized from analyst’s s/s

2018-098-sal-comp1.csv & xlsx –initial comparison of rosette bottle and CTD salinity

2018-098-fl-chl-comp1.csv & xlsx –initial comparison of rosette bottle and CTD salinity

2018-098-che-final.rpt – final header check of bottle files

2018-098-che-xref.lis – cross-reference listing of bottle files

2018-098-bottles-final.xlsx – summary of bottle data in CHE files

addsamp.csv – spreadsheet relating bottle numbers to sample numbers

*hdr.txt – header file with comments for CHE files

[osdcommon\cruise_data\2018-098\PROCESSING\HYDRO]

*.bot - ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.chl – extracted CHL files

*.ed1 & *.log – output of CTDEDIT for 4 files; ED1 copied to BOT files

*.mrg – after addition of bottle data to BOT files

*.mrgadd – after addition of bottle integrity flag channel

*.mrgrem – last step before Change Units for DO

*.mrgreo – just before final head edits – use if need to adjust comments/formats

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted from simplified version of analyst’s spreadsheet

*.sam – bottle files with sample numbers added

[osdcommon\cruise_data\2018-098\PROCESSING\IOS] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.clip – after removal of data from soak period

*.clipdo – after removal of data from soak period from CLNDO files

*.cln - data after conversion to IOS HEADERS and a run of CLEAN with wrong DO sensor

*.clndo - data after conversion to IOS HEADERS and a run of CLEAN with correct DO sensor

*.ctd-  final files

*.del – after DELETE with wrong DO sensor

*.deldo – after DELETE with correct DO sensor

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.edtcln - *.EDT with seabird comments removed

*.ios – data after conversion to IOS SHELL format

*.log - log files created by CTDEDIT

*.mrg – merge of EDTCLN and DELDO to get correct DO in edited files

*.rem – after REMOVE (just before Header Edit for SBE25 files)

*.reo– just before final Header Edit – use if need to update comments

Delete.log - log from routine DELETE

---------------------------------------------------------------------------------------------------------------------------------------

[osdcommon\cruise_data\2018-098\RAW]

[osdcommon\cruise_data\2018-098\RAW\Chemistry] salinity and CHL data from analysts

[osdcommon\cruise_data\2018-098\RAW\CTD] raw CTD data

[osdcommon\cruise_data\2018-098\RAW\DOC] documents

[osdcommon\cruise_data\2018-098\RAW\LOGS] logs