- File List for SeaBird Data          		Cruise Number: _2020-007____

[osdcommon\cruise_data_processing\2020-007\ARCHIVE]

[osdcommon\cruise_data_processing\2020-007\ARCHIVE\Bottle] *.BOT

[osdcommon\cruise_data_processing\2020-007\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data_processing\2020-007\ARCHIVE\DOC] general documents

[osdcommon\cruise_data_processing\2020-007\ARCHIVE\DOC\Hydro] documents re BOT files

[osdcommon\cruise_data_processing\2020-007\ARCHIVE\LOGS]

[osdcommon\cruise_data_processing\2020-007\PROCESSING]

[osdcommon\cruise_data_processing\2020-007\PROCESSING\CONVERT] *.CNV – after conversion

[osdcommon\cruise_data_processing\2020-007\PROCESSING\DERIVE]

*.CNV – after conversion, wfilter, align, celltm, derive

[osdcommon\cruise_data_processing\2020-007\PROCESSING\DOC]

2020-007_Processing_Report.doc - processing notes

- 2020-007_File.doc - this document

- 2020-007-ctd.xmlcon – configuration file

- 2020-007-ctd-final.rpt – CTD file header check including channel summary

- 2020-007-ctd-xref.lis – cross-reference list of casts

- 2020-007-SBE25-hdr.txt –header file with comments for CTD files

[osdcommon\cruise_data_processing\2020-007\PROCESSING\DOC\Hydro]

2020-007-bottles_plus_CTD_6linehdr.csv – bottle data + CTD data with 6-line header

2020-007-bottles_CTD_comparison.xlsx – CTD and bottle data and comparisons

2020-007-bottles-final.csv – export of BOT file contents

2020-007-bottles-final.rpt – final header check of bottle files

2020-007-bot-xref.txt – cross-reference listing of bottle files

- 2020-007-bot-hdr.txt – header file with comments for BOT files

- [osdcommon\cruise_data_processing\2020-007\PROCESSING\hydro]

*.bot –Final files

*.ios – files converted from spreadsheet

*.reo – after removing unnecessary channels and reorder

*.sort – 2nd last step to order on pressure

- [osdcommon\cruise_data_processing\2020-007\PROCESSING\IOS]

- *.atc - 	after correction of time to UTC

- *.avg & avgrev- data after metre-averaging using BINAVE (downcast & upcast)

- *.clip - after removing soak data

- *.cln - after fixing header errors

*.ctd - final files

*.del & delrev – after DELETE

*.edt & *.log – output from graphical editing

*.ios - data after conversion to IOS HEADERS

*.reo – last step before Head Edit

*.rev – reversed files

Delete.log - log from routine DELETE