File List for SeaBird Data          Cruise Number: _2020-008____

[osdcommon\cruise_data\2020-008\ARCHIVE]

[osdcommon\cruise_data\2020-008\ARCHIVE\Bottle] *.CHE

[osdcommon\cruise_data\2020-008\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data\2020-008\ARCHIVE\CTD Pictures] *.CTD

[osdcommon\cruise_data\2020-008\ARCHIVE\DOC] most documents from RAW and PROCESSING

[osdcommon\cruise_data\2020-008\ARCHIVE\DOC\HYDRO] COMPARE files

[osdcommon\cruise_data\2020-008\ARCHIVE\DOC\TSG] TSG documents

[osdcommon\cruise_data\2020-008\ARCHIVE\LOGS] any scanned or S/S logs

[osdcommon\cruise_data\2020-008\ARCHIVE\Underway] TSG and Loop data

[osdcommon\cruise_data\2020-008\PROCESSING]

[osdcommon\cruise_data\2020-008\PROCESSING\CONVERT]	*.CNV – after conversion

[osdcommon\cruise_data\2020-008\PROCESSING\DERIVE]*.CNV – after convert/wildedit/celltm/derive

[osdcommon\cruise_data\2020-008\PROCESSING\DOC]

2020-008-recal1.ccf – calibration control files

2020-008-post.xmlcon – calibration parameters from December 2020

2020-008_File.doc - this document

2020-008_Processing_Report.doc - processing notes

2020-008 Sampling notes.doc – notes from chief scientist

2020-008-ctd*.xmlcon – configuration files used in processing

2020-008-ctd-final.rpt – CTD file header check including channel summary

2020-008-ctd-xref.lis – cross reference list of CTD files

2020-008-altimeter-ctd.csv – report on header entries for altimetry

*hdr.txt – header file with comments for CTD files

[osdcommon\cruise_data\2020-008\PROCESSING\DOC\hydro]

2020-008chl.csv – extracted chlorophyll data simplified from analyst’s s/s

2020-008DMS.csv – DMS data simplified from analyst’s s/s

2020-008DMSP.csv – DMSP data simplified from analyst’s s/s

2020-008nuts.csv – nutrients simplified from analyst’s s/s

2020-008oxy.csv – dissolved oxygen data simplified from analyst’s s/s

2020-008sal.csv – salinity data from Autosal, simplified from analyst’s s/s

2020-008-fl-chl-comp1.csv & .xlsx – comparison of extracted CHL and CTD Fluorometer

2020-008-dox-comp1.csv & .xlsx – initial comparison of titrated DO & CTD DO

2020-008-dox-comp2.csv & .xlsx –comparison of titrated DO & CTD DO after initial recal

2020-008-dox-comp3.csv & .xlsx – comparison of titrated DO & downcast CTD DO

2020-008-sal-comp1.csv & xlsx –initial comparison of rosette bottle and CTD salinity

2020-008-sal-comp2.csv & xlsx –comparison of rosette bottle and CTD after initial recal

addsamp.csv – spreadsheet relating bottle numbers to sample numbers

2020-008-bottles-final.xslx – summary of bottle data in CHE files

2020-008-altimeter-che.xlsx – altimetry and water depth information

2020-008-che-final.rpt – final header check of bottle files

2020-008-che-xref.lis – cross-reference listing of bottle files

*hdr.txt – header file with comments for CHE files

[osdcommon\cruise_data\2020-008\PROCESSING\DOC\tsg]

2020-008-ctd-tsg-loop-rosette-comp.xlsx – comparisons of TSG data, loop, CTD, rosette

2020-0080-hdr-tsg.txt – header with with comments for TOB files.

2020-008-tsg_xref.txt – summary of starting times and positions for TOB files

2020-008-tsg-recal1.ccf – recalibration file for TSG files

[osdcommon\cruise_data\2020-008\PROCESSING\HYDRO]

*.bot - ros files converted to IOS HEADERS + added event numbers

*.che – final bottle-averaged rosette files merged with chemistry data

*.chl – extracted chlorophyll data converted from simplified version of spreadsheet

*.dms – DMS files converted from simplified version of analyst’s spreadsheet

*.dmsp – DMSP files converted from simplified version of analyst’s spreadsheet

*.mrg – after addition of oxygen and salinity bottle data to BOT files

*.mrgrem – after removal of unneeded channels

*.mrgreo – just before final step running Head Edit

*.nut – nutrient files converted from simplified version of analyst’s spreadsheet

*.oxy – dissolved oxygen files converted from simplified version of analyst’s spreadsheet

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted from simplified version of analyst’s spreadsheet

*.sam – bottle files with sample numbers added.

[osdcommon\cruise_data\2020-008\PROCESSING\IOS] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.cln - data after conversion to IOS HEADERS and a run of CLEAN

*.ctd - final files

*.del – after DELETE

*.edt - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios – data after conversion to IOS SHELL format

*.log - log files created by CTDEDIT

*.rem – after removing some channels

*.reo – files just before header edit

Delete.log - log from routine DELETE

[osdcommon\cruise_data\2020-008\PROCESSING\TSG] before conversion to IOS SHELL

*.csv – spreadsheet versions of raw files

*Raw – original files

[osdcommon\cruise_data\2020-008\PROCESSING\Loop] before conversion to IOS SHELL

2020-008*.atc – che files with sigma-T derived, clipped to 7m, End Time added

2020-008-surface.loop – final file in IOS Header format with comments

2020-008-che-surface.csv – surface data from CHE files including CTD & bottle data.

2020-008-surface-6linehdr.csv – as above but with 6-line headers and loop data added

2020-008-0001.ios –6linehdr csv file converted to IOS Header format

2020-008-loop-hdr-comments.txt – header comments for the final loop file

[osdcommon\cruise_data\2020-008\PROCESSING\TSG\IOS] after conversion to IOS SHELL

*.atc – after adding Julian time

*.avg – files averaged over 6 records for comparison with CTD data

*.cln – after adding positions and times to headers and time/date channels

*.cor – after recalibration

*.dqt2 – after recal & derivation of salinity and then Sigma-T

*.edt & *.log–after CTDEDIT

*.ios – after conversion to IOS HEADERS

*.rem – after removing fluorescence, pressure, T difference and record #

*.reo –TSG files after REORDER – use if want to create TOB files later