Sampling notes, Line P cruise, 2020-028, 16 to 22 June 2020

CTD/Rosettes:

73 files, 0001 to 0106.

0 stand-alone CTD.

27 rosette casts.

46 cast with no needed Niskin

1 cast needed two files

0 file not needed

CTD 0443 used for the whole cruise.

Seasave version 7.26.7.121

Configuration file:

The config file 2020-028start.xmlcon was used for casts 0001 to 0028.

Config file 2020-028start-newPoffset.xmlcon was used for the rest of the cruise, with a different pressure offset.

Standard procedure:

The rosette was brought to the surface.  Pumps were turned ON.  The rosette was brought down to 10m and kept there for 30 seconds.  Once back at the surface, the data started to be archived, with the rosette at the surface for 30 seconds longer.  Then the cast would start.

Problems with CTD:

Casts 0001 to 0006: problem with primary pump.  It got replaced after cast 0006.

Casts 0001 to 0028: wrong pressure offset in config file.

Casts 0001 to 0014 and 0086 to 0106 done with Computer 102; casts 0016 to 0086 done with old Tully laptop.

Cast 0013: the computer rebooted itself during the cast.  Last bottle is in file 0014.

Cast 0063: switch computer from local to UTC time.

Casts 0063 to 0082: fuzzy fluo signal.  Fluo cable changed after cast 0082.

- Niskins & Samples at CTD/Rosette casts:

Cast 0034 – 27.5: Niskin 1 was closed at the surface just to check the pressure in the hydro file.  No sampling, no CHE file needed.

Cast 0062 – 56: Niskins 2 and 3 not closed, not needed.  Station was shallower than planned.

File names – Headers:

Casts 0001 to 0014 and 0087 to 0106: Instead of Ship: Tully; Cruise 2020-028, the header has Tully:2020-028.

Casts 0016 to 0086: Ship:Tully has a ‘ after Tully.

Cast 0001 – SI: Has no station name and no depth in header, should be SI and 175.

Cast 0008 – 19: Wrong depth in header, is 163 should be 262.

Cast 0074 – 61: wrong depth in header, is 271 should be 224.

Cast 0104 – 74: wrong depth in header, is 240 should be 193.

Cast 0105 – 75: wrong depth in header, is 240 should be 221.

Casts with no PAR sensor:

None.

Cast out of order:

None.

Loops:

See loops log.

TSG:

We used the SBE-45.  All files in SCS format.