## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |

## PROCESSING NOTES

Cruise: 2021-017

Agency: IOS, Ocean Sciences Division, Sidney BC

Chief Scientist: O’Neill C.  		Platform: Vector

Location: SoG/JdeF/WCVI		Project: Acoustic Moorings

Date: 30 May 2021 –8 June 2021

Processed by: Germaine Gatien

Date of Processing: 10 June 2021 – 16 June 2021

Number of original HEX files: 93		Number of CTD files:	93

# INSTRUMENT SUMMARY

A SeaBird Model SBE-911+ CTD (s/n 0506) was used with temperature sensors #2374 and 4883, conductivity sensors #3184 and 4395, SeaPoint Fluorometer (#3949) on the primary pump (3X gain), dissolved oxygen sensor #1119 on the secondary pump.

The CTD deck unit was an SBE11+ (#0508) and the logging computer was WP Gamer laptop (Acer).

Software Version Seasave V 7.26.7.121 was used for acquisition.

# SUMMARY OF QUALITY AND CONCERNS

There was no personnel list in the log book Daily Science Log Book.

The fluorometer data were removed because they lacked the normal detail and the sensor appeared to have a long response time.

There was no calibration sampling. All sensors had been serviced recently. Dissolved Oxygen values appear reasonable, but are likely to be slightly low; corrections of +2 to +3% are typical for this type of sensor. Salinity drift is expected to be small. The primary and secondary salinity values differed by an average of 0.0035psu in the deepest water sampled, so accuracy is likely on the order of +/- 0.005psu. Differences between temperature channels were very small, ~0.0002C°.

# PROCESSING SUMMARY

##### Seasave -

This step was completed at sea.

##### Preliminary Steps

The Daily Log was obtained. There were no significant problems noted during the cruise.

The cruise summary sheet was completed.

##### Conversion of Full Files from Raw Data

Files were converted using 2021-017-ctd.xmlcon.

2 files had errors in the file names; an extra 0 was removed.

The Tau function was selected but not the hysteresis function as there was no sampling below 1000db. Depth was included in the conversion.

A few casts were examined and all expected channels are present. The altimetry is excellent within 100db of the bottom. The temperature and conductivity traces are close to each other, particularly during downcasts. The upcast traces are cleaner than usual, presumably due to the absence of a rosette. The dissolved oxygen profiles look normal. Descent rates varied greatly being steady in protected waters and extremely noisy in exposed waters.

The one thing that looked poor was fluorescence. The offset between downcast and upcast is much larger than usual and there is no detail in the profiles. We expect spikiness.

##### WILDEDIT

Program WILDEDIT was run to remove spikes from the pressure, conductivity & temperature only in the

full cast files (*.CNV).

Parameters used were: 	Pass 1    Std Dev = 2 	Pass 2    Std Dev = 5 	Points per block = 50

The parameter “Keep data within this distance of the mean” was set to 0 so all spikes would be removed.

##### ALIGN DO

ALIGNCTD was run on all casts using +2.5s.

##### CELLTM

CELLTM was run using the default setting (α = 0.0245, β=9.5) for both the primary and secondary conductivity. A few casts were checked and this step does improve the data.

##### DERIVE and Channel Comparisons

Program DERIVE was run on all casts to calculate primary and secondary salinity and dissolved oxygen concentration.

DERIVE was run a second time on the final cast to find differences between the pairs of temperature, conductivity and salinity channels. There were no deep casts.

| Cast # | Press | T1-T0 | C1-C0 | S1-S0 | Descent Rate |
| --- | --- | --- | --- | --- | --- |
| 2021-017-0075 | 320 | -0.0002 | -0.00035 | -0.0035 | High, F.Steady |
| 2021-017-0108 | 340 | -0.0002 | -0.00030 | -0.0035 | High, Steady |

##### Conversion to IOS Header Format

The IOSSHELL routine was used to convert SEA-Bird 911+ CNV files to IOS Headers.

CLEAN was run to add event numbers and to replace pad values in the pressure channel with interpolated values based on record number.

##### Checking Headers

The header check was run. The only problem found is that the minimum fluorescence value is >2.5ug/L which is unbelievable. A CTD technician discovered that the fluorometer had been attached incorrectly which may account for the problem. That channel will be removed later.

The cross-reference list was checked against the log book.  The only problem noted was that there was a CTD Cast #1 entered in the log but no data were found for that event. It was a Saanich Inlet test cast, so not a great concern.

The altimeter and water depth readings from the headers of the CLN files were exported to spreadsheets. A check was made by subtracting maximum depth sampled plus altimetry header reading from the water depth and problems were found in some casts. For 1 cast there was clearly an error in the header so the log entry was used to correct it. For 2 casts there was a note in the log that the depth varied a lot during the cast. For those 2 plus 9 other events the sum of the depth sampled plus altimetry header value were used to replace the header depth entry; a few of the original entries looked like typos. In this way the header depth reflects the depth at the time of the deepest sampling.

A surface check was run and shows an average surface pressure for the cruise was 2.9db. Acquisition did not begin until after the 10m soak so this is reasonable. The lowest pressure recorded was 1db and conductivity values indicate the CTD was in water. Acquisition stopped at 1 to 2db as soon as the pumps were turned off, so there is no indication of how accurate the pressure is.

##### Shift

Fluorescence

This channel will not be archived so no further processing was applied to it.

Dissolved Oxygen

The Dissolved Oxygen voltage channel was aligned earlier and the results look good.

Conductivity

Tests were run on 5 casts to find the best setting for aligning conductivity with temperature.

SHIFT was run applying a setting of -0.8 records to the primary conductivity and -0.7records for the secondary conductivity. Salinity was recalculated for both channels.

##### DELETE

The following DELETE parameters were used:

Surface Record Removal: Last Press Min

Maximum Surface Pressure (relative): 10.00

Surface Pressure Tolerance: 1.0                  Pressure filtered over 15 points

Swells deleted. Warning message if pressure difference of 2.00

Drop rates <   0.30m/s (calculated over 11 points) will be deleted.

Drop rate applies in the range:  10db to 10db less than the maximum pressure

Sample interval = 0.042 seconds. (taken from header)

COMMENTS ON WARNINGS: There were no warnings.

##### Calibration information

Previous experience with these sensors –  None. All factory serviced recently.

Historic ranges – All Salinity fell within the local climatology except for 5 casts in the westernmost part of Juan de Fuca Strait that had low salinity between about 80 and 120db. All temperatures fell within the climatology except for a very slight excursions in one cast that also had a salinity excursion. These excursions do not suggest a calibration problem as the 3-standard deviation climatology is not really suitable to near-shore sampling.

Repeat Casts – Many nearby casts but they are too shallow to provide a reasonable comparison.

Post-Cruise Calibration – None.

##### DETAILED EDITING

There was little difference between the T/S pairs but the secondary was slightly more stable in T-S space so was selected for archiving since it should require a little less editing.

CTDEDIT was used to remove records that appear to be corrupted by shed wakes or ship effects. Salinity was cleaned where there were small spikes clearly caused by slight mismatches in timing. Data were only removed from near the top and bottom of most casts though a few with a very noisy CTD descent rate required removal more corrupted records. Salinity was cleaned lightly for many casts.

All files required some editing except for events #47, 71, 76, 78, 79, 88, 89, 97, 98, 102, 114 and 123.

After editing T-S plots were examined for all casts. There are some small unstable features which could well be real as they come from areas of active mixing. No further editing was applied.

##### Recalibration

There was no recalibration.

##### Fluorescence Processing

The fluorescence data are bad so no filter was applied.

##### BIN AVERAGE of CTD files

The following Bin Average values were applied to the EDT files (output AVG):

Bin channel = pressure	Averaging interval = 1.000	Minimum bin value =   .000

Average value will be used.	Interpolated values are NOT used for empty bins.

On-screen T-S plots were examined. There are only the small instabilities that are expected in this region.

Profile plots were examined to see if there any problems. The fluorescence values were very high at depth. No other problems were noted.

##### Final CTD File Steps (REMOVE and HEADEDIT)

REMOVE was run to remove the following channels:

Scan_Number, Temperature:Primary, Salinity:T0:C0, Conductivity:Primary,  Oxygen:Voltage:SBE, Fluorescence, Altimeter, Status:Pump, Descent_Rate and Flag.

A second SBE DO channel (with umol/kg units) was added.

REORDER was run to get the two DO channels together.

HEADER EDIT was used to fix formats and channel names and to add header comments about the data processing.

The Standards Check routine was run and no problems were found.

The Header Check was run and no problems were found.

Profile and T-S plots were examined. There are some unstable features in T-S space where waters are well-mixed, but those are very small and may be real.

The sensor history was updated.

##### Dissolved Oxygen Study

As a final check of dissolved oxygen data, % saturation was calculated and plotted. The surface values varied from about 55% to 170%. The highest values came from the offshore section ranging from 90% to 170% with most between 90% and 110%. The lowest values were in Juan de Fuca Strait, Haro Strait and among the Gulf Islands where mixing was very strong. In the southern Strait of Georgia values ranged from 90% to 140% with most between 90% and 110%. The range of values observed looks reasonable for this region. Dissolved oxygen values from this type of sensor generally drift lower; from a newly serviced instrument values low by 2% would be typical.

Particulars

61. On a steep slope – bottom depth started at 85m – later 97m.

64. Depth kept increasing during cast.

CRUISE SUMMARY

| Cruise ID#: 2021-017 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Dates: Start: 30 May 2021 End: 8 June 2021 |  |  |  |  |  |  |
| Location: |  |  |  |  |  |  |
| Chief Scientist: O’Neill C. |  |  |  |  |  |  |
| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |  |
| 1 | SEABIRD | 911+ | 0506 | No | Yes |  |

Make/Model/Serial#:	SEABIRD/SBE911+/0506  Cruise ID#:		2021-017

| Calibration Information – 0443 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sensor |  | Pre-Cruise |  | Post Cruise |  |
| Name | S/N | Date | Location | Date | Location |
| Temperature | 2374 | 03Feb2021 | Factory |  |  |
| Conductivity | 3184 | 03Mar2021 | Factory |  |  |
| Secondary Temp. | 4883 | 04Feb2021 | Factory |  |  |
| Secondary Cond. | 4395 | 03Mar2021 | Factory |  |  |
| SBE 43 DO sensor | 1119 | 05Feb2021 | Factory |  |  |
| SeaPoint Fluor. | 3949 |  |  |  |  |
| Pressure Sensor | 0443 | 29Jan2021 | Factory |  |  |
| Altimeter | 76341 |  | Factory |  |  |