File List for SeaBird Data          Cruise Number: _2021-038____

This section refers to data and documents for all 3 CTDs

[2021-038\ARCHIVE]

[2021-038\ARCHIVE\Bottle] *.CHE

[2021-038\ARCHIVE\CTD] *.CTD

[2021-038\ARCHIVE\DOC] documents

[2021-038\ARCHIVE\DOC\HYDRO] COMPARE files and reports

[2021-038\ARCHIVE\LOGS] digital or scanned logs

---------------------------------------------------------------------------------------------------------------------------------------

The following sections contain folders in separate groups for each CTD

SBE911+

[2021-038\Processing\SBE911+\PROCESSING\CONVERT] *.CNV after conversion

[2021-038\Processing\SBE911+\PROCESSING\DERIVE] *.CNV – after derive

[2021-038\Processing\SBE911+\PROCESSING\DOC]

2021-038-0002.xmlcon – configuration file

2021-038-SBE911-final.rpt – header check

2021-038-sbe911-hdr.txt – header comments

[2021-038\Processing\SBE911+\PROCESSING\IOS]

*.avg - data after metre-averaging using BINAVE

*.clip – after removal of data from soak period

*.cln - data after conversion to IOS HEADERS and a run of CLEAN with wrong DO sensor

*.ctd-  final files

*.del – after DELETE with wrong DO sensor

*.edt - data after editing has been done

*.ios – data after conversion to IOS SHELL format

*.log - log files created by CTDEDIT

*.rem – after REMOVE

*.reo– after reordering channels

*.sat – with Diss. Oxy. Saturation

Delete.log - log from routine DELETE

SBE19+

[2021-038\Processing\SBE19+\PROCESSING\CONVERT]	*.CNV

[2021-038\Processing\SBE19+\PROCESSING\DERIVE]*.CNV – after derive

[2021-038\Processing\SBE19+\PROCESSING\DOC]

2021-038-ctd19+-final.rpt – CTD file header check including channel summary

2021-038-SBE19+-Header-merge.csv – Header information to add to files

2021-038-altimeter-ctd.csv – report on header entries for altimetry

2021-038_File_List.Doc – This report.

2021-038_Processing_Report.doc – Report on Processing for all 3 CTDs

2021-038-sbe19plus.xmlcon – configuration file used for all casts

*.PNG – pictures of track plots

*hdr.txt – header file with comments for CTD files

[2021-038\Processing\SBE19+\PROCESSING\DOC\hydro]

2021-038-che-final.rpt – Header check for CHE files

2021-038sal.csv – salinity data from Autosal, simplified from analyst’s s/s

2021-038-sal-comp1.csv & xls –initial comparison of rosette bottle and CTD salinity

addsamp.csv – spreadsheet relating bottle numbers to sample numbers

bottles.csv – summary of bottle files

*hdr.txt – header file with comments for CHE files

[2021-038\Processing\SBE19+\PROCESSING\HYDRO]

*.bot - ros files converted to IOS HEADERS + added event numbers

*.che – final CTD plus chemistry data after CLEAN to fix channel table

*.hdr – 2nd last step after fixing formats and headers

*.mrg – after addition of bottle data to BOT files

*.mrgcln2 – after addition of 0s to empty quality flags

*.mrgrem2 – after 2 runs of REMOVE

*.mrgreo –after recording channels

*.ros  - rosette files created by DATCNV, no other processing done.

*.sal – salinity files converted from simplified version of analyst’s spreadsheet

*.sam – bottle files with sample numbers added

[2021-038\Processing\SBE19+\PROCESSING\IOS] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.cln – step after conversion to fix headers

*.ctd-  final files (after CLEAN run to fix channel limits)

*.del– after DELETE

*.delx– after DELETE with low drop rate turned off

*.dqt – with DO saturation channel

*.edt - data after editing has been done

*.hdr – 2nd last step – fix formats and add header comments

*.ios – data after conversion to IOS SHELL format

*.log - log files created by CTDEDIT

*.mrh –2nd run of adding header into to include water depths

*.rem – after REMOVE (just before Header Edit for SBE25 files)

Delete.log - log from routine DELETE

RBR

[2021-038\Processing\RBR\DOC]

2021-038_CTD_Data.csv – RBR data

2021-038_CTD_Data-6linehdr.csv -RBR data in 6-line-header format

2021-038_Header-merge.csv – Info to go into CTD headers

2021-038_METADATA.csv – Info from RBR headers

2021-038-RBR.ccf – recalibration file for conductivity

2021-038-SBE911-final.rpt – header check

2021-038-RBR-hdr.txt – header comments for CTD files

[2021-038\Processing\RBR\Processing]

*.avg – after metre-averaging

*.cln – after addition of start time to headers

*.cor1 – after change of conductivity format

*.ctd – final files – CLEAN applied to fix header secction

*.del – after removal of upcast and reversals.

*.edt – after graphical ediitng

*.fil – after filtering T,C,P

*.hdr – after Header Edit

*.ios – after conversion to IOS Headers

*.mrh – after addition of header information