File List for SeaBird Data          Cruise Number: _2021-048____

[osdcommon\cruise_data\2021-048\ARCHIVE]

[osdcommon\cruise_data\2021-048\ARCHIVE\CTD] *.CTD

[osdcommon\cruise_data\2021-048\ARCHIVE\DOC] most documents from RAW and PROCESSING

[osdcommon\cruise_data\2021-048\ARCHIVE\LOGS] scanned log

[osdcommon\cruise_data\2021-048\PROCESSING]

[osdcommon\cruise_data\2021-048\PROCESSING\CONVERT]	*.CNV – after conversion

[osdcommon\cruise_data\2021-048\PROCESSING\DERIVE]*.CNV – after convert/wildedit/celltm/derive

[osdcommon\cruise_data\2021-048\PROCESSING\DOC]

2021-048-ctd.xmlcon – configuration file used in processing for SBE911

2021-048_File.doc - this document

2021-048_Processing_Report.doc - processing notes

2021-048-ctd-final.rpt – CTD file header check including channel summary

2021-048-ctd-xref.txt – cross reference list of CTD files –both CTDs

2021-048-altimeter-ctd.xlsx – study of altimetry & water depth entries

Pressure-study.xlsx – deck pressure readings

recal1.ccf – calibration file to correct fluorescence

*hdr.txt – header file with comments for CTD files

[osdcommon\cruise_data\2021-048\PROCESSING\IOS] after conversion to IOS SHELL

*.avg - data after metre-averaging using BINAVE

*.cln - data after conversion to IOS HEADERS and a run of CLEAN

*.ctd-  final files (after correction of ECO FL)

*.ctdo- files before correction of fluorescence

*.del – after DELETE

*.edt & log - data after editing has been done (an EDT file is prepared even if no editing is required.)

*.ios – after conversion to IOS Header format

*.rem – after REMOVE (just before Header Edit for SBE25 files)

*.reo– just before final Header Edit – use if need to update comments

Delete.log - log from routine DELETE