#### 2022-008

CCGS John P Tully

#### Dimethylsulfide (DMS) & Dimethylsulfoniopropionate (DMSP) Report

#### August 09-August 25, 2022

Prepared by Michael Arychuk for Danielle Caleb

- Sample Collection

Samples were collected from stations  P2, P4, P12, P16, P20 and P26 for DMS, DMSPD (dissolved) & DMSPT (total).

  - DMS

The sampling depths were adjusted on this cruise.  Eleven water samples (100m, 75m, 50m, 40m, 30m, 25m, 20m, 15m, 10m, 5m, surface) were collected at station P2.  Ten samples (400m, 150m, 100m, 50m, 30m, 20m, 15m, 10m, 5m, surface) were collected at  P12, P16 and P20   At P26 there were fourteen samples collected (400m, 200m, 150m, 100m, 75m, 50m, 40m, 30m, 25m, 20m, 15m, 10m, 5m, surface). Duplicates were taken at 20m.  In all cases, samples were collected in 250mL ground glass stoppered bottles and stored in a fridge, in the dark and removed one at a time before analysis.

  - DMSP

Seven samples( 400m, 200m, 150m, 30m, 20m, 5m, surface) for both DMSPD and DMSPT were collected at P4, P12, P16, P20 & P26.  Four samples (100m, 20m, 5m, surface) were collected at P2. Duplicates were taken at 20m.

- Analysis

2.1 DMS

A sample was loaded onto the stripper and purged with UHP Helium for 10 minutes at ~100mL/min.  The DMS was extracted from the water and absorbed onto a Tenax TA trap kept at -80oC.  The trap was subsequently desorbed at 100oC (with a dewar containing boiling water) onto a Chromasil 330 column which eluted onto a Flame Photometric Detector (FPD).  All samples were run as soon as possible after being collected.

2.2 DMSPD

Approximately 50-75mL of seawater was allowed to flow directly from the niskin into a filtration funnel containing a 0.7m GF/F filter.  The first 3.5mL was collected in a 15mL or 5mL polypropylene tube.  The entire 3.5mL was then transferred into a 5ml, glass, serum bottle and 50L of a 50% sulphuric acid/water solution was added.    The sample was then crimp sealed and stored in the dark and at 4˚C where it would be analysed back at IOS at a later date.

2.3 DMSPT

Exactly 3.5mL of seawater was collected directly from the niskin into a 15mL or 5mL polypropylene tube. The entire 3.5mL was then transferred into a 5ml, glass, serum bottle and 50L of a 50% sulphuric acid/water solution was added.    The sample was then crimp sealed and stored in the dark and at 4˚C where it would be analysed back at IOS at a later date.

- Calibration

3.1 DMS

A four to six level calibration table was used for calculating the concentrations of DMS.  The standards were prepared in water and run under the same conditions, as described above, for the samples.  A calibration curve was valid for 12 hours.  If analysis exceeded 12 hours, a continuing calibration standard was run to ensure the calibration curve was still within acceptable limits.

- Quality Control

4.1 DMS

System blanks and duplicates were run approximately every 13 samples to ensure the system remained free of contamination and had acceptable reproducibility.

4.2 DMSP

Blanks and duplicates were collected at every station.  Blanks were done by either using MQ water, some of the bottled SQ water brought from the lab or using some deep sea water (2000 m or deeper) and treating them as actual samples.    For example, in the case of DMSPD,   3.5mL was collected from a separate funnel and for DMSPT    3.5mL  was added directly to the polypropylene tube.  Like the samples, they were then transferred into a 5ml, glass, serum bottle and 50L of a 50% sulphuric acid/water solution was added.    The blank was then crimp sealed and stored in the dark and at 4˚C where it would be analysed back at IOS at a later date.

- Data & Results

  - DMS

Data from this cruise showed some abnormally high values for P12, P16, P20 and P26.  It was also noted that DMS was detected at 400m for stations P4 and P16.  Although it is extremely rare to see DMS at this depth the values cannot be ignored based on the higher values seen overall and the fact there is no reason to discount them.  It would be advisable to perhaps collect samples at a deeper depth than 400m when higher values are seen at other depths so as to determine if there is a trend or pattern developing.

  - DMSP

The DMSP samples were collected for all stations.  Samples were run back in the lab in late September 2022.

- Conclusions

6.1 DMS

There continues to be a strong suspicion that the ship water is affecting the quality of the MQ water on board the ship and as a result the water used for standards and blanks for DMS is now obtained differently. Deep sea water access is a problem for P2 and P4 because of the  depth (typically the water needs to be deeper than 1500m) and the timing of the stations.  For this reason SQ water is brought from the lab to accommodate the blank and standard requirements for those two stations.  After P4 deep sea water (2000m or deeper) is now collected at stations before the major station, stored in an amber glass bottle in the walk-in cooler and then used for blanks and standards.  As long as the water is used within 12-18 hours there does not appear to be any contamination issues.  Going forward this will be implemented as part of the procedure for standard and blank preparation.

  - DMSP

Sample collection went well without any issues.