## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |

## PROCESSING NOTES

Cruise: 2022-052

Agency: Ocean Sciences Division (OSD)

Location: Arctic

Project: Kitikmeot Study 2022

Party Chief: Macoun, P.

Platform: CCGS Sir Wilfrid Laurier

Date: 16-30 August 2022

Processed by: Jeannette Bedard

Date of Processing: 14 May 2025 – 4 June 2025

Number of original HEX files: 7

Number of CTD files:  	7	Number of CTD casts processed: 7

Number of bottle files: 	7	Number of bottle casts processed: 7

Number of TSG files:   0	Number of TSG files processed:  0

# INSTRUMENT SUMMARY

CTD #0941 was used for the entire cruse. Piggybacked instruments are listed in the table at the end of this report. No TSG files were found.

Seasave version 7.26.7.107 was used for acquisition.

There was no thermosalinograph data.

# SUMMARY OF QUALITY AND CONCERNS

In general, the data was in a good state and all required documentation was provided.

# PROCESSING SUMMARY

GENERAL

##### Seasave

This step was completed at sea; the raw data files have extension HEX.

##### Preliminary Steps

- The digital daily log and rosette log sheets were obtained and checked for comments.

- Chlorophyll, Salinity, Nutrients and Dissolved Oxygen data were obtained in QF spreadsheets.

- The history of the pressure sensor, conductivity and dissolved oxygen sensors were checked.

- The calibration control files were obtained – as they stayed the same through out the cruise 2023-080-0001.xmlcon was used for processing.

##### BOTTLE FILE PREPARATION

The ROS files were created using 2022-052-0001.xmlcon. The hysteresis correction was not selected since there were no very deep casts; the Tau correction was selected.

The ROS files were converted to IOS format

The files were put through CLEAN to create BOT files. Temperature and salinity were plotted for all BOT files to check for outliers, none were found.

The BOT files were bin-averaged on bottle number and the output was used to create file ADDSAMP.csv.

- Sample numbers were added based on chemistry.csv file provided (there were no rosette logs).

The ADDSAMP file was then sorted on event number & then sample number. Converted to CST files using Convert Spreadsheet.

It was used to add sample numbers to the BOT files – output *.SAM.

The SAM files were bin-averaged on bottle # and called SAMAVG.

The addsamp.csv file was converted to CST files, which will form the framework for the bottle files.

Next, the analysis spreadsheets were examined to see what comments the analysts wanted included in the header file. These were saved in file 2022-052-bot-hdr.txt.

In this case all chemistry results were in a single file: QF 2022-052_SWL_Chemistry_2025-02-27c.xlsx which was converted to individual SAL file – note the variables included salinity, nutrients and oxygen 18.

All the bottle data was merged with the CST files at once. The files were then put through CLEAN to reduce the headers to File and Comment sections only. These files were ordered on sample number, but the SAMAVG files are ordered on bottle number, so the MRGCLN1 files were reordered on Bottle_Number and saved as *. MRGCLN1s. The MRGCLN1s files were then merged with SAMAVG files using merge channel Bottle_Number.

##### Compare  (note: there were no dissolved oxygen samples taken)

Salinity

There were only seven casts and they were all shallow. However, the fit against pressure is reasonably flat. For more detail see file 2022-052-sal-comp.xls. An offset of 0.0132 was applied.

##### Conversion of Full CTD Files from Raw Data

All files were converted using files 2022-052-0001.xmlcon. The hysteresis correction was not selected since there were no deep casts; the Tau correction was selected. A few casts were examined and all expected channels are present.

##### WILDEDIT

Program WILDEDIT was run to remove spikes from the pressure, conductivity & temperature only in the

full cast files (*.CNV).

Parameters used were: 	Pass 1    Std Dev = 2 	Pass 2    Std Dev = 5 	Points per block = 50

The parameter “Keep data within this distance of the mean” was set to 0 so all spikes would be removed.

##### ALIGN DO

ALIGNCTD was run on all casts using +2.5s.

##### CELLTM

CELLTM was run using the default settings (α = 0.0245, β=9.5) for both the primary and secondary conductivity.

##### DERIVE

Program DERIVE was run on all casts to calculate primary and secondary salinity and dissolved oxygen concentration.

##### Conversion to IOS Header Format

The IOSSHELL routine was used to convert SeaBird 911+ CNV files to IOS Headers.

CLEAN was run to add event numbers and to replace pad values in the pressure channel with interpolated values based on record number. There are no negative fluorescence values.

##### Checking Headers

A header check and cross-reference list was run and no issues were found The track plots were added to the end of this report, one with station names and the other with cast number (note, bathymetry was not available for the entire sample area).

Because there were only 7 casts and all were shallow, the altimetry reading were checked by hand and found to be fine. Surface check was not performed due to the noisy conditions of the surface data.

##### Shift

Fluorescence

SHIFT was run on the fluorescence channel with an advance of 24 records. Plots showed this setting was appropriate.

Dissolved Oxygen

The Dissolved Oxygen voltage channel was aligned earlier using Seabird software. No further alignment is needed for the DO concentration channel.

Conductivity

Due to the shallow nature of these casts a suitable conductivity shift was impossible to determine.

##### DELETE

The following DELETE parameters were used:

Surface Record Removal: Last Press Min

Maximum Surface Pressure (relative): 10.00

Surface Pressure Tolerance: 1.0                  Pressure filtered over 15 points

Swells deleted. Warning message if pressure difference of 2.00

Drop rates <   0.30m/s (calculated over 11 points) will be deleted.

Drop rate applies in the range:  10db to 10db less than the maximum pressure

Sample interval = 0.042 seconds. (taken from header)

##### Other Comparisons

Previous experience with these sensors – These sensors were used on 2023-029 shortly before this cruise (there was one cruise in between).

Historic ranges – These casts were all taken in coastal sites and fit within climatology values temperature for the most part. The salinity values do not agree as well.. There is no indication of calibration problems.

Post-Cruise Calibration – There were no post-cruise calibrations available.

##### DETAILED EDITING

The primary T-S pair were chosen for editing and eventual archiving. For this cruise both primary and secondary were very similar.

CTDEDIT was used to remove records near the top and bottom of the casts where the descent rate was below 0.2 m/s. Multiple shed wakes were removed from cast 1.

The edited files were copied to *.EDT.

After editing T-S plots were examined for all casts; there remain many small unstable features but they are in areas when they may well reflect real conditions. No further editing was applied.

##### Fluorescence Processing

A median filter with a width of was applied to the fluorescence channel.

##### Recalibration

The minimum CTD Salinity was below the <25 psu threshold that requires correction to silicate samples. Silicate was calibrated using 2022-052-recal-SIL.ccf.

The salinity correction was:

CTD Salinity Corrected = CTD Salinity + 0.0132

For full details for the COMPARE run see file 2022-052-sal-comp.xls.

##### BIN AVERAGE of CTD files

The following Bin Average values were applied to the FIL files (output AVG):

Bin channel = pressure	Averaging interval = 1.000	Minimum bin value =   .000

Average value will be used.	Interpolated values are NOT used for empty bins.

On-screen T-S plots were examined.

Profile plots were examined to see if there any problems. No problems were noted.

##### Remove unwanted channels

The following channels were removed from cast data:

- Scan_Number

- Temperature:Secondary

- Conductivity:Secondary

- Salinity:T1:C1

- Altimeter

- Status:Pump

- Descent_Rate

- Flag

- Oxygen:Voltage:SBE

##### Final CTD File Steps (REMOVE and HEADEDIT)

- A second SBE DO channel (with umol/kg units) was derived.

- HEADER EDIT was used to fix formats and channel names and to add comments to the headers.

- The Standards Check routine was run and no problems were found.

- The Header Check was run; no problems were found.

- Profile and T-S plots were examined and no problems were found.

- The sensor history was updated.

##### Final Bottle Files

- The MRGCOR1 files were put through SORT to order on increasing pressure.

- Remove unwanted channels:

  - Scan_Number

  - Temperature:Secondary

  - Conductivity:Secondary

  - Altimeter

  - Status:Pump

  - Descent_Rate

  - Flag

  - Oxygen:Voltage:SBE

- A second SBE DO channel with mass units was added for both the CTD DO and REORDER was run to get the pair of DO channels together.

- HEADER EDIT was run to ensure formats and units are correct

- Standards check was run and no problems were found.

- Bottle data were exported to a spreadsheet and no problems were found.

- A header check were run. No problems were found.

- A cross-reference listing was produced for the CHE files.

CRUISE SUMMARY – CTD

| CTD# | Make | Model | Serial# |  | Used with Rosette? |  | CTD Calibration Sheet Competed? |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SEABIRD | 911+ | 0941 |  | Yes |  | Yes |  |  |  |
| Calibration Information - 0941 |  |  |  |  |  |  |  |  |  |  |
| Sensor |  |  |  | Pre-Cruise |  |  |  | Post Cruise |  |  |
| Name |  | S/N |  | Date |  | Location |  | Date | Location |  |
| Temperature |  | 5048 |  | 12Dec2018 |  | Factory |  |  |  |  |
| Conductivity |  | 3579 |  | 12Dec2018 |  | Factory |  |  |  |  |
| Secondary Temp. |  | 5073 |  | 13Dec2018 |  | Factory |  |  |  |  |
| Secondary Cond. |  | 3581 |  | 13Dec2018 |  | Factory |  |  |  |  |
| Transmissometer |  | 9993 |  |  |  |  |  |  |  |  |
| SBE 43 DO sensor |  | 1202 |  | 25Oct2017 |  | Factory |  |  |  |  |
| SeaPoint Fluor. |  | 3575 |  |  |  | Factory |  |  |  |  |
| Pressure Sensor |  | 0941 |  | 6Apr2015 |  | Factory |  |  |  |  |
| Altimeter |  | 60143 |  |  |  | Factory |  |  |  |  |