CTD FILES

CTD files consist of a header followed immediately by columns of data.

The header is composed of 12 lines of 80 characters each.  The following

tables describe the contents of the header.

Line One:

---------

POSITION   DATA TYPE   NAME         LENGTH    DESCRIPTION

______________________________________________________________________

1 - 4     CHARACTER  CRUISE	      4	      the cruise identifier

5 - 8     CHARACTER  STN             4	      the station identifier

9 - 10    INTEGER    LATD	      2       latitude degrees

11 - 15    REAL	      LATM	      5       latitude minutes

16 - 16    CHARACTER  NS	      1	      latitude north or south

17 - 19    INTEGER    LONGD	      3	      longitude degrees

20 - 24    REAL	      LONGM	      5       longitude minutes

25 - 25    CHARACTER  EW	      1	      longitude east or west

26 - 27    INTEGER    YEAR	      2	      year data was collected

28 - 29    INTEGER    MONTH           2	      month data was collected

30 - 31    INTEGER    DAY	      2	      day data was collected

32 - 33    INTEGER    HOUR	      2	      hour data was collected

34 - 35    INTEGER    MINUTE	      2	      minute data was collected

35 - 38    CHARACTER  ZONE	      3	      time zone

39 - 41    INTEGER    CONSEC          3	      (usually consecutive) cast number

42 - 53    CHARACTER  PROJ	     12	      cruise description

54 - 55    INTEGER    NCHAN	      2	      number of data channels

56 - 67    CHARACTER  CHANDS(12)     12 X 1   channel descriptors (see ** below)

68 - 71    INTEGER    MAXDEP	      4	      maximum depth

72 - 76    INTEGER    NREC	      5	      number of data records

77 - 79    REAL       DEPAVG	      3	      depth averaging interval (m)

80 - 80    CHARACTER  CODE	      1	      1 for CTD data

Line Two:	instrumentation information ie type of probe, serial # etc.

---------

1 - 80    CHARACTER  INSTRM	     80

Line Three:  primary calibration data, a,b in aX+b

-----------

1 - 20    REAL	      PC0, PC1       20 (2 X 10)   Pressure

21 - 40    REAL	      TC0, TC1	     20 (2 X 10)   Temperature

41 - 60    REAL       CC0, CC1	     20 (2 X 10)   Conductivity

61 - 80    REAL       TR0, TR1       20 (2 X 10)   Transmissivity

Line Four:  secondary calibration data, a,b in aX+b

----------

1 - 20    REAL       TT1, TT2	     20 (2 X 10)   Temperature

21 - 40    REAL	      SS1, SS2	     20 (2 X 10)   Salinity

Line Five:  aquisition software information,

----------	eg program name, programmer's name, sampling rate

1 - 80    CHARACTER  AQSOFT	     80

Line Six: 	data record format string (Fortran formatting)

---------

1 - 80    CHARACTER   FT	     80

Lines 7 - 12:  Comments

-------------

1 - 80    CHARACTER   CMNTS(6)      80	      comments

**  NOTE that the channel descriptors in line 1 of the header are given

as a 1 character code per channel.  The code is as follows:

P = Pressure

T = Temperature

S = Salinity

% = Transmissivity

The order of the code characters is the order of the data

channels, and the order will almost always be

Pressure, Temperature, Salinity.