Summary of Hydrographic/CTD Observations Made Along Line P and at Station P: 1991-1999

B. G. Minkley and F. A Whitney

Fisheries and Oceans Canada

Institute of Ocean Sciences

9860 West Saanich Road

Sidney, British Columbia V8L 4B2

2001

Canadian Data Report of

Hydrography and Ocean Sciences 156

Canadian Data Report of Hydrography

and Ocean Sciences 156

2001

Summary of Hydrographic/CTD Observations Made Along Line P and at Station P: 1991-1999

by

B.G. Minkley and F.A. Whitney

Fisheries and Oceans Canada

Institute of Ocean Sciences

9860 West Saanich Road

Sidney, BC, Canada

V8L 4B2

© Her Majesty the Queen in Right of Canada, 2001,

as represented by the Minister of Fisheries and Oceans.

Cat. No. Fs-97-16/156E  ISSN 0711-6721

Correct citation for this publication:

Minkley, B.G., and F.A. Whitney.  2001. Summary of hydrographic/CTD observations made along Line P and at Station P: 1991-1999.  Can. Data Rep. Hydrogr. Ocean Sci. 156: 45 p.

#### CONTENTS

ABSTRACT	iv

Introduction	1

Ships and Personnel	2

Table 1. List of ships, dates, and personnel, 1991 to 1999	5

Table 2. Types of samples taken on each mission.	17

Water Sampling Equipment	19

Table 3 of JGOFS and WOCE designated missions.	21

Plots of CTD Locations for missions done 1991 to 1999	23

Figure 1.  IOS Rosette	35

Figure 2.  General Oceanics Rosette	37

Figure 3.  Heave Compensator mounted on CCGS Tully A-frame	39

Figure 4.  CSS Parizeau	41

Figure 5.  CCGS J.P. Tully	43

Figure 6.  CNAV Endeavour	45

## ABSTRACT

Minkley, B.G., and F.A. Whitney.  2001. Summary of hydrographic/CTD observations made along Line P and at Station P: 1991-1999.  Can Data Rep Hydrogr Ocean Sci 156: 45 p.

A summary of 30 missions which sampled part or all of Line P between January 1991 and December 1999 is provided.  Plots of mission tracks show the areas surveyed, participants are listed, and types of sampling, inventoried.  A review of sampling equipment outlines the conversion from 1.7 litre Niskin bottles mounted on 5/32-inch wire hydro wire to CTD/Rosette cast sampling using 10 litre bottles.  Missions which supported programs for World Ocean Circulation Experiment and Joint Global Ocean Flux Studies are listed.

A previous report contained observations for the period January 1980 to December 1989 and is found in Canadian Data Report of Hydrography and Ocean Sciences No.136, by Tabata and Brown.

Key words:	North Pacific Ocean, open-ocean time-series, hydrographic/CTD data.

Résumé

Minkley, B.G., and F.A. Whitney.  2001. Summary of hydrographic/CTD observations made along Line P and at Station P: 1991-1999.  Can. Data Rep. Hydrogr. Ocean Sci. 156: 45 p.

Ce rapport fourni un sommaire de 30 croisières océanographiques durant lesquelles la ligne P a été échantillonnée en tout ou en partie entre janvier 1991 et décembre 1999. Chaque sommaire comprend la région explorée représentée par la trajectoire du navire, la liste des participants, ainsi qu'un inventaire du type d'échantillons collectés. Une révision du matériel d'échantillonnage souligne la conversion de bouteilles Niskin de 1.7 litre montées sur un câble de 5/32" à des profils de CTD/Rosette utilisant des bouteilles de 10 litres. Les croisières faisant partie des programmes WOCE (World Ocean Circulation Experiment) et JGOFS (Joint Global Ocean Flux Studies) sont énumérées.

Un rapport précédant contenant les observations pour la période de janvier 1980 à décembre 1989 se trouve dans Can. Data Rep. Hydrogr. Ocean Sci. 136, par Tabata et Brown.

## Introduction

The Line P program has been an important part of ocean research in Canada since the 1950s.  Considerable effort has been expended to keep this monitoring program viable in the Northeast Pacific Ocean, the result being that the time series of measurements is proving extremely valuable in demonstrating the impact of climate change and variability on oceans.  Early history and data from this program have been summarized by Tabata and Peart (1985).  Dr. Tabata was largely responsible for ensuring the quality of the large temperature, oxygen and salinity data set up to 1990.

In the early 1990s, the Line P program came under the umbrella of the World Ocean Circulation Experiment (WOCE).  Funding became available to upgrade virtually every measurement.  Over a period of three years, the Institute of Ocean Sciences (IOS) switched from hydrographic bottle casts to CTD/rosette casts, with the result that CTD and water sampling became much better meshed, and depth measurements for samples improved.  This was also the era that saw improved horizontal positioning due to the increased use and availability of the GPS for ship’s navigation.  With time, we added instruments to the CTD/rosette package so that we now routinely profile particle distribution to deep ocean, and light and fluorescence to 400 m.  Data from 15 Line P surveys between 1991 and 1997 were submitted to the WOCE Hydrographic Program Office, and are available from http://oceanic.cms.udel.edu/woce/; ftp://nemo.ucsd.edu/woce/Data/Pacific/Repeat/PR6/.

Improvements were also made to routine chemical analyses.  More analyses were done on fresh samples at sea than ever before.  In part this was possible because the research vessel, CCGS John P Tully, has adequate lab space in which to set up analytical instruments.  In addition to oxygen and nutrients, in the past decade we have measured CFCs, iron, total CO2, alkalinity, pCO2, chlorophyll and dimethyl sulfide at sea.

Line P also became a focus of activity under the Joint Global Ocean Flux Study program (JGOFS).  The Canadian JGOFS program, from 1992 to 1997, decided to concentrate effort on understanding the controls of biological growth in the Gulf of Alaska.  Great effort to solve this riddle came from the University of British Columbia (UBC) and other Canadian universities and international collaborators.  A result of this work was the publication of a special volume of Deep Sea Research II (vol. 46, No. 11-12, 1999) which contains a review of the Line P program (Whitney and Freeland, 1999) and an additional 25 papers on biological, geochemical and physical processes of the NE Pacific.  Core measurements from JGOFS studies are available from the Marine Environment Data Service, Fisheries and Oceans at http://www.meds-sdmm.dfo-mpo.gc.ca/.

The focus on processes in the Northeast Pacific during WOCE and JGOFS have spurred several important findings that relate climate change and variability to ocean productivity.  Freeland et al (1998) have shown that the upper ocean mixed layer has been shoaling over the past 40 years, with the result that nutrient supply to phytoplankton is weaker.  Between the 1970s and 1990s, Whitney and Freeland (1999) found an approximately 30% decline in the winter supply of nitrate to surface waters.  Also, about 40% of the interannual variability in this supply appears to be modulated by El Nino – La Nina cycles (Whitney et al, 1998).  We are now confident that iron is a limiting nutrient in open waters in this area (Boyd et al, 1996) and still are uncertain about the interactions of light, temperature and nutrient supply in controlling plankton growth.  Also unresolved is the understanding of how various nutrients are transported to the surface ocean.

Interest in open ocean research remains strong.  The Alaskan Gyre region of the Northeast Pacific is a major habitat of Pacific salmon and a sizeable heat sink/source which moderates Canadian west coast weather.  We foresee the Line P program continuing as an important contribution in understanding the impacts of climate variability on world oceans.

## Ships and Personnel

The majority of the scientific personnel participating in these mission were from IOS.  A few members have made 10 or more trips to Ocean Station Papa during this decade:

|  | Trips |  | trips |
| --- | --- | --- | --- |
| Ron Bellegay | 12 | Wendy Richardson | 16 |
| Reg Bigham | 10 | Marie Robert | 14 |
| John Love | 11 | Tim Soutar | 21 |
| Hugh Maclean, UBC | 21 | Frank Whitney | 19 |
| Bernard Minkley | 23 |  |  |

In addition to IOS staff, personnel from the following agencies participated:

Canadian Wildlife Service

Japanese scientists from both industry and universities

Memorial University, St. Johns, Newfoundland

McGill University

Pacific Biological Station, Nanaimo

Pacific Oceanological Institute of Russia

Princeton University

Third Institute of Oceanography of China

University of British Columbia

University of Rimouski

University of Southampton

University of Toronto

University of Victoria

### References:

Boyd, P.W., D.L. Muggli, D.E. Varela, R.H. Goldblatt, R. Chretien, K.J. Orians, and P.J. Harrison.  1996.  In Vitro Iron Enrichment Experiments in the Northeast Subarctic Pacific.  Mar. Ecol. Prog. Ser. 136: 179-193.

Endeco/YSI Inc.,1992.  Shock Absorber Cord.  13 Atlantis Drive, Marion, Massachusetts, USA

Foster, Derek , 1993, Pneumatic Heave Compensator.  Lands End Technology, Victoria, BC.

Freeland, H.J., K.L. Denman, C.S. Wong, F.A. Whitney, and R. Jacques. 1998.  Evidence of Change in the Winter Mixed Layer in the Northeast Pacific Ocean.  Deep-Sea Res. 44: 2117-2129.

Guildine Instruments, 1994, Operating manual for model 8730Expandable precision digital CTD System, 21 Gilroy Street, P.O. Box 99, Smiths Falls, Ontario, Canada, K7A 4S9.

Seabird Electronics, Inc., 1996, CTD System Operating and Repair Manual for 911plus CTD, 1808 136th Place Northeast, Bellevue, Washington, 98005, USA

Tabata, S and L.J. Peart.  1985.  Statistics of Oceanographic Data Based on Hydrographic/STD Casts Made at Ocean Station P During August 1956 through June 1981. Can. Data Rep. Hydrogr. Ocean Sci. 31: 133 p.

Tabata, S. and Brown, R.  1994.  Hydrographic/CTD Observations Made During Ocean Climate Monitoring Study: 1981-1991 – A Summary of Operational Phase of Study, Can. Data Rep. Hydrogr. Ocean Sci. 136: 42 p.

Whitney, F.A. and H.J. Freeland.  1999.  Variability in Upper Ocean Water Properties in the Northeast Pacific Ocean.  Deep-Sea Res. II, 46: 2351-2370.

Whitney, F.A., C.S. Wong, and P.W. Boyd.  1998.  Interannual Variability in Nitrate Supply to Surface Waters of the Northeast Pacific Ocean.  Mar. Ecol. Prog. Ser. 170: 15-23.

## Table 1. List of ships, dates, and personnel, 1991 to 1999

Mission     ship	start date 	end date	master	area

91-03	Tully	19-Feb-91	11-March-91	John Anderson	Line P, Line Z, line R,

lines MB and MA

CTD: Guildline 8715 SN 53501.

Ron	Bellegay 	IOS	chief scientist

Beg	Bigham	IOS

Gilles	Desmeules

Bernard	Minkley 	IOS

Tim	Soutar 	IOS

Darren	Tuele	IOS

Jinping	Wu	UBC

91-08	Parizeau	28-Apr-91	10-May-91	John Campbell	line P, line R, line MA

CTD: Guildline SN 53501 and 58483

Sus	Tabata	IOS	chief scientist

Mary-Beth	Bérubé	IOS

John	Love 	IOS

Bernard	Minkley 	IOS

Graeme	Quin	contractor

Les	Spearing	IOS

Jin Ping	Wu	UBC

91-15	Endeavour	17-Oct-91	1-Nov-91	W. McMunagle	Line P, line Z

CTD: Guildline 8706 SN 43571 and 8715 SN 58483

Reversing thermometers used with 1.7 litre Niskin bottles.

Ron	Bellegay 	IOS	chief scientist

Mike	Dempsey	contractor

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Robert	Mugo	UTC

Ron	Perkin	IOS

Tim	Soutar 	IOS

Les	Spearing	IOS

Jinping	Wu	UBC

92-01	Tully	03-Feb-92	14-Feb-92	John Anderson	Line P, line R (partial)

CTD: Guildline 8706 SN 43501 and 8715 SN 58483

Frank	Whitney 	IOS	chief scientist

Ron	Bellegay 	IOS

Keith	Johnson	IOS

Robert	Millar	co-op student

Bernard	Minkley 	IOS

Tom	Moore	Guildline Instruments

Wendy	Richardson	IOS

Les	Spearing	IOS

Clayton	Stark	co-op student

Shu Lun	Wang	UBC

92-06	Endeavour	26-Mar-92	13-April-91	W. McMunagle	Line P line Z, line R,

and line J

CTD: Guildline 8715 SN 53501 and 8715 SN 58483

Reversing thermometers used with 1.7 litre Niskin bottles.

Ron	Perkin	IOS	chief scientist

Ron	Bellegay 	IOS

Reg	Bigham	IOS

Bernard	Minkley 	IOS

Tim	Soutar 	IOS

Les	Spearing	IOS

Jin Ping	Wu	UBC

92-15	Tully	08-Sep-92	29-Sept-92	Paul Frost	Line P, line Z,

line R (partial)

CTD: Guildline SN 58483 and 59901 (testing only), Swann CTD winch

General Oceanics Rosette frame.

Reversing thermometers used for depths greater than 3000 m.

Frank	Whitney	IOS	chief scientist

Reg	Bigham	IOS

Marty	Davelaar	IOS

Nick	Hall-Patch	Co-op student

John	Love 	IOS

Hugh	Maclean 	UBC

Brad	McKelvey	UBC

Bernard	Minkley 	IOS

Marie	Robert	contractor

Tim	Soutar 	IOS

Diana	Varela	UBC

93-03	Tully	06-Mar-93	18-March-93	John Anderson	Line P

CTD: Guildline SN 59901 and 58483, Lantec winch used for Rosette casts.

New and much larger A-frame on deck

1 ½ “ diameter shock cords used to help eliminate ship motion from Rosette.

Reversing thermometers used for depths greater than 3000 m.

Ron	Perkin	IOS	chief scientist

Ron	Bellegay 	IOS

Philip	Boyd	UBC

Anna	Flak	Poland

John	Love 	IOS

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Helen	Nicolidakas	UBC

Wendy	Richardson	IOS

Marie	Robert	contractor

Tim	Soutar 	IOS

Diana	Varela	UBC

Frank	Whitney 	IOS

93-09	Tully	14-May-93	31-May-93	John Anderson	Line P, line R, line J

CTD: Guildline SN 59901 and 58483, Lantec winch used for Rosette casts

1 ½ “ diameter shock cords used to help eliminate ship motion from Rosette.

Strain gauge on wire to determine maximum loading of wire.

Ron	Bellegay 	IOS

Reg	Bigham	IOS

Philip	Boyd	UBC

Sean	Doherty	UBC

Nick	Hall-Patch	Co-op student

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Marie	Robert	contractor

Steven	Ruskey

Tim	Soutar 	IOS

Diana	Verela	UBC

Mark	Wen	UBC

Frank	Whitney 	IOS	chief scientist

Jin Ping	Wu	UBC

94-01	Tully	07-Feb-94	18-Feb-94	John Anderson	Line P (partial)

CTD: Guildline SN 59901, Lantec winch used for Rosette casts.

A-frame mounted heavy compensator system.

Ron	Perkin	IOS	chief scientist

Ron	Bellegay 	IOS

Reg	Bigham	IOS

Philip	Boyd	UBC

Sean	Doherty	UBC

Bill	Green	IOS

John	Love 	IOS

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Dario	Stucchi	IOS

Diana	Varela	UBC

Mark	Wen	UBC

Frank	Whitney 	IOS

94-02	Tully	10-May-94	27-May-94	John Anderson	Line P, R16, R12 and R8.

CTD: Guildline SN 59901 and 58483, Lantec winch

Training mission in Saanich Inlet and Haro Strait for the following personnel:

Mary Beth Bérubé, Robin Brown, Tracy Feeney, Howard Freeland, John Garrett, Bill Green, Rhiannon, Johnson, Tom Juhasz, Bernard Minkley , Taimi Mulder, Ron Perkin, Wendy Richardson, Dennis Simmott, Terry Sowden, Carol Steward, Neil Sutherland, Sarah Thornton, Linda White, Frank Whitney , Bob Wilson.

Most of these people departed the ship in Esquimalt harbour and the following personnel joined the ship for mission out to Ocean Station Papa:

Frank	Whitney 	IOS	chief scientist

Ron	Bellegay 	IOS

Reg	Bigham	IOS

Philip	Boyd	UBC

Marty	Davelaar	IOS

Sean	Doherty	UBC

Keith	Johnson	IOS

John	Love 	IOS

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Jim	Powlik	contractor

Wendy	Richardson	IOS

Marie	Robert	contractor

Tim	Soutar 	IOS

Diana	Varela	UBC

Mark	Wen	UBC

94-03	Tully	06-Sep-94	9-Nov-94	John Anderson	Line P, WOCE line P15N

CTD: Guildline SN 59901 and 58483, Lantec winch used.

John	Garrett	IOS	chief scientist

Andrei	Andreev	POI, Vladivostak Russia

Ron	Bellegay 	IOS

Mary-Beth	Bérubé	IOS

Reg	Bigham	IOS

Valerie	Knight	contractor

John	Love 	IOS

Bernard	Minkley 	IOS

Galina	Pavlova	POI, Vladivostak, Russia

Leo	Rebele	UBC

Marie	Robert	contractor

Tim	Soutar 	IOS

Dario	Stucchi	IOS

Sarah	Thornton	UBC

Louise	Timmermans	UBC

Pavel	Tishchenko	POI, Vladivostak Russia

Linda	White	contractor

Frank	Whitney 	IOS

95-01	Tully	07-Feb-95	23-Feb-95	Al Combs	Line P

CTD: Guildline SN 59901 and 58483, Swann 329 winch used for rosette casts.

Frank	Whitney 	IOS	chief scientist

Reg	Bigham	IOS

Philip	Boyd	UBC

Tracy	Feeney	UBC

Robert	Goldblatt	UBC

John	Love 	IOS

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Tim	Soutar 	IOS

95-05	Tully	09-May-95	25-May-95	Al Coombs	Line P, 130W grid

CTD: Guildline SN 59901 and 58483

Frank	Whitney 	IOS	chief scientist

John	Berges	UBC

Reg	Bigham	IOS

Philip	Boyd	UBC

Stuart	Brock	Co-op student

John	Love 	IOS

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Kate	Read

Wendy	Richardson	IOS

Tim	Soutar 	IOS

Sarah	Thornton	UBC

95-12	Tully	22-Aug-95	13-Sept-95	Al Coombs	Line P

CTD: Guildline SN 59901 and 58483

Philip	Boyd	UBC	chief scientist

Janet	Barwell-Clarke	contractor

Ron	Bellegay 	IOS

Reg	Bigham	IOS

John	Love 	IOS

Hugh	Maclean 	UBC

Wendy	Richardson	IOS

96-01	Tully	20-Feb-96	8-March-96	John Anderson	Line P

CTD: Guildline SN 59901 and 58483

Frank	Whitney 	IOS	chief scientist

Ron	Bellegay 	IOS

Jim	Bishop	UVIC

Philip	Boyd	UBC

Stacey	Cooper	UVIC

Ken	Crocker	MUN

Robert	Goldblatt	UBC

Nick	Hall-Patch	Co-op student

Bernard	Minkley 	IOS

Todd	Mudge	UVIC

Jennifer	Putland	MUN

Wendy	Richardson	IOS

Susanne	Roy	Rimouski U

Nelson	Sherry	UBC

Maureen	Soon	UBC

Tim	Soutar 	IOS

96-09	Tully	07-May-96	30-May-96	John Anderson	Line P, 129W grid

CTD: Guildline SN 59901 and 58483

Philip	Boyd	UBC	chief scientist

Janet	Barwell-Clarke	contractor

Ron	Bellegay 	IOS

Jim	Bishop	UVIC

Ken	Crocker	MUN

Robert	Goldblatt	UBC

John	Love 	IOS

Hugh	Maclean 	UBC

Paul	Mathews	MUN

Bernard	Minkley 	IOS

Ken	Morgan	CWS

Todd	Mudge	UVIC

Jennifer	Putland	MUN

Wendy	Richardson	IOS

Nelson	Sherry	UBC

Robert	Schultz	UVIC

Maureen	Soon	UBC

Tim	Soutar 	IOS

Delphine	Thibault	Rimouski U

96-18	Tully	12-Aug-96	6-Sept-96	John Anderson	Line P, line R

CTD: Guildline SN 59901 and 58483

Frank	Whitney 	IOS	chief scientist

Doug	Anderson	IOS

Ron	Bellegay 	IOS

Michael	Bentley	contractor

Philip	Boyd	UBC

Ken	Crocker	MUN

Edmund	Fok	IOS

Robert	Goldblatt	UBC

Julie	Granger	McGill U

Hugh	Maclean 	UBC

Maite	Maldonado	McGill U

Paul	Matthews	MUN

Wendy	Richardson	IOS

Nelson	Sherry	UBC

Maureen	Soon	UTC

Tim	Soutar 	IOS

Delphine	Thibault	Rimouski U

97-02	Tully	10-Feb-97	28-Feb-97	Kay Gimbel	Line P

CTD: Seabird 0437

Frank	Whitney 	IOS	chief scientist

Laurent	Audaire

Michael	Bentley	contractor

Jim	Bishop	UVIC

Philip	Boyd	UBC

Robert	Goldblatt	UBC

John	Love 	IOS

Hugh	Maclean 	UBC

Maite	Maldonado	McGill U

Paul	Matthews	MUN

Bernard	Minkley 	IOS

Todd	Mudge	UVIC

Wendy	Richardson	IOS

Nelson	Sherry	UBC

Maureen	Soon	UBC

Tim	Soutar 	IOS

Todd	Wood	UVIC

Jim	Zakreski	contractor

97-11	Tully	02-Jun-97	26-June-97	John Anderson	Lines P, Z and AB

CTD: Seabird 0433

Philip	Boyd	UBC	chief scientist

Doug	Anderson	IOS

Janet	Barwell-Clarke	contractor

Michael	Bentley	contractor

Robert	Goldblatt	UBC

Tara	Ivanochko	UBC

Keith	Johnson	IOS

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Wendy	Richardson	IOS

Nelson	Sherry	UBC

Maureen	Soon	UBC

Tim	Soutar 	IOS

Nes	Sutherland	contractor

Darren	Tuele	IOS

97-15	Tully	25-Aug-97	17-Sept-97	Paul Frost	Lines P and MS (south of P)

CTD: Seabird 0437

Frank	Whitney 	IOS	chief scientist

Stuart	Brock	IOS

Lyle	Fairley	contractor

Kimio	Fukami	Japan

Takahiko	Inoue	Japan

Tara	Ivanochko	UBC

Keith	Johnson	IOS

Hiroshi	Kiyosawa	JAPAN

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Jun	Nishioka	JAPAN

Wendy	Richardson	IOS

Marie	Robert	contractor

Mark	Schmidt	U. Delaware

Tim	Soutar 	IOS

Nes	Sutherland	contractor

Shigenobu	Takeda	Japan

97-37	Tully	20-Oct-97	30-Oct-97	Tom Hull 	lines BP, G, D, line P

(5 to 10), C, and B.

CTD: Seabird 0437

Doug	Yelland	IOS	chief scientist

Mike	Bentley	contractor

Beth	Bornhold	UBC

Bridgett	Ferris

Shannon	Harris	UBC

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Doug	Moore	IOS

Jennifer	Putland	UBC

Marie	Robert	contractor

Steve	Romaine	IOS

Diana	Varela	UBC

97-45	Laurier	01-Dec-97	4-Dec-97	Syd Webb	Line P (1-12)

CTD: Seabird 0437

Marie	Robert	contractor

Sheila	Toews	contractor

98-02	Laurier	18-Jan-98	21-Jan-98	Syd Webb	Line P (1-12), El Nino

Watch

CTD: Seabird 0437

Marie	Robert	contractor

Sheila	Toews	contractor

98-03	Tully	16-Feb-98	6-March-98	Tom Hull	Line P

CTD: Seabird 0437

Frank	Whitney 	IOS	chief scientist

Doug	Anderson	IOS

Michael	Bentley	contractor

David	Crawford	IOS

Tara	Ivanochko	UBC

Keith	Johnson	IOS

Michael	Lipsen	UBC

Hugh	Maclean 	UBC

Bernard	Minkley 	IOS

Tom	Plath	volunteer

Wendy	Richardson	IOS

Marie	Robert	contractor

Nes	Sutherland	contractor

S.	Takeda	Japan

Sheila	Toews	contractor

98-13	Laurier	07-Apr-98	10-April-98	Norm Thomas	Line P (1-12)

CTD: Seabird 0443

Doug	Anderson	IOS

Linda	White	contractor

98-15	Tully	01-Jun-98	26-June-98	Paul Frost	Line P, line Z to Alaska,

Bowie Seamount

CTD: Seabird 0443 and 0437

Ron	Perkin	IOS	chief scientist

Doug	Anderson	IOS

Janet	Barwell-Clarke	IOS

Michael	Bentley	contractor

David	Crawford	UBC

Keith	Johnson	IOS

Michael	Lipsen	UBC

Hugh	Maclean 	UBC

Jun	Nishioka	UBC

Wendy	Richardson	IOS

Tim	Soutar 	IOS

Nes	Sutherland	contract

Phillipe	Tortell	Princeton U

Darren	Tuele	IOS

Cathleen	Vestfalls	UBC

98-29	Tully	24-Aug-98	20-Sept-98	John Anderson	Line P, eddy

CTD: Seabird 0443

Frank	Whitney 	IOS	chief scientist

Elaine	Baird	contractor

David	Crawford	UBC

Keith	Johnson	IOS

Hiroshi	Kiyosawa	Japan

Michael	Lipsen	UBC

Hugh	Maclean 	UBC

Adrian	Marchetti	IOS, student

Bernard	Minkley 	IOS

Jennifer	Putland	contractor

Wendy	Richardson	IOS

Marie	Robert	contractor

Tim	Soutar 	IOS

Nes	Sutherland	contractor

99-01	Tully	8-Feb-99	28-Feb-99	John Anderson	Line P, eddy

CTD: Seabird 0443

Frank	Whitney 	IOS	chief scientist

Doug	Anderson	IOS

Michael	Bentley	contractor

David	Crawford	IOS

Keith	Johnson	IOS

Michael	Lipsen	UBC

Hugh	Maclean 	UBC

Jun	Nishioka	Japan

Wendy	Richardson	IOS

Marie	Robert	contractor

Tim	Soutar 	IOS

Nes	Sutherland	contractor

Cathleen	Vestfals	UBC

99-10	Tully	31-May-99	24-June-99	Bill Noon	Lines P1E and P

CTD: Seabird 0443

Ron	Perkin	IOS	chief scientist

Michael	Arychuk	IOS

Elaine	Baird	contractor

Janet	Barwell-Clarke	IOS

Michael	Bentley	contractor

Vincent	Coronini	IOS, student

Mary	Davelaar	IOS

Michael	Lipsen	UBC

Hugh	Maclean 	UBC

Angelica	Pena	IOS

Melanie	Quenneville	contractor

Wendy	Richardson	IOS

Marie	Robert	contractor

Tim	Soutar 	IOS

Nes	Sutherland 	contractor

Darren	Tuele	IOS

Cathleen	Vestfals	UBC

99-21	Tully	23-Aug-99	14-Sept-99	Paul Frost	Line P, line Z to Alaska,

Bowie Seamount

CTD: Seabird 0437

Frank	Whitney 	IOS	chief scientist

Doug	Anderson	IOS

Michael	Arychuk	IOS

Michael	Bentley 	contractor

Beth	Bornhold	IOS

David	Crawford	U. Southampton

Chantale	Freeman	UVIC

Phoebe	Lam	Princeton U

Michael	Lipsen	UBC

Hugh	Maclean 	UBC

Jacqueline	O Connell	UBC

Jennifer	Putland	contractor

Marie	Robert 	contractor

Tim	Soutar 	IOS

Philippe	Tortell	Princeton U

## Table 2. Types of samples taken on each mission.

| mission | Start date | ship | Sal | oxy | nut | NH4 | Chl | POC | DON | TCO2 | Alk | pCO2 | CFCs | Fe | 13C | 18O | DMS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 91-03 | 19-Feb-91 | T | X | X | X(f) |  |  | X |  | X | X |  |  |  |  |  |  |
| 91-08 | 28-Apr-91 | P | X | X | X(f) |  |  |  |  |  |  |  |  |  |  |  |  |
| 91-15 | 17-Oct-91 | E | X | X | X(f) |  |  |  |  |  |  |  |  |  |  |  |  |
| 92-01 | 03-Feb-92 | T | X | X | X |  |  | X |  | X |  |  | X |  |  |  |  |
| 92-06 | 26-Mar-92 | E | X | X | X |  |  | X |  |  |  |  |  |  |  |  |  |
| 92-15 | 08-Sep-92 | T | X | X | X |  |  | X |  | X |  |  |  |  |  |  |  |
| 93-03 | 06-Mar-93 | T | X | X | X |  |  | X |  | X |  |  | X |  | X |  |  |
| 93-09 | 14-May-93 | T | X | X | X |  |  | X |  | X |  |  |  |  | X |  |  |
| 94-01 | 07-Feb-94 | T | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 94-02 | 10-May-94 | T | X | X | X |  |  | X |  | X | X | X | X |  |  |  |  |
| 94-03 | 06-Sep-94 | T | X | X | X |  |  |  |  | X | X |  | X |  | X |  |  |
| 95-01 | 07-Feb-95 | T | X | X | X |  |  | X |  |  |  |  |  |  | X |  |  |
| 95-05 | 09-May-95 | T | X | X | X |  | X | X |  | X |  | X | X |  | X |  |  |
| 95-12 | 22-Aug-95 | T | X | X | X |  |  | X |  |  |  | X |  |  | X |  |  |
| 96-01 | 20-Feb-96 | T | X | X | X |  | X | X | X | X | X | X |  |  | X | X | X |
| 96-09 | 07-May-96 | T | X | X | X |  | X | X | X | X | X | X |  |  | X | X | X |
| 96-18 | 12-Aug-96 | T | X | X | X | X | X | X | X | X | X | X |  |  | X | X | X |
| 97-02 | 10-Feb-97 | T | X | X | X | X | X |  | X | X | X | X |  |  | X | X |  |
| 97-11 | 02-Jun-97 | T | X | X | X | X | X | X | X | X | X | X |  | X | X | X | X |
| 97-15 | 25-Aug-97 | T | X | X | X | X | X | X | X | X | X | X |  | X | X | X | X |
| 97-37 | 20-Oct-97 | T | X | X | X(f) | X | X |  |  |  |  |  |  |  |  |  |  |
| 97-45 | 01-Dec-97 | L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 98-02 | 18-Jan-98 | L | X | - | X(f) |  |  |  |  |  |  |  |  |  |  |  |  |
| 98-03 | 16-Feb-98 | T | X | X | X |  | X | X | X |  |  | X |  | X | X | X | X |
| 98-13 | 07-Apr-98 | L | X | - | X(f) |  |  |  |  |  |  |  |  |  |  |  |  |
| 98-15 | 01-Jun-98 | T | X | X | X | X | X | X |  | X | X | X |  | X | X | X | X |
| 98-29 | 24-Aug-98 | T | X | X | X | X | X | X | X | X | X |  | X | X |  | X | X |
| 99-01 | 08-Feb-99 | T | X | X | X |  | X | X |  | X | X | X | X | X | X | X | X |
| 99-10 | 31-May-99 | T | X | X | X | X | X | X |  | X | X | X | X |  | X |  | X |
| 99-21 | 23-Aug-99 | T | X | X | X | X |  | X |  | X | X | X | X | X | X | X |  |

Abbreviations:

| T | CCGS John P Tully | E | CNAV Endeavour |
| --- | --- | --- | --- |
| P | CCS Parizeau | L | CCGS Sir Wilfrid Laurier |
| sal | Salinity | TCO2 | Total carbon dioxide |
| oxy | dissolved oxygen | Alk | Alkalinity |
| nut | nutrients, Si, NO3,PO4 (f = frozen) | pCO2 | partial pressure of CO2 in water |
| NH4 | Ammonium | CFCs | Chlorofluorocarbons |
| chl | Chlorophyll | Fe | Iron |
| POC | Particulate Organic Carbon (+ N) | 13C | Carbon 12/13 ratio in TCO2 |
| DON | Dissolved Organic Nitrogen | 18O | Oxygen 16/18 ratio in water |
| DMS | dimethyl sulphide | X(f) | Nutrient samples frozen |

## Water Sampling Equipment

### Reversing Thermometers

Deep sea reversing thermometers had been used on all missions up to 92-01 to obtain water temperatures.  These thermometers were mounted on 1.7 litre Niskin bottles which were clamped to 5/32 inch diameter hydro wire (Seale construction, right regular lay, improved plough steel, galvanized, torque-balanced).  The Niskin bottles collected water for oxygen and salinity analyses.  Two protected and one unprotected thermometer was mounted in each bottle.  In September 1992 we started using a General Oceanic Rosette frame with 10 litre Niskin bottles.  Reversing thermometers were not used with this Rosette.  For pressures greater than 3000 dbars (maximum depth of CTD) a bottle cast was done using 1.7-litre bottles fitted with reversing thermometers.  After March 1993 reversing thermometers were no longer required to obtain temperatures, as an improved pressure sensor on the CTD allowed profiles to reach the ocean bottom.

### CTD’s used during years 1991 to 1999

A digital CTD made by Guildline Instruments of Smith Falls, Ontario was used for missions 91-03 to 92-15.  Digital CTD model number 8715, serial numbers 53501 and 58483 have the following specifications:

| Parameter | Accuracy | Resolution | Stability |
| --- | --- | --- | --- |
| Conductivity | ±.005 psu | .001 psu | .002 psu/6 month |
| Temperature | ±.005° | .001° | .002/30 days |
| Pressure | ±.15% | .15 % FS | FS = 3000 dbars |

In September 1992, a Guildline CTD, model 8737 (WOCE) serial number 59901 was purchased to allow us to meet World Ocean Circulation Study (WOCE) requirements for CTD observations.  During mission 92-15, the high precision Guildline CTD was tested.  This new system has the following specifications:

| Parameter | Accuracy | Resolution | Response |
| --- | --- | --- | --- |
| Conductivity | ±.002 psu | .00006 psu | < 50 ms |
| Temperature | ±.002°C | .0004°C | < 50 ms |
| Pressure | ±2 dbars | .3 dbars | < 1 ms |

Accuracy specified is for 30 days and includes linearity, resolution, hysteresis, temperature and calibration uncertainty relative to national standards.

During years 1993 to1996, the WOCE CTD and CTD 58483 were used.  Due to difficulties with the temperature sensors of these CTD’s, a replacement system was purchased from Seabird Electronics, Seattle, Washington, USA.  This system was a model SBE 11 plus deckunit, model SBE 911 plus CTD, and SBE Carousel water sampler model 32 for 24 bottles.  The CTD has the following specifications:

| Parameter | Accuracy | Resolution | Stability |
| --- | --- | --- | --- |
| Conductivity | ±.003 psu | .0004 psu | .0003 psu/month |
| Temperature | ±.001° | .0002° | .0002°/month |
| Pressure | ±.015% | .001 % FS | .0015% full scale |

### Rosette

Prior to 1997, a rosette frame made by General Oceanics was used (Figure 2).  This unit had a diameter of 152.4 cm and area of 18,200 cm2, and could hold 24 ten-litre Niskin bottles.  With 136 kg’s of lead added to assist the frame to “fall”, this unit required an additional 90 kg’s of weight on a bridle suspended beneath the frame to stop it from tilting.  The tilt angle was measured by two tilt sensors mounted on the rosette frame, and the signal feed into the CTD for logging.  With no bridle on the Rosette frame, tilt angles greater than 45 degrees were measured.  The 90 kg lead weight reduced tilt to a few degrees.

A more compact rosette frame but still holding 24 bottles, modelled on a unit designed by Pacific Marine Environmental Laboratory (PMEL), Seattle, Washington, USA, was built in June 1999 (Figure 1).  This unit has a diameter of 121.9 cm, an area of 11,700 cm2 and a height of 183 cm.

Water samplers from Ocean Test Equipment and from General Oceanics have both been used on these rosettes.  In recent years, nylon coated steel springs have been used to close bottles instead of 11mm latex tubing.  These springs are a good choice for relatively clean sampling and minimal Freon contamination.  Sensors added to the Seabird CTD include transmissometers, fluorometers, an underwater light sensor, and a pH probe.  Sea-bird CTD’s make the addition of other sensors simple.  In addition an independent 12 kHz pinger was used to detect the proximity of the CTD/rosette package to the bottom.

### Heave Compensation:

On missions 93-03 and 93-09, a load accumulating system using Endeco 1” diameter shock-cords was used to decrease cable loading due to ships motion from the CTD/rosette package.  This system consisted of:

- three doubled 1” shock-cords spliced to a 1” square braid rope

- A 1.5” double braid nylon pennant, longer than the shock-cords, all attached at both ends, by 4.25T shackles, to steel yokes.

- One end is anchored to a load cell, a pennant from the outer end passes through a block and the pennant suspends the block through which the CTD/rosette is lowered.

In shallow depths the shock-cord acts as a spring to reduce motion on the rosette, at greater depths the shock-cord is at full stretch and the 1.5” double braid nylon pennant also carries the load.  For safety reasons this system of heavy compensation was discontinued and replaced in September of 1993, with a pneumatic heave compensator installed on the large A-frame on the stern of the CCGS JOHN P. TULLY (figure 3).

The heave compensator consists of a double-ended air cylinder with double rope sheaves mounted on each rod end.  Two equal lengths of wire rope are attached to fixed points on the underside of the cylinder.  Each rope passes over one sheave at each end of the cylinder then through a swinging deflector sheave block and is terminated on the travelling Tri-plate block.  When the load on the Tri-plate block increases (due to line tension), the trapped air pressure in the cylinder no longer fully supports the load and the cylinder compresses until a load/pressure equilibrium is again established.  A decrease in line load similarly causes the cylinder to extend raising the Tri-plate.  The trapped air is also contained in expansion chambers located adjacent to the cylinder.  One chamber is permanently open to the cylinder charge pressure.  When only the expansion chamber is in use, the compensator provides a “stiff” system.  (A relatively large change in line tension is required to fully stroke the cylinder.)  When the second larger expansion chamber is also utilized the compensator provides a “soft” system.  (a much smaller change in line tension is required to stroke the cylinder.)  The rod ends of the cylinder are connected to a low pressure (approximately 5 psi.) expansion chamber to ensure that the cylinder does not suck in any moist air when it is being compressed.  The large expansion chamber is always connected to the cylinder main charge pressure via a small orifice.  This ensures that the large chamber slowly follows the system pressure and there will not be any major change of pressure and Tri-plate position when the expansion chamber is engaged.

This system has been used on missions since 1994 to diminished cable loading during CTD/rosette casts.  Also used on missions 93-03 and 93-09 was a cable tensiometer to measure maximum cable loading.  Tension measurements were in the range of 1360 to 1812 kilograms, but did on occasion exceed 2265 kilograms when the Rosette was started back up after reaching maximum depth (4200 metres) or the ship manoeuvred to correct the wire angle.

## Table 3 of JGOFS and WOCE designated missions.

JGOFS = Joint Global Ocean Flux Study, WOCE = World Ocean Circulation Experiment

| Mission | JGOFS | WOCE | Comments |
| --- | --- | --- | --- |
| 91-03 |  |  |  |
| 91-08 |  |  |  |
| 91-15 |  | X | First WOCE survey on line PR6 (line P) |
| 92-01 |  | X |  |
| 92-06 |  | X |  |
| 92-15 | X | X | First Canadian JGOFS mission |
| 93-03 | X | X |  |
| 93-09 | X | X |  |
| 94-01 | X | X |  |
| 94-02 | X | X |  |
| 94-03 |  | X | Line P and P15N (down 165°W) |
| 95-01 | X | X |  |
| 95-05 | X | X |  |
| 95-12 | X | X |  |
| 96-01 | X | X |  |
| 96-09 | X | X |  |
| 96-18 | X | X |  |
| 97-02 | X | X | Last WOCE repeat hydrographic line |
| 97-11 | X |  | Last JGOFS mission on line P |
| 97-15 |  |  | First attempt to deploy open ocean enclosures |
| 97-37 |  |  | GLOBEC mission |
| 97-45 |  |  | El Nino Watch |
| 98-02 |  |  | El Nino Watch |
| 98-03 |  |  |  |
| 98-13 |  |  | El Nino Watch |
| 98-15 |  |  |  |
| 98-29 |  |  | First eddy survey |
| 99-01 |  |  |  |
| 99-10 |  |  |  |
| 99-21 |  |  |  |

## Plots of CTD Locations for missions done 1991 to 1999

## Figure 1.  IOS Rosette

## Figure 2.  General Oceanics Rosette

## Figure 3.  Heave Compensator mounted on CCGS Tully A-frame

## Figure 4.  CSS Parizeau

## Figure 5.  CCGS J.P. Tully

## Figure 6.  CNAV Endeavour